3GPP TSG-RAN WG4 Meeting#105      	R4-2218281

Toulouse. France, 14 – 18 Nov, 2022

Title:	Intra-band non-collocated EN-DC/NR-CA type 4 UE

Agenda item:	8.11.2

Source:	Nokia

Document for:	Approval

# 1	Background

In RAN4#104-bis meeting under [1] [104-bis-e][134] WF[1] was approved where it was decided that handheld UE type3a/3b is to be prioritised.

- Prioritize Type 3a/3b discussion for 4MIMO layer in Rel-18.

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

# 2 	Discussion

In our opinion even though Type 3a/3b is prioritized in discussion after that discussion is concluded or parallel if discussion is not converging RAN4 should define requirements for FWA UE type 4a/4b.

This is important as FWA volumes are all the time increasing globally. For example, according to GSA [1]

“5G FWA customer premises equipment shipments more than doubled to 3.6 million in 2021 – a year-on-year increase of over 162%. FWA CPE shipments are forecast to double again to 7.6 million in 2022”

Some of the FWA devices currently produced are 4G or 4G+5G where 5G can be FR2 device but the trend is clear. There is a need for FWA UE type in future. Therefore, we propose

Proposal 1: RAN4 will also develop requirements for UE type 4a/4b.

From requirement perspective UE type 4a/4b is easier that type 3a/3b as all signal paths have dedicated antenna and LNA thus UE type 2 power imbalance requirements can be re-used.

Proposal 2: UE type 4a/4b power imbalance requirements are same as UE type 2 i.e. 25 dB over full dynamic range.

# 3	Conclusion

In this contribution we have done two proposals

Proposal 1: RAN4 will also develop requirements for UE type 4a/4b.

Proposal 2: UE type 4a/4b power imbalance requirements are same as UE type 2 i.e. 25 dB over full dynamic range.

# 4	References

[1] R4-2217734, WF on NonCol_intraB_ENDC_NR_CA for New Type UE, RAN4#104bis, KDDI

[2] GSA Fixed Wireless Access vendor survey reveals 5G FWA CPE shipments forecast to more than double 2022 - GSA (gsacom.com)