3GPP TSG-RAN WG4 Meeting #105	R4-2218309

Toulouse, France, November 14 – November 18, 2022

Agenda Item:	6.3.6.2.2

Source:	Ericsson

Title:	The remaining issues for PDCCH and PBCH requirements in FR2-2

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on the remaining issues for PDCCH and PBCH demodulation requirements for FR2-2. Furthermore, in RAN4 meeting #104-e, the propagation environments have been agreed to align with BS requirements and provided in the Appendix.

For PDCCH: Test number; CBW/ SCS; CORESET duration; CORESET RB; Interleaver Size; REG bundle size; Aggregation level; Propagation Condition; DCI format; Information Bit; Antenna configuration and correlation Matrix. 1; 100/120; 1; 60; 3; 2; 2; TDLA30-200; 1_0; 40; 1x2 Low. 2; 100/120; 1; 60; 2; 6; 4; TDLA30-650; 1_1; 56; 1x2 Low. 3; 100/120; 1; 60; 3; 2; 8; TDLA30-200; 1_1; 56; 2x2 Low. 4; 100/120; 2; 60; 3; 2; 16; TDLA30-650; 1_0; 40; 2x2 Low. 5; 400/480; 1; 60; 2; 6; 4; TDLA10-200; 1_1; 56; 1x2 Low. 6; 400/480; 1; 60; 2; 6; 8; TDLA10-200; 1_1; 56; 1x2 Low. 7; 400/480; 2; 60; 3; 2; 16; TDLA10-200; 1_0; 40; 2x2 Low. For PBCH: Channel Model for FR2-2 PBCH Requirements Test 1-1 (120kHz SCS) - Proposals - Option 1: TDLA30-200; - Option 3: TDLA30-650; Interested companies are encouraged to provide alignment and impairment results for both options in RAN4#105. Test number; SCS/CBW; Antenna configuration; Propagation condition; PBCH SNR (dB) @ 1% Pm-bch; SS/PBCH block index. 1-1; 120kHz/100MHz; 1x2 Low; [TDLA10-200]; New; Unknown. 1-2; 480kHz/400MHz; TDLA10-200; New; Unknown.

In this paper, we bring our view on the remaining issues related to PDCCH and PBCH demodulation.

# 2	Remaining issues for PDCCH requirements

As it has been stated in WF [1], the set of PDCCH test parameters have been agreed. In the following proposal, and based on our simulation results [2], we define the PDCCH requirements as follow.

Proposal 1: Define PDCCH demodulation requirements for UE in FR2-2 with the following test setup.

| Test num | CBW (MHz) / SCS (kHz) | Interleaver size | REG bundle size | CORE SET RB | CORE SET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/ 120 | 3 | 2 | 60 | 1 | 2 | TDLA30-200 | 1x2 ULA Low | 1_0 | 40 | [3.5 + 3.0] |
| 2 | 100/ 120 | 2 | 6 | 60 | 1 | 4 | TDLA30-650 | 1x2 ULA Low | 1_1 | 56 | [1.14 + 3.0] |
| 3 | 100/ 120 | 3 | 2 | 60 | 1 | 8 | TDLA30-200 | 2x2 ULA Low | 1_1 | 56 | [-2.3 + 3.0] |
| 4 | 100/ 120 | 3 | 2 | 60 | 2 | 16 | TDLA30-650 | 2x2 ULA Low | 1_0 | 40 | [-6.2 + 3.0] |
| 5 | 400/ 480 | 2 | 6 | 60 | 1 | 4 | TDLD10-200 | 1x2 ULA Low | 1_1 | 56 | [-1.5 + 3.0] |
| 6 | 400/ 480 | 2 | 6 | 60 | 1 | 8 | TDLA10-200 | 1x2 ULA Low | 1_1 | 56 | [-1.49 + 3.0] |
| 7 | 400/ 480 | 3 | 2 | 60 | 2 | 16 | TDLA10-200 | 2x2 ULA Low | 1_0 | 40 | [-6.35 + 3.0] |

Proposal 2: As per our simulation results, TDLA10-200 is not applicable for 400 MHz/480 kHz with AL4. We can replace it by TDLD10-200, as shown in the Table below

SCS 480 kHz/CBW 400 MHz

| Test case | Interleaver Size | REG bundle size | CORE SET RB | CORE SET duration | Aggre-gation level | Antenna config | DCI format | Infor Bit | Propagation condition | SNR (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-5 | 2 | 6 | 60 | 1 | 4 | 1x2 ULA Low | 1_1 | 56 | TDLA10-200 | 1.27 |
|  |  |  |  |  |  |  |  |  | TDLD10-200 | -1.5 |

# 3	Remaining issues for PBCH requirements

As it has been stated in WF [1], the channel model needs to be fixed for 100 MHz/120 kHz set. Based on our simulation results in [2], we propose the following.

Proposal 3: Define PBCH demodulation requirements with the following test setup

| Test number | SCS/CBW | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120kHz/100MHz | 1x2 Low | TDLA30-650 | [-6.95 + 3.0] | Unknown |
| 1-2 | 480kHz/400MHz |  | TDLA10-200 | [-6.36 + 3.0] | Unknown |

# 4	Summary

In this paper, we tried to contribute with the following proposals for further discussions:

Proposal 1: Define PDCCH demodulation requirements for UE in FR2-2 with the following test setup.

| Test num | CBW (MHz) / SCS (kHz) | Interleaver size | REG bundle size | CORE SET RB | CORE SET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/ 120 | 3 | 2 | 60 | 1 | 2 | TDLA30-200 | 1x2 ULA Low | 1_0 | 40 | [3.5 + 3.0] |
| 2 | 100/ 120 | 2 | 6 | 60 | 1 | 4 | TDLA30-650 | 1x2 ULA Low | 1_1 | 56 | [1.14 + 3.0] |
| 3 | 100/ 120 | 3 | 2 | 60 | 1 | 8 | TDLA30-200 | 2x2 ULA Low | 1_1 | 56 | [-2.3 + 3.0] |
| 4 | 100/ 120 | 3 | 2 | 60 | 2 | 16 | TDLA30-650 | 2x2 ULA Low | 1_0 | 40 | [-6.2 + 3.0] |
| 5 | 400/ 480 | 2 | 6 | 60 | 1 | 4 | TDLD10-200 | 1x2 ULA Low | 1_1 | 56 | [-1.5 + 3.0] |
| 6 | 400/ 480 | 2 | 6 | 60 | 1 | 8 | TDLA10-200 | 1x2 ULA Low | 1_1 | 56 | [-1.49 + 3.0] |
| 7 | 400/ 480 | 3 | 2 | 60 | 2 | 16 | TDLA10-200 | 2x2 ULA Low | 1_0 | 40 | [-6.35 + 3.0] |

Proposal 2: As per our simulation results, TDLA10-200 is not applicable for 400 MHz/480 kHz with AL4. We can replace it by TDLD10-200, as shown in the Table below

SCS 480 kHz/CBW 400 MHz

| Test case | Interleaver Size | REG bundle size | CORE SET RB | CORE SET duration | Aggre-gation level | Antenna config | DCI format | Infor Bit | Propagation condition | SNR (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-5 | 2 | 6 | 60 | 1 | 4 | 1x2 ULA Low | 1_1 | 56 | TDLA10-200 | 1.27 |
|  |  |  |  |  |  |  |  |  | TDLD10-200 | -1.5 |

Proposal 3: Define PBCH demodulation requirements with the following test setup

| Test number | SCS/CBW | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120kHz/100MHz | 1x2 Low | TDLA30-650 | [-6.95 + 3.0] | Unknown |
| 1-2 | 480kHz/400MHz |  | TDLA10-200 | [-6.36 + 3.0] | Unknown |

# References

- R4-2217391, “WF on FR2-2 UE demodulation requirements”, Qualcomm.

- R4-2218310, “Simulation results for PDCCH and PBCH demodulation in FR2-2”, Ericsson.

# Appendix	Simulation assumption

## A.1	Channel models

Channel model tap delay resolution

- Delay resolution: 5ns for below or equal to 200MHz CBW, 2ns for above 200MHz CBW

- [Maximum Number of Taps]: 12 taps for CBW smaller than 200MHz and 16 taps for CBW larger  than 200MHz;

- Adopt channel model according to the channel bandwidth as:

| SCS [kHz] | CBW [MHz] | Applicable channel models | Tap resolution [ns] | [Maximum Number of Taps] |
| --- | --- | --- | --- | --- |
| 120 | 100 | TDLA30-650 TDLD30-200 | 5 | 12 |
| 120 480 [960] | ≥400 | TDLA10-650 TDLD10-200 | 2 | 16 |

Companies can bring simulation results based on following simplified TDL model:

Table 1.1 Simplified TDLA10 model (16 taps, 2ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -16.1 | Rayleigh |
| 2 | 4 | 0 | Rayleigh |
| 3 | 6 | -4 | Rayleigh |
| 4 | 8 | -10.2 | Rayleigh |
| 5 | 16 | -18.6 | Rayleigh |
| 6 | 18 | -9.3 | Rayleigh |
| 7 | 22 | -13.7 | Rayleigh |
| 8 | 24 | -17.9 | Rayleigh |
| 9 | 26 | -13.5 | Rayleigh |
| 10 | 30 | -14 | Rayleigh |
| 11 | 40 | -15.4 | Rayleigh |
| 12 | 44 | -18.9 | Rayleigh |
| 13 | 46 | -21.0 | Rayleigh |
| 14 | 48 | -21.6 | Rayleigh |
| 15 | 50 | -19.3 | Rayleigh |
| 16 | 96 | -25.9 | Rayleigh |

Table 1.2 Simplified TDLA30 model (12 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 | Rayleigh |
| 3 | 15 | -5.1 | Rayleigh |
| 4 | 20 | -5.1 | Rayleigh |
| 5 | 25 | -9.6 | Rayleigh |
| 6 | 50 | -8.2 | Rayleigh |
| 7 | 65 | -13.1 | Rayleigh |
| 8 | 75 | -11.5 | Rayleigh |
| 9 | 105 | -11.0 | Rayleigh |
| 10 | 135 | -16.2 | Rayleigh |
| 11 | 150 | -16.6 | Rayleigh |
| 12 | 290 | -26.2 | Rayleigh |

Table 1.3 Simplified TDLD10 model (10 taps, 2ns resolution)

| Tap # | 2ns resolution |  | Fading distribution |
| --- | --- | --- | --- |
|  | Delay [ns] | Power [dB] |  |
| 1 | 0 | -0.2 | LOS |
|  | 0 | -12.4 | Rayleigh |
| 2 | 6 | -21.1 | Rayleigh |
| 3 | 14 | -16.7 | Rayleigh |
| 4 | 18 | -18.3 | Rayleigh |
| 5 | 26 | -22 | Rayleigh |
| 6 | 40 | -27.9 | Rayleigh |
| 7 | 80 | -23.7 | Rayleigh |
| 8 | 94 | -24.9 | Rayleigh |
| 9 | 98 | -30.0 | Rayleigh |
| 10 | 126 | -27.7 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |

Table 1.4 Simplified TDLD30 model (10 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -12.4 | Rayleigh |
| 2 | 20 | -21 | Rayleigh |
| 3 | 40 | -16.7 | Rayleigh |
| 4 | 55 | -18.3 | Rayleigh |
| 5 | 80 | -21.9 | Rayleigh |
| 6 | 120 | -27.8 | Rayleigh |
| 7 | 240 | -23.6 | Rayleigh |
| 8 | 285 | -24.8 | Rayleigh |
| 9 | 290 | -30.0 | Rayleigh |
| 10 | 375 | -27.6 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |