3GPP TSG-RAN WG4 Meeting #105	R4-2218314

Meeting, 14 November – 18 November 2022

Agenda Item:	8.6.6.1

Source:	Ericsson

Title:	UE demodulation and CSI reporting requirements for 8 Rx in FR1

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on UE demodulation and CSI reporting requirements for enhanced 8 Rx in FR1. Further RF enhancements are being discussed, while in this paper, we will provide our views and which parameters to propose in this first meeting discussing the UE performance part. In RP-221496 [1], the objective for UE demodulation with 8 Rx has been stated as follow.

### 4.1 Objective of SI or Core part WI or Testing part WI Enable 8Rx for CPE/FWA/vehicle/industrial devices [RAN4] - Example bands: - TDD bands: n41, n77/ n78 - FDD bands: n7 - Note 1: the total number of example band should be limited to 3. n77/n78 are considered as one band during the study. - Note 2: other bands to be introduced in the release independent way later on from Rel-18 - Note 3: specifying requirements for TDD bands has first priority - Specify the UE RF requirements to support 8Rx - Study and specify the requirements to support SRS antenna switching for t1r8, t2r8, t4r8 - Discussion on t4r8 shall start after at least one PC for 4Tx is completed NOTE: Requirements are specified with phase approach. Objectives with 1st priority are considered first. ### 4.2 Objective of Performance part WI - Enable 8Rx for CPE/FWA/vehicle/industrial devices - Specify RLM test cases to support 8Rx - Investigate if the existing 4Rx RLM test can be reused or the new test will be specified - Specify UE demodulation performance and CSI requirements with up to 8 layers to support 8Rx - Investigate and, if necessary, specify the requirements with up to 8 DL MIMO layers - Specify the SDR requirements with 8 MIMO layers

# 2	General aspects for UE demodulation and CSI reporting requirements

In this section, we refer to [1] where the priorities have been set to:

- Enable 8Rx for CPE/FWA/vehicle/industrial devices

- Priority to consider TDD bands: n41, n77/ n78

- Specify RLM test cases to support 8Rx

- Specify UE demodulation performance and CSI requirements with up to 8 layers to support 8Rx

- Specify the SDR requirements with 8 MIMO layers

Accordingly, and to be more efficient and focus the efforts on relevant parameters, we propose to extend the test cases in FR1 for 8 Rx UE as follow.

- Applicable numerology

  - SCS = 30 kHz, CBW = 40 MHz, and 106 RB allocation.

- Maximum number of DL MIMO layers

  - A UE with 8Rx branches and up to 8 DL MIMO layers is supported.

- Maximum modulation order

  - Up to a 1024QAM modulation is supported.

Proposal 1: RAN4 defines the UE demodulation and CSI reporting requirements for FR1 with:

- TDD mode only

- Number of receive antennas: 8Rx

- Modulation order: Up to 1024QAM

- Rank: Up to 8 DL MIMO layers.

Proposal 2: RAN4 defines the UE demodulation and CSI reporting requirements with:

- FR1 TDD n77/ n78: fc = 3.5 GHz

- FR1 TDD: SCS = 30 kHz, CBW = 40 MHz and 106 RBs.

- Channel models: TDLA, TDLD and TDLC (30 ns delay spread)

- Max Doppler frequencies: up to 100 Hz (vehicular).

# 3	UE Demodulation Requirements

## 3.1 RF Parameters

In TS38.101-1 [2], it has been stated in Clause 7.2 that the UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n48, n77, n78, n79 where the UE is required to be equipped with a minimum of four Rx antenna ports. This requirement applies when the band is used as a standalone band or as part of a band combination. In this WID, the UE will be equipped with 8 Rx, and the focus will be on TDD bands.

Proposal 3: Align the RF parameters to the RF enhancements in the core part, and consider as a starting point

- Tx EVM = 6%

- Rx EVM = 0%

## 3.2 PDSCH demodulation requirements

The parameters specified in Table 5.2-1 [3] are valid for all PDSCH tests unless otherwise stated. However, to save efforts, it is preferrable to limit our requirements to PDSCH Type A.

Proposal 4: Define the 8 Rx UE demodulation requirements for PDSCH Type A only.

For 8 Rx UE, we can focus on 70% of the PDSCH peak throughput metric. Further discussions are needed to conclude whether 30% of peak throughput metric is needed or not.

Proposal 5: Define the 8 Rx UE demodulation requirements for PDSCH with 70% of the peak throughput metric only.

For UL-DL pattern, we do not see the usefulness of defining a new pattern. We would prefer to stick to FR1.30-1 for PDSCH requirements’ definition.

Proposal 6: Define the 8 Rx UE demodulation requirements for PDSCH using FR1.30-1 TDD UL-DL pattern, given by 7DS2U and S: 6D+4G+4U.

We summarize the needed parameters to define the PDSCH requirements in Proposals 7 and 8.

Proposal 7: Define PDSCH demodulation requirements for UE at 70% of the peak throughput using the following test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 3.5 |
| Subcarrier Spacing [kHz] | 30 |
| Channel bandwidth [MHz] | 40 |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA, TDLC, TDLD (30 ns delay spread) |
| Antenna configuration | 8x8 ULA Low |
| Velocity | 3 km/h, 30 km/h |
| PA Model | None |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 5, Rank 6, Rank 7, and Rank 8 |
| DMRS Configuration | 4 DMRS symbols at (2,3,10,11) symbol indexes |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 17 (64QAM). |
| NOH | 6 |
| Number of HARQ Processes | 8 |

Proposal 8: Referring to the Table in Proposal xx, consider PDSCH test cases at 70% of the peak throughput for 40 MHz/30 kHz SCS as follow

FR1 TDD, 40 MHz/30 kHz, Metric: 70% Peak Throughput

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | TDD UL/DL Pattern | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 (106) | QPSK, 0.30 | FR1.30-1 | 8 | TDLC30-100 | 8x8, ULA Low | 70 | TBD |
| 1-2 | 40 / 30 (106) | 16QAM, 0.48 | FR1.30-1 | 8 | TDLA30-5 | 8x8, ULA Low | 70 | TBD |
| 1-3 | 40 / 30 (106) | 16QAM, 0.48 | FR1.30-1 | 7 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-4 | 40 / 30 (106) | 16QAM, 0.48 | FR1.30-1 | 6 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-5 | 40 / 30 (106) | 64QAM, 0.43 | FR1.30-1 | 6 | TDLD30-5 | 8x8, ULA Low | 70 | TBD |
| 1-6 | 40 / 30 (106) | 16QAM, 0.48 | FR1.30-1 | 5 | TDLA30-5 | 8x8, ULA Low | 70 | TBD |
| 1-7 | 40 / 30 (106) | 64QAM, 0.43 | FR1.30-1 | 5 | TDLD30-5 | 8x8, ULA Low | 70 | TBD |

## 3.4	PDCCH demodulation

We propose to define PDCCH for 8 Rx UE with all possible aggregation levels 2, 4, 8, and 16 by reusing the existing test case specified in TS38.101-4 Clause 5.3.3.2 for 30 kHz in FR1.

Proposal 9: Define PDCCH demodulation requirements for 8 Rx UE in FR1 with the following test setup.

- FR1 TDD, 40 MHz/30 kHz

| Test num | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNR (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 2 | 102 | 1 | 2 | TDLA30-10 | 1x8 ULA Low | 1_0 | 41 | New |
| 2 | 3 | 2 | 102 | 1 | 4 | TDLA30-10 | 1x8 ULA Low | 1_1 | 53 | New |
| 3 | NA | 6 | 90 | 1 | 8 | TDLA30-10 | 2x8 ULA Low | 1_1 | 53 | New |
| 4 | 3 | 6 | 48 | 2 | 16 | TDLA30-10 | 2x8 ULA Low | 1_0 | 41 | New |

- Using the common test parameters

| Parameter | Unit | 1 Tx Antenna |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR1.30-1 |  |  |
| CCE to REG mapping type |  | Test 3: Non-interleaved Other tests: interleaved | Interleaved |  |
| Interleaver size |  | 3 |  |  |
| REG bundle size |  | 2 | 6 |  |
| Shift Index |  | 0 |  |  |

## 3.5	PBCH demodulation

Referring to TS 38.101-4 Clause 5.1.1.2, requirements for PBCH with 4Rx is up to UE declaration. In our case, we believe that RAN4 does not need to define PBCH requirements for 8 Rx.

Proposal 10: Do not define PBCH demodulation requirements for 8 Rx UE in FR1.

## 3.6	SDR

The purpose of the SDR requirements is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. According to TS38.101-4, the existing MCS index table has already covered the configuration applicable for UEs up to rank 4 for QPSK/16QAM/64QAM/256QAM, while up to rank 2 for 1024QAM.

Observation 1: RAN4 needs to extend the SDR applicability rule for FR1 where 6 PDSCH MIMO layers using 8 Rx are considered.

According to Observation 1, we propose the following.

Proposal 11: Define SDR requirements applicable to FR1 CC and FR1 CA, considering

- Update Table 5.5A-5 to support ‘Maximum number of PDSCH MIMO layers = 8’

- RAN4 may consider ‘Maximum number of PDSCH MIMO layers = 6’

- Update Table 5.5A-6 (1024QAM) to support 8Rx. Note the Maximum number of PDSCH MIMO layers is 2.

- RAN4 study the maximum MCS for scaling factor 1.

# 4	CSI reporting requirements

RAN4 has defined CQI, PMI and RI reporting requirements for FR1 in Rel-15. Regarding the CQI reporting, we propose to define requirements under the static condition for UE with 8Rx. Since the existing CQI reporting tests verifies the CQI using 256 QAM for PDSCH FR1 with 4 Rx (see 38.101-4 Table A.4-3), we need to define the CQI reporting requirements for 8Rx UE while we limit the modulation order to 64QAM.

Proposal 12: Define the CQI reporting requirements for 8Rx UE under static conditions considering

- 40 MHz/30 kHz, 106 RBs.

- CQI index for up to 64QAM.

- Rank 4.

For 8 Rx UE, new requirements for PMI reporting should be defined. We propose the following.

Proposal 13: Define PMI reporting requirements for 8 Rx UE considering

- 40 MHz/30 kHz

- Propagation environment: TDLA30-5

- Single PMI- with 8 Tx Type I with single panel codebook and antenna configuration: High XP 8 x 8 with (N1, N2) = (4, 1)

- Multiple PMI with 16 Tx Type II codebook and antenna configuration: Medium XP 16x8 with (N1, N2) = (4, 2)

In TS38.101-4 Clause 6.4.3.2, the reporting rank indicator has been limited to Rank 2 transmission even when considering 4 Rx. We believe that such a limitation will not change anything when considering 8 Rx. Therefore, we will not define RI reporting for 8 Rx.

Proposal 14: Do not define RI reporting for 8 Rx UE.

# 5	Specification structure

Table 1	Specification structure for UE Demodulation performance requirements in FR1 (Conducted requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 5.1 | General | Updates on FR1 8Rx |
| 5.2 | PDSCH demodulation requirements |  |
| 5.2.1 | 1RX requirements |  |
| 5.2.2 | 2RX requirements |  |
| 5.2.3 | 4RX requirements |  |
| 5.2.4 | 8RX requirements | New section |
| 5.2.4.1 | FDD | Void |
| 5.2.4.2 | TDD | New section |
| 5.2.4.2.1 | Minimum requirements for PDSCH Mapping Type A | New section |
| 5.2A | PDSCH demodulation requirements for CA |  |
| 5.2A.4 | 8RX requirements | New section |
| 5.3 | PDCCH demodulation requirements |  |
| 5.3.1 | 1RX requirements |  |
| 5.3.2 | 2RX requirements |  |
| 5.3.3 | 4 RX requirements |  |
| 5.3.4 | 8RX requirements | New section |
| 5.3.4.1 | FDD | Void |
| 5.3.4.2 | TDD | New section |
| 5.5 | Sustained downlink data rate provided by lower layers | Updates on FR1 8 Rx |
| 5.5.1 | FR1 single carrier requirements | Updates on FR1 8 Rx |
| 5.5A | Sustained downlink data rate provided by lower layers |  |
| 5.5A.1 | FR1 CA requirements | Updates on FR1 8 Rx |

Table 2	Specification structure for UE CSI reporting requirements in FR1 (Conducted requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 6.1 | General | Updates on FR1 8 Rx |
| 6.2 | Reporting of Channel Quality Indicator (CQI) |  |
| 6.2.4 | 8RX requirements | New section |
| 6.2.4.1 | FDD | Void |
| 6.2.4.2 | TDD | New section |
| 6.2A | Reporting of Channel Quality Indicator (CQI) for CA |  |
| 6.2A.5 | 8RX requirements | New section |
| 6.3 | Reporting of Precoding Matrix Indicator (PMI) |  |
| 6.3.4 | 8RX requirements | New section |
| 6.3.4.1 | FDD | Void |
| 6.3.4.2 | TDD | New section |

# 6	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Proposal 1: RAN4 defines the UE demodulation and CSI reporting requirements for FR1 with:

- TDD mode only

- Number of receive antennas: 8Rx

- Modulation order: Up to 1024QAM

- Rank: Up to 8 DL MIMO layers.

Proposal 2: RAN4 defines the UE demodulation and CSI reporting requirements with:

- FR1 TDD n77/ n78: fc = 3.5 GHz

- FR1 TDD: SCS = 30 kHz, CBW = 40 MHz and 106 RBs.

- Channel models: TDLA, TDLD and TDLC (30 ns delay spread)

- Max Doppler frequencies: up to 100 Hz (vehicular).

Proposal 3: Align the RF parameters to the RF enhancements in the core part, and consider as a starting point

- Tx EVM = 6%

- Rx EVM = 0%

Proposal 4: Define the 8 Rx UE demodulation requirements for PDSCH Type A only.

Proposal 5: Define the 8 Rx UE demodulation requirements for PDSCH with 70% of the peak throughput metric only.

Proposal 6: Define the 8 Rx UE demodulation requirements for PDSCH using FR1.30-1 TDD UL-DL pattern, given by 7DS2U and S: 6D+4G+4U.

Proposal 7: Define PDSCH demodulation requirements for UE at 70% of the peak throughput using the following test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 3.5 |
| Subcarrier Spacing [kHz] | 30 |
| Channel bandwidth [MHz] | 40 |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA, TDLC, TDLD (30 ns delay spread) |
| Antenna configuration | 8x8 ULA Low |
| Velocity | 3 km/h, 30 km/h |
| PA Model | None |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 5, Rank 6, Rank 7, and Rank 8 |
| DMRS Configuration | 4 DMRS symbols at (2,3,10,11) symbol indexes |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 17 (64QAM). |
| NOH | 6 |
| Number of HARQ Processes | 8 |

Proposal 8: Referring to the Table in Proposal xx, consider PDSCH test cases at 70% of the peak throughput for 40 MHz/30 kHz SCS as follow

FR1 TDD, 40 MHz/30 kHz, Metric: 70% Peak Throughput

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | TDD UL/DL Pattern | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 (106) | QPSK, 0.30 | FR1.30-1 | 8 | TDLC30-100 | 8x8, ULA Low | 70 | TBD |
| 1-2 | 40 / 30 (106) | 16QAM, 0.48 | FR1.30-1 | 8 | TDLA30-5 | 8x8, ULA Low | 70 | TBD |
| 1-3 | 40 / 30 (106) | 16QAM, 0.48 | FR1.30-1 | 7 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-4 | 40 / 30 (106) | 16QAM, 0.48 | FR1.30-1 | 6 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-5 | 40 / 30 (106) | 64QAM, 0.43 | FR1.30-1 | 6 | TDLD30-5 | 8x8, ULA Low | 70 | TBD |
| 1-6 | 40 / 30 (106) | 16QAM, 0.48 | FR1.30-1 | 5 | TDLA30-5 | 8x8, ULA Low | 70 | TBD |
| 1-7 | 40 / 30 (106) | 64QAM, 0.43 | FR1.30-1 | 5 | TDLD30-5 | 8x8, ULA Low | 70 | TBD |

Proposal 9: Define PDCCH demodulation requirements for 8 Rx UE in FR1 with the following test setup.

- FR1 TDD, 40 MHz/30 kHz

| Test num | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNR (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 2 | 102 | 1 | 2 | TDLA30-10 | 1x8 ULA Low | 1_0 | 41 | New |
| 2 | 3 | 2 | 102 | 1 | 4 | TDLA30-10 | 1x8 ULA Low | 1_1 | 53 | New |
| 3 | NA | 6 | 90 | 1 | 8 | TDLA30-10 | 2x8 ULA Low | 1_1 | 53 | New |
| 4 | 3 | 6 | 48 | 2 | 16 | TDLA30-10 | 2x8 ULA Low | 1_0 | 41 | New |

- Using the common test parameters

| Parameter | Unit | 1 Tx Antenna |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR1.30-1 |  |  |
| CCE to REG mapping type |  | Test 3: Non-interleaved Other tests: interleaved | Interleaved |  |
| Interleaver size |  | 3 |  |  |
| REG bundle size |  | 2 | 6 |  |
| Shift Index |  | 0 |  |  |

Proposal 10: Do not define PBCH demodulation requirements for 8 Rx UE in FR1.

Observation 1: RAN4 needs to extend the SDR applicability rule for FR1 where 6 PDSCH MIMO layers using 8 Rx are considered.

Proposal 11: Define SDR requirements applicable to FR1 CC and FR1 CA, considering

- Update Table 5.5A-5 to support ‘Maximum number of PDSCH MIMO layers = 8’

- RAN4 may consider ‘Maximum number of PDSCH MIMO layers = 6’

- Update Table 5.5A-6 (1024QAM) to support 8Rx. Note the Maximum number of PDSCH MIMO layers is 2.

- RAN4 study the maximum MCS for scaling factor 1.

Proposal 12: Define the CQI reporting requirements for 8Rx UE under static conditions considering

- 40 MHz/30 kHz, 106 RBs.

- CQI index for up to 64QAM.

- Rank 4.

Proposal 13: Define PMI reporting requirements for 8 Rx UE considering

- 40 MHz/30 kHz

- Propagation environment: TDLA30-5

- Single PMI- with 8 Tx Type I with single panel codebook and antenna configuration: High XP 8 x 8 with (N1, N2) = (4, 1)

- Multiple PMI with 16 Tx Type II codebook and antenna configuration: Medium XP 16x8 with (N1, N2) = (4, 2)

Proposal 14: Do not define RI reporting for 8 Rx UE.

# References

- RP-221496, “Revised WID on Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)”, Huawei.

- 3GPP TS 38.101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, v17.7.0.

- 3GPP TS 38.101-4, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”, v.17.6.0.

# Appendix	Simulation assumption

## A.1	PDSCH

### FR1

Please refer to TS.38.101-4 Clause 5.2.3.2.1

Table 5.2.3.2.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Tests 1-1, 1-8, 1-9 wideband for Test 3-1 2 for other tests |
|  | Resource allocation type |  | Test 1-2: Type 1 with start RB = 50, LRBs = 6 Other tests: Type 0 |
|  | RBG size |  | Test 1-2: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Tests 1-1, 1-7, 1-8, 1-9, 1-10, 1-11 1 for other tests |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | Tests 1-8, 1-9: l0 = 4 for CSI-RS resource 1 and 3 l0 = 8 for CSI-RS resource 2 and 4 Other tests; Table 5.2-1. |
|  | CSI-RS periodicity | Slots | Test 1-7, 1-10, 1-11: 20 for CSI-RS resource 1,2,3,4. Other tests: Table 5.2-1. |
|  | CSI-RS offset | Slots | Test 1-7, 1-10, 1-11: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. Other tests: Table 5.2-1. |
|  | Frequency Occupation |  | Test 1-7, 1-10, 1-11: Start PRB 0 Number of PRB = 52 Other tests: Table 5.2-1. |
| Number of HARQ Processes |  |  | 16 for Test 1-4 10 for Test 1-9 8 for other tests |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

## A.2	SDR

Table 5.5A-5: MCS indexes for indicated UE capabilities

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 1 | 8 | 1 | 26 |
| 1 | 8 | 0.8 | 21 |
| 1 | 8 | 0.75 | 20 |
| 1 | 8 | 0.4 | 11 |
| 1 | 6 | 1 | 27 |
| 1 | 6 | 0.8 | 23 |
| 1 | 6 | 0.75 | 22 |
| 1 | 6 | 0.4 | 14 |
| 1 | 4 | 1 | 16 |
| 1 | 4 | 0.8 | 16 |
| 1 | 4 | 0.75 | 16 |
| 1 | 4 | 0.4 | 10 |
| 1 | 2 | 1 | 9 |
| 1 | 2 | 0.8 | 9 |
| 1 | 2 | 0.75 | 9 |
| 1 | 2 | 0.4 | 4 |
| 2 | 8 | 1 | 26 |
| 2 | 8 | 0.8 | 21 |
| 2 | 8 | 0.75 | 20 |
| 2 | 8 | 0.4 | 11 |
| 2 | 6 | 1 | 27 |
| 2 | 6 | 0.8 | 23 |
| 2 | 6 | 0.75 | 22 |
| 2 | 6 | 0.4 | 14 |
| 2 | 4 | 1 | 16 |
| 2 | 4 | 0.8 | 16 |
| 2 | 4 | 0.75 | 16 |
| 2 | 4 | 0.4 | 10 |
| 2 | 2 | 1 | 9 |
| 2 | 2 | 0.8 | 9 |
| 2 | 2 | 0.75 | 9 |
| 2 | 2 | 0.4 | 4 |
| 4 | 8 | 1 | 26 |
| 4 | 8 | 0.8 | 23 |
| 4 | 8 | 0.75 | 22 |
| 4 | 8 | 0.4 | 12 |
| 4 | 6 | 1 | 27 |
| 4 | 6 | 0.8 | 24 |
| 4 | 6 | 0.75 | 23 |
| 4 | 6 | 0.4 | 14 |
| 4 | 4 | 1 | 16 |
| 4 | 4 | 0.8 | 16 |
| 4 | 4 | 0.75 | 16 |
| 4 | 4 | 0.4 | 11 |
| 4 | 2 | 1 | 9 |
| 4 | 2 | 0.8 | 9 |
| 4 | 2 | 0.75 | 9 |
| 4 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

Table 5.5A-6: 1024QAM MCS indexes for indicated UE capabilities

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 2RX | 1 | 10 | 1 | 23 |
|  | 1 | 10 | 0.8 | 21 |
|  | 1 | 10 | 0.75 | 19 |
|  | 1 | 10 | 0.4 | 9 |
| 4RX | 1 | 10 | 1 | 24 |
|  | 1 | 10 | 0.8 | 21 |
|  | 1 | 10 | 0.75 | 19 |
|  | 1 | 10 | 0.4 | 9 |
|  | 2 | 10 | 1 | 23 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: For the band(s) on which UE supporting “Maximum modulation format” of 10, with 2 RX and 2 MIMO layers, the MCS index is derived from the rows with “Maximum modulation format” of 8 in Table 5.5A-5 |  |  |  |  |

## A.3	CQI reporting in static condition

### A.3.1	Minimum requirement for CQI periodic reporting

The purpose of the requirements is to verify that the reported CQI values are in accordance with the CQI definition given in TS38.214 [12]. The reporting accuracy of CQI under AWGN condition is determined by the reporting variance and BLER performance using the transport format indicated by the reported CQI median. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

For the parameters specified in Table 6.2.3.2.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Table 6.2.4.2.1.1-1: CQI reporting definition test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| andwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  | dB | 5 | 6 | 11 | 12 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 4×8 with static channel specified in updated Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3,(6) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-3, TBS.3-4 |  |  |  |