3GPP TSG-RAN WG4 Meeting #105	R4-2218315

Toulouse, France, November 14 – November 18, 2022

Agenda Item:	8.6.6.1

Source:	Ericsson

Title:	Initial simulation results for 8Rx PDSCH demodulation in FR1

Document for:	Information

# 1	Introduction

In this paper, we present some initial results on 8Rx PDSCH in FR1. Our initial results will be limited to some use cases with Rank 6 and Rank 8 and different propagation conditions.

### 4.1 Objective of SI or Core part WI or Testing part WI Enable 8Rx for CPE/FWA/vehicle/industrial devices [RAN4] - Example bands: - TDD bands: n41, n77/ n78 - FDD bands: n7 - Note 1: the total number of example band should be limited to 3. n77/n78 are considered as one band during the study. - Note 2: other bands to be introduced in the release independent way later on from Rel-18 - Note 3: specifying requirements for TDD bands has first priority - Specify the UE RF requirements to support 8Rx - Study and specify the requirements to support SRS antenna switching for t1r8, t2r8, t4r8 - Discussion on t4r8 shall start after at least one PC for 4Tx is completed NOTE: Requirements are specified with phase approach. Objectives with 1st priority are considered first. ### 4.2 Objective of Performance part WI - Enable 8Rx for CPE/FWA/vehicle/industrial devices - Specify RLM test cases to support 8Rx - Investigate if the existing 4Rx RLM test can be reused or the new test will be specified - Specify UE demodulation performance and CSI requirements with up to 8 layers to support 8Rx - Investigate and, if necessary, specify the requirements with up to 8 DL MIMO layers Specify the SDR requirements with 8 MIMO layers

# 2	PDSCH Simulation Results

## 2.1 Simulation setup

According to the SID [1], we will use as general parameters:

TDD n77/78:

- fc = 3.5 GHz

- CBW/SCS: 40 MHz/30 KHz

- QPSK and 16QAM for Rank 8

- 16QAM and 64QAM for Rank 6

- Max Doppler frequency/DMRS

- UE speed at 3 Km/h (f_D = 10 Hz), DMRS at (2, 3, 10, 11) indexes

- UE speed at 30 Km/h (f_D = 100 Hz), DMRS at (2, 3, 10, 11) indexes

- Pre-loaded Tx EVM: 6%

- Additive Rx EVM: 0%

Summary of test cases:

| Metric | Purpose | Test index |
| --- | --- | --- |
| 70% of peak Throughput | Verify the PDSCH mapping Type A normal performance under 8 receive antenna conditions, with different channel models, different MCSs but up to 8 MIMO layer (=< Rank 8). | 1-1, 1-2, 1-3, 1-4 |

Test cases:

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 | 8 | TDLC30-100 | 8x8, ULA Low | 70 | TBD |
| 1-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-3 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 | 6 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-4 | 3.5 | 40 / 30 (106) | 64QAM, 0.43 | 6 | TDLD30-5 | 8x8, ULA Low | 70 | TBD |

## 2.2 Simulation results

In this section, we present the simulation results in terms of the normalized PDSCH throughput as function of the signal to noise ratio (SNR). We depict the SNR value corresponding to 70% of the peak throughput. All figures have been provided in the Appendix.

Test cases:

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 | 8 | TDLC30-100 | 8x8, ULA Low | 70 | 1.5 |
| 1-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 13.5 |
| 1-3 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 | 6 | TDLA30-10 | 8x8, ULA Low | 70 | 9.1 |
| 1-4 | 3.5 | 40 / 30 (106) | 64QAM, 0.43 | 6 | TDLD30-5 | 8x8, ULA Low | 70 | 17.8 |

Observation 1: Referring to Proposal 3 in [2] where Tx EVM has been set to 6%, we can note that 8RX UE can support 8 DL MIMO Layers using MCS13 under TDLA30-10 propagation environment. The SNR value is at 13.5 dB, thus 11 dB below the maximum SNR levels for conducted requirements (at 6% of Tx EVM).

Observation 2: Referring to Proposal 3 in [2] where Tx EVM has been set to 6%, we can note that 8RX UE can support 6 DL MIMO Layers using MCS17 under TDLD30-5 propagation environment. The SNR value is at 17.8 dB, thus 6.6 dB below the maximum SNR levels for conducted requirements (at 6% of Tx EVM).

# 3	Summary

In this paper, we provided some initial simulation results that should help in the definition of the PDSCH requirements. In the following, some useful observations that can be further discussed.

Observation 1: Referring to Proposal 3 in [2] where Tx EVM has been set to 6%, we can note that 8RX UE can support 8 DL MIMO Layers using MCS13 under TDLA30-10 propagation environment. The SNR value is at 13.5 dB, thus 11 dB below the maximum SNR levels for conducted requirements (at 6% of Tx EVM).

Observation 2: Referring to Proposal 3 in [2] where Tx EVM has been set to 6%, we can note that 8RX UE can support 6 DL MIMO Layers using MCS17 under TDLD30-5 propagation environment. The SNR value is at 17.8 dB, thus 6.6 dB below the maximum SNR levels for conducted requirements (at 6% of Tx EVM).

# References

- RP-221496, “Revised WID on Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)”, Huawei.

- R4-2218314, “UE demodulation and CSI reporting requirements for 8 Rx in FR1”, Ericsson.

# Appendix

(a) QPSK (MCS 4), Rank 8, TDLC30-100           (b) 16QAM (MCS 13), Rank 8, TDLA30-10

(c) 16QAM (MCS 13), Rank 6, TDLA30-10            (d) 64QAM (MCS 17), Rank 6, TDLD30-5

Fig. 1 8RX PDSCH simulation results depicting the normalized throughput as function of SNR, where Rank 6 and Rank 8 have been considered for 40 MHz/30 kHz (106 RBs).