3GPP TSG-RAN WG4 Meeting # 105                                                                R4-2218480

# Toulouse, France, November 14 – November 18, 2022

Title: 	Discussion on ATG co-existence system parameters and modeling

Source: 	CATT

Agenda item:	8.13.1.3

Document for:	Discussion

- Background

A WF[2] was agreed for the ATG co-existence simulation assumption, but many assumptions are still open. This contribution provides our views on the remaining issues for ATG co-existence scenario and network layout.

- Discussion

  - ATG BS and UE parameters

There’re not many open ATG BS parameters with some assumptions agreed in last meeting, such as 1 UE, coverage is 100km, etc.

Table 2: BS system parameters

| DL scheduled bandwidth per UE | Full BW |
| --- | --- |
| UL scheduled bandwidth per UE | Full BW |
| Traffic model | Full buffer |
| ATG BS maximum output power | [48 dBm] |
| ATG BS noise figure | 5dB |
| Handover margin | Not needed |
| NOTE: ATG BS max TX power is defined per polarization |  |

UE parameters are shown in the following table.

Table 3: UE system parameters

| ATG UE max TX power in dBm | [40 dBm] |
| --- | --- |
| ATG UE min TX power in dBm | The same as TN UE Note: It’s estimated that the min TN Tx power scenario desn’t happen for ATG UE because of the large distance between BS and UE, so it has sufficient margin for the co-existence simulation. |
| ATG UE noise figure | 9 dB |

  - Antenna and beamforming pattern modelling

For the BS vertical coverage range, when BS antenna height is assumed 25-30m, cell coverage is 100km, and UE altitude can be 3km-12km, the vertical coverage should be as large as possible. For the blind area, 10 degree equals 2km diameter, 20 degree equals 43km diameter. It should be decided what’s the diameter of the blind area and also considering the antenna performance. Current TN AAS antenna assumption is as following,

Table 6.2.3.2-1: AAS antenna parameters for 2GHz (copied from TR 38.863)

|  |  | Rural | Macro urban |
| --- | --- | --- | --- |
| 1 | Base Station Antenna Characteristics |  |  |
| 1.1 | Antenna pattern | TR 38.921[14] |  |
| 1.2 | Element gain (dBi) (Note 2) | 7.1 | 6.4 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V | 90º for H 65º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º | Linear ±45º |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 8 × 8 elements | 8 × 8 elements |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 |
| 1.9 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 25 | 25 |
| 1.10 | Base station maximum coverage angle in the horizontal plane (degrees) | 120 | 120 |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-120 |
| 1.12 | Mechanical downtilt (degrees) | 3 | 10 |

If ATG BS use uptilt and the same antenna as TN, it seems rural antenna is more suitable with uptilt.

Table 4: ATG BS AAS antenna parameters

| Antenna pattern | TR 38.921 |
| --- | --- |
| Element gain (dBi) | 7.1 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 8 × 8 elements |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V |
| Array Ohmic loss (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 25 |
| Base station maximum coverage angle in the horizontal plane (degrees) | 120 |
| Base station vertical coverage range (degrees) | 80-90 |
| Mechanical up-tilt (degrees) | 87 |

  - Other parameters and modeling

For TN BS and UE antenna parameters, they can refer TR 38.863. Other remaining parameters can also refer TR 38.863.

- Summary

This contribution provides our further consideration some of the ATG co-existence parameters.

Reference

[1] R4-2217788, “Email discussion summary for [104-bis-e][136] NR_ATG_UERF_part1”, CMCC

[2] R4-2217736, “WF on FR1 co-existence evaluation for ATG”, CMCC