3GPP TSG-RAN WG4 Meeting #105	R4-2218511

Toulouse, France, Nov 14 – 18, 2022

Agenda item:	10.2.1.1

Source: 	Qualcomm Incorporated

Title: 	On the UE capabilities for DL-PRS measurements with reduced number of samples

Document for:	Approval

# Discussion

A discrepancy in the definitions of the UE capabilities for PRS measurements with reduced number of samples was brought up in a recent LS from RAN2 [1]. It was observed that the UE capability for connected state (FG 27-3-1) is defined ‘per-band’ whereas the UE capability for inactive state (FG 14-2) is defined ‘per-UE.’ RAN2 asked RAN1 and RAN4 whether the definitions of the two capabilities should be harmonized. RAN1 responded in the affirmative and recommended that the definition of FG 14-2 be changed to ‘per-band’ [2].

We agree with RAN1’s recommendations and propose that the definition of FG 14-2 should be updated accordingly.

Proposal 1: Update the definition of FG 14-2 (PRS measurement for reduced sample in RRC_inactive state) to be ‘per-band.’

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. |
| --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-3-1 | M-sample measurements | The capability to support reporting a measurement based on measuring M=1 samples (instances) of a DL PRS resource set | 13-1 | No |  |

| Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| If the UE does not provide the capability, the UE is assumed to support M=4 only | per band | n/a | n/a | n/a | The candidate values are {1 [FFS others]} Need for location server to know if the feature is supported Note: The sample number M=1 does not account for the potential AGC sample Note: this feature is supported for both UE-assisted and UE based positioning | Optional with capability signaling |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. |
| --- | --- | --- | --- | --- | --- | --- |
| 14 NR_pos_enh | 14-2 | PRS measurement for reduced sample in RRC_inactive state | Capability of supporting reduced number of samples (M=1, 2) for PRS measurement in RRC_inactive state | 27-17 | no |  |

| Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| The reduced number of samples (M=1,2) for PRS measurement in RRC_inactive state cannot be supported. The UE is assumed to support M=4 only. | Per UE | No | No | N/A |  | Optional with capability signaling |

# Conclusions

Proposal 1: Update the definition of FG 14-2 (PRS measurement for reduced sample in RRC_inactive state) to be ‘per-band.’

Reply LS can be found in the Appendix.

# References

[1] R2-2208797, LS on DL-PRS measurements with reduced samples capability, RAN2#119-e

[2] R1-2207940, Reply LS on DL-PRS measurements with reduced samples capability, RAN1#110

# Appendix

3GPP TSG-RAN WG4 Meeting #105	R4-22XXXXX

Toulouse, France, Nov 14 – 18, 2022

Title:	Reply LS on UE capabilities for DL-PRS measurements with reduced number of samples

Response to:	R2-2208797 (LS on DL-PRS measurements with reduced samples capability) and R1-2207940 (Reply LS on DL-PRS measurements with reduced samples capability)

Release:	Release 17

Work Item:	NR_pos_enh-Core

Source:	RAN4

To:	RAN2, RAN1

Cc:

Contact Person:

- Name:	Carlos Cabrera-Mercader

- E-mail Address:	ccmercad@qti.qualcomm.com

Attachments: -

1. Overall Description:

RAN4 would like to thank RAN2 and RAN1 for their corresponding LSs concerning the UE capabilities for DL-PRS measurements with reduced number of samples (R2-2208797, R1-2207940).

RAN4 has discussed the discrepancy between the types of UE features FG 27-3-1 (M-sample measurements in RRC_CONNECTED) and FG 14-2 (PRS measurement for reduced sample in RRC_inactive state). RAN4 agrees with RAN1’s observation that a unified design is desirable and agrees with the recommendation to change the type of FG 14-2 to ‘per band.’

2. Actions:

To RAN2 and RAN1:

RAN4 kindly asks RAN2 and RAN1 to consider the above response in their future work.

3. Date of Next TSG-RAN4 Meetings:

TSG-RAN4 Meeting #106			Feb 27 – Mar 3, 2023, Athens, Greece

TSG-RAN4 Meeting #106-bis-e			Apr 17 – 26, 2023, Electronic Meeting