3GPP TSG-WG4 Meeting #105 	R4-2219052

# Toulouse, France, November 14th – 18th, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on remaining issues for NR_IIOT_URLLC performance requirements

Agenda item:	6.7.2

Document for:	Approval

Introduction

In RAN4#104bis-e meeting, averaged results were provided based on the simulation results provided in earlier meetings by different companies. In [1] we have presented our updated simulation results for TRS measurement accuracy.

In this paper we address these remaining open issues based on our updated simulation results.

Discussion

UE Rx-Tx time difference measurement requirement for TRS

In RAN4#104bis-e meeting, averaged results were provided based on the simulation results provided in earlier meetings by different companies. Based on the differences spotted, we have verified and updated our results from previous rounds, where the sampling frequency used in our algorithms was 2*N, achieving consequently higher resolution when defining the timing requirements.

The outcome of the 2nd round discussion was:

Issue 2-1: TRS measurement accuracy requirements

- Recommended WF

  - Agree on following tables with the FR2 120KHz SCS with TRS BW of 64 and 128 RBs in []:

|  |  | TUE-RX accuracy with TRS, 4 samples, AWGN, TDD |
| --- | --- | --- |
|  |  | Accuracy (Tc) |
| SCS [KHz] | TRS bandwidth RB | Average |
| 15 | 24 | 103 |
|  | 52 | 53 |
|  | 104 | 26 |

    - Average TUE-RX accuracy with TRS, 4 samples, AWGN, TDD, FR1

|  |  | TUE-RX accuracy with TRS, 4 samples, AWGN, TDD |
| --- | --- | --- |
|  |  | Accuracy (Tc) |
| SCS [KHz] | TRS bandwidth RB | Average |
| 60 | 24 | 26 |
|  | 64 | 13 |
|  | 132 | 7 |
| 120 | 32 | 13 |
|  | 64 | [6, 7] |
|  | 128 | [3, 4] |

    - Average TUE-RX accuracy with TRS, 4 samples, AWGN, TDD, FR2

  - Further discuss and agree on the TUE-RX accuracy for FR2 with 120KHz SCS with TRS BW of 64 and 128 RBs

  - Further clarify the differing results and agree on the averaged results marked FFS in the following table:

|  |  | TUE-RX accuracy with TRS, 4 samples, AWGN, TDD |
| --- | --- | --- |
|  |  | Accuracy (Tc) |
| SCS [KHz] | TRS bandwidth RB | Average |
| 30 | 24 | FFS |
|  | 48 | FFS |
|  | 132 | FFS |
| 60 | 24 | FFS |
|  | 64 | FFS |
|  | 132 | FFS |

    - Average TUE-RX accuracy with TRS, 4 samples, AWGN, TDD, FR1

Table 1 shows the collected results based on simulation results for TUE-RX with 4 samples TRS in AWGN provided in earlier meetings by other companies and with the updated results from Nokia:

|  |  | TUE-RX accuracy with TRS, 4 samples, AWGN, TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Accuracy (Tc) |  |  |  |  |  |  |  |  |  |
| SCS [KHz] | TRS bandwidth RB | Ericsson,R4-2212901, FR1 | Ericsson, R4-2212901, FR2 | Huawei, R4-2209237 | Huawei, R4-2209237 | Nokia, R4- 2219053, FR1 | Nokia,R4-2219053, FR2 | vivo, R4-2208821, FR1 | vivo, R4-2208821, FR2 | Qualcomm, R4-2210226 | Qualcomm, R4-2212207 |
| 15 | 24 | 114.0 |  | 88.0 |  | 129 |  | 128.0 |  | 120.5 |  |
|  | 52 | 59.0 |  | 56.0 |  | 64 |  | 62.3 |  | 56.8 |  |
|  | 104 | 29.0 |  | 24.0 |  | 31 |  | 30.4 |  | 29.9 |  |
| 30 | 24 | 58.0 |  | 40.0 |  | 64 |  | 62.3 |  | 56.4 |  |
|  | 48 | 29.0 |  | 28.0 |  | 31 |  | 30.5 |  | 28.3 |  |
|  | 132 | 14.0 |  | 16.0 |  | 15 |  | 14.8 |  | 15.1 |  |
| 60 | 24 | 29.0 | 29.0 |  | 24.0 | 31 | 31 | 30.4 | 30.4 |  | 28.9 |
|  | 64 | 14.0 | 14.0 |  | 14.0 | 15 | 15 | 14.8 | 14.8 |  | 14.3 |
|  | 132 | 7.2 | 7.0 |  | 8.0 | 8 | 8 | 7.3 | 7.3 |  | 7.4 |
| 120 | 32 |  | 14.0 |  | 16.0 |  | 16 |  | 14.5 |  | 14.3 |
|  | 64 |  | 7.0 |  | 7.0 |  | 8 |  | 7.3 |  | 7.2 |
|  | 128 |  | 4.0 |  | 4.0 |  | 4 |  | 3.6 |  | 3.7 |

Based on Table 1, we do average over companies’ results to compute the following table 2 and table 3

|  |  | TUE-RX accuracy with TRS, 4 samples, AWGN, TDD |
| --- | --- | --- |
|  |  | Accuracy (Tc) |
| SCS [KHz] | TRS bandwidth RB | Average |
| 15 | 24 | 116 |
|  | 52 | 60 |
|  | 104 | 29 |
| 30 | 24 | 56 |
|  | 48 | 29 |
|  | 132 | 15 |
| 60 | 24 | 29 |
|  | 64 | 14 |
|  | 132 | 7 |

Table 2 Average TUE-RX accuracy with TRS, 4 samples, AWGN, TDD, FR1

|  |  | TUE-RX accuracy with TRS, 4 samples, AWGN, TDD |
| --- | --- | --- |
|  |  | Accuracy (Tc) |
| SCS [KHz] | TRS bandwidth RB | Average |
| 60 | 24 | 29 |
|  | 64 | 14 |
|  | 132 | 7 |
| 120 | 32 | 15 |
|  | 64 | 7 |
|  | 128 | 4 |

Table 3 Average TUE-RX accuracy with TRS, 4 samples, AWGN, TDD, FR2

- Capture the averaged accuracy requirements in table 2 and table 3 in 38.133.

Conclusion

Based on the discussion and updated simulation results we propose capturing the averaged results in tables 2 and 3 the UE measurement accuracy requirements for TUE-RX accuracy with TRS, 4 samples, AWGN, TDD.

- Capture the averaged accuracy requirements in table 2 and table 3 in 38.133.

In [2] we have provided an updated CR proposal.

- References

[1] R4-2210953, Simulation Results for TRS measurement Accuracy, Nokia, Nokia Shanghai Bell

[2] R4-2219054, DraftCR on UE Rx-Tx time difference measurement accuracy requirements for RTT-based PDC, Nokia, Nokia Shanghai Bell