3GPP TSG-RAN WG4 Meeting #105                                           R4-2219145

Toulouse, France, November 14 – 18, 2022

Source: 	Huawei, HiSilicon

Title: 	On the co-existence study for NR duplex operation

Agenda Item:	8.18.2.1

Document for:	Approval

Introduction

In the last two RAN4 meetings, co-existence simulation assumptions have been discussed. Initial simulation results have also been provided by different companies. Related agreements can be found in [1~3]. In this contribution, we will provide our initial simulation results for some interested scenarios.

Discussion

Preliminary simulation results

According to the WF in [1], Urban Macro -> Urban Macro is the high priority scenario for both FR1 and FR2. Meanwhile, legacy TDD DL as victim and SBFD as aggressor is the co-existence study case with high priority.

Table 1: Scenarios for SBFD co-ex study in [1]

| FR | Scenario No. | Deployment Scenario (Aggressor -> Victim) | Priority |
| --- | --- | --- | --- |
| FR1 (4GHz) | 1 | Urban Macro -> Urban Macro | High |
|  | 2 | Indoor -> Indoor | Low |
| FR2 (30GHz) | 3 | Urban Macro -> Urban Macro | High |
|  | 4 | Urban Micro -> Urban Micro | Low |
|  | 5 | Indoor -> Indoor | Low |

In the latest agreed WF [3], a clarification for the scenario has been captured.

Table 2: Scenarios for SBFD co-ex study in [2]

| Victim | Aggressor | Figures | Aggressor baseline | Priority |
| --- | --- | --- | --- | --- |
| NR TDD DL | SBFD (DUD) | Case 1 | NR TDD DL | High |
|  | SBFD (DU) | Case 2 | NR TDD DL | High |
|  |  | Case 3 | NR TDD DL | Low |
| NR TDD UL | SBFD(DUD) | Case 4 | NR TDD UL | Low |
|  | SBFD(DU) | Case 5 | NR TDD UL | Low |
|  |  | Case 6 | NR TDD UL | Low |
| SBFD (DUD) | NR TDD DL | Case 1 | No system in adjacent channel | High |
| SBFD (DU) | NR TDD DL | Case 2 |  | High |
|  |  | Case 3 |  | Low |
| SBFD(DUD) | NR TDD UL | Case 4 |  | Low |
| SBFD(DU) | NR TDD UL | Case 5 |  | Low |
|  |  | Case 6 |  | Low |
| Note: The above combination sets may be down-scaled if some sets are equivalent in SLS study perspective after agreed on other assumptions. |  |  |  |  |

Thus we would like to provide initial simulation results for FR2 as below. Most of the simulation assumptions are aligned with WF in [1].

Fig. 1 FR2 Macro-Macro SBFD co-ex with legacy TDD: SINR and Throughput CDF curve

Table 3. Initial results for SBFD co-ex with legacy TDD in FR1 Macro-Macro scenario

We pick @50% observation point and find out that the SINR degradation could be 0.12dB, which is 0.36% to the legacy TDD DL system. Thus we have the following observation.

Observation: The preliminary simulation results for FR2 Macro-Macro SBFD co-ex with legacy TDD show that the DL SINR degradation @50% CDF of the victim legacy TDD system is 0.12dB (%0.36).

Conclusion

In this contribution we discussed on co-existence study for NR duplex operation. According to the contribution, we have the following observations and proposals:

Observation: The preliminary simulation results for FR2 Macro-Macro SBFD co-ex with legacy TDD show that the DL SINR degradation @50% CDF of the victim legacy TDD system is 0.12dB (%0.36).

- References

- R4-2214378, “WF on adjacent channel co-existence study”, Samsung, 3GPP TSG-RAN WG4 Meeting #104-e, Electronic meeting, August 15 – 26, 2022.

- R4-2214379, “WF on Simulation assumption for adjacent co-existence study”, CMCC, 3GPP TSG-RAN WG4 Meeting #104-e, Electronic meeting, August 15 – 26, 2022.

- R4-2217466, “WF on adjacent channel co-existence evaluation of SBFD operation”, Samsung, CMCC, 3GPP TSG-RAN WG4 Meeting #104bis-e, Electronic meeting, October 10 – 19, 2022.