3GPP TSG-RAN WG4 Meeting #105	R4-2219327

Toulouse, France, Nov. 14 – Nov. 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2745 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on cell reselection in Idle mode for NR-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-11-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The spec. specifies that UE shall initiate cell selection procedures for the selected PLMN after 10s. However, considering CCA failure in NR-U, the cell selection evaluation time is farly larger than 10s which implies the UE will initiate cell selection for the selected PLMN regardless of UE finishing once serving cell evaluation in NR-U. DRX cycle length [s]; Nserv_CCA [number of DRX cycles]; Tevaluate,NR_CCA [s] (number of DRX cycles). 0.32; M1*4+M1*Ms; 0.32x(16+Me) x M2 {(16+Me)xM2}. 0.64; M1*4+M1*Ms; 0.64x(8+ Me) {8+ Me}. 1.28; 2+Ms; 1.28x(5+ Me) {5+ Me}. 2.56; 2+Ms; 2.56x(3+ Me) {3+ Me}. |
| Summary of change: |  | - Introduce the max function between the fixed value 10s, scaling factor with DRX cycles and CCA failure number P1s.Add the inter-RAT LTE measurement delay requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may unecessarily and frequently initiate cell selection procedures for the selected PLMN causing severe performance degradation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2A.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------NEXT CHANGE----------------------------

#### 4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell

The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every (1+Mn)*M1*N1 DRX cycles in Nserv_CCA consecutive DRX cycles; where:

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤  0.64 second,

-	otherwise M1=1.

-	N1*Mn is the maximum separation in DRX cycles between two measurements that are used for filtering.

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2 but not separated in time by more than N1*Mn, where Mn=2.

If the UE has evaluated according to Table 4.2A.2.2-1 in Nserv_CCA consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.

UE shall initiate measurements on neighbour cells indicated by the serving cell if it is unable to measure on the serving cell for at least N1*Mp consecutive number of DRX cycles each with at least one SMTC occasion not available at the UE, where Mp=4 when DRX cycle length <1.28 s, Mp=2 when DRX cycle length ≥1.28 s.

UE shall initiate the measurements on neighbour cells of any intra-frequency or inter-frequency if it is unable to measure on serving cell during at least consecutive N1*Mq number of DRX cycles each with at least one SMTC occasion not available at the UE, regardless of any condition of SnonIntraSearchP and SnonIntraSearchQ, where Mq=8 when DRX cycle length <1.28 s, Mq=4 when DRX cycle length ≥1.28 s.

If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information for 10 s, the UE is allowed to further search any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information till T = max(10 s, (K1+ P1s)*TDRX ) before initiating cell selection procedures for the selected PLMN as defined in TS 38.304 [1], where:

- K1=16 if DRX cycle is 0.32s, or 8 if DRX cycle is 0.64s,

Otherwise, K1=4.

- P1s  is the number of DRX cycles each with at least one SMTC occasion not available during the TPLMN and P1s ≤ P1s,max.

- P1s,max = [64] if DRX cycle is 0.32s, or [32] if DRX cycle is 0.64s;

Otherwise, P1s,max = [16].

The UE shall initiate cell selection procedures for the selected PLMN if P1s exceeds P1s,max.

Table 4.2A.2.2-1: Nserv_CCA

| DRX cycle length [s] | Scaling Factor (N1) |  | Nserv_CCA [number of DRX cycles] |
| --- | --- | --- | --- |
|  | FR1 | FR2-2 |  |
| 0.32 | 1 | [8] | N1*M1*(4+ Ms) |
| 0.64 |  | [5] | N1*M1*(4+ Ms) |
| 1.28 |  | [4] | N1*(2+Ms) |
| 2.56 |  | [3] | N1*(2+Ms) |
| Note 1: Ms is the number of groups of consecutive N1 DRX cycles each group with at least one SMTC occasion not available at the UE during Nserv_CCA, and Ms< Ms,max Note2: Ms,max=8 for DRX cycle length < 1.28 s, Ms,max= 4 for DRX cycle length ≥ 1.28 s. |  |  |  |

The UE shall restart the measurements used for serving cell evaluation if Ms exceeds Ms,max.

----------------------END OF CHANGES----------------------------