3GPP TSG-RAN WG4 Meeting # 105-e	R4-2219511

# Toulouse, France, November 14 – November 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.101-4 | CR | <CR#> | rev | <Rev#> | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of FR2-2 PDSCH performance requirements for CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei.HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf |  |  |  |  |  | Date: |  |  | 2022-10-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the work plan, RAN 4 should submit the draft CR for FR2-2 UE and BS performance requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced FR1+FR2-2 CA PDSCH performance requirments in TS 38.101-4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.2A	PDSCH demodulation for CA

9.2A.1	NR CA between FR1 and FR2

The performance requirements for SCell on FR2-2 band are specified in Table 9.2A.1-3, with the additional test parameters for SCell which are specified for FR2-2 PDSCH requirements in Table [7.2.2.2.1-2], the test parameters for PCell in Table 9.2A.1-2 and the downlink physical channel setup according to Annex C.3.1. In this test, Pcell is in FR1 and Scell is in FR2-2 and only requirements for Scell should be verified.

The test purposes are specified in Table 9.2A.1-1.

Table 9.2A.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance in FR2-2 Scell CC in CA between FR1 and FR2-2 under 2 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1,1-2,1-3,1-4,1-5,1-6 |

Table 9.2A.1-2: Test parameters for PCell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| TDD pattern |  |  | FR1.30-1 |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Dmrs-AdditionalPosition |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 9.2A.1-3: Minimum performance for Rank 1 (FRC) in Scell CC

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of max through-put (%) | SNRBB (dB) |
| 1-1 | TBD | [400 / 120] | QPSK, 0.30 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | 70 | TBD |
| 1-2 | TBD | [400 / 120] | 16QAM, 0.48 | FR2.120-1 | TDLA30-200 | 2x2 ULA Low | 70 | TBD |
| 1-3 | TBD | [400 / 120] | 16QAM, 0.48 | FR2.120-1 | [TDLA30-650] | 2x2 ULA Low | 30 | TBD |
| 1-4 | TBD | [400 / 120] | 64QAM, 0.43 | FR2.120-1 | TDLD30-200 | 2x2 ULA Low | 70 | TBD |
| 1-5 | TBD | [400 / 480] | QPSK, 0.30 | FR2.480-1 | TDLA10-200 | 2x2 ULA Low | 70 | TBD |
| 1-6 | TBD | [400 / 480] | 16QAM 0.48 | FR2.480-1 | TDLD10-200 | 2x2 ULA Low | 70 | TBD |