3GPP TSG-RAN WG4 Meeting #105                                		R4-2219616

Toulouse, France, 14 November– 18 November, 2022

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.876 to capture system parameter assumption and antenna modelling

Agenda Item:	8.13.1.3

Document for:	Approval

# 1. Introduction

In RAN4#104#bis meeting, RAN4 reached some agreements on system parameter assumption and antenna modelling based on the WF [1]. In this paper, we’d like to capture these agreements into this TR.

# 2.	References

[1] R4-2217736, WF on FR1 co-existence evaluation for ATG, CMCC

# Text proposal for TR 38.876

<<Start of TP for TR 38.876>>

6.2.2 System parameters

6.2.2.1 ATG parameters

The initial system parameters for ATG BS is assumed as below.

Table 6.2.2.1-1: system parameters for ATG BS

| ATG BS altitude | 30m |
| --- | --- |
| Carrier frequency | 2GHz, 4GHz |
| Frequency reuse factor | 1 |
| Duplex mode | FDD@2GHz, TDD@4GHz |
| Channel bandwidth | 20MHz@2GHz, 100MHz@4GHz |
| Subcarrier spacing (SCS) | 15k@2GHz, 30k@4GHz |
| Number of cells | TBC |
| Environment1 | Rural |
| UE distribution | TBC |
| Indoor UE percentage | 0% |
| Number of DL active UEs per cell (NOTE 2) | 1 |
| Number of UL active UEs per cell (NOTE 2) | 1 |
| DL scheduled bandwidth per UE | 1 |
| UL scheduled bandwidth per UE | 1 |
| Traffic model | Full buffer |
| ATG BS maximum output power | 43dBm@2GHz 49dBm@4GHz |
| ATG BS noise figure | 5dB |
| Handover margin | 3dB |
| NOTE 1: ATG BS is assumed to serve UEs in the rural environment. NOTE 2: Same as the number of BS beam(s); NOTE 3: ATG BS max TX power is defined per polarization |  |

6.2.2.2 ATG UE parameters

6.2.2.3 TN BS and UE parameters

The system parameters for TN BS and TN UE is assumed as below.

Table 6.2.2.3-1: system parameters for TN BS and UE

| Parameters | Rural | Rural |
| --- | --- | --- |
| Carrier frequency | 2GHz | 4GHz |
| Channel bandwidth | 20MHz | 100MHz |
| Scheduled channel bandwidth per UE (DL) | 1 | 1 |
| Scheduled channel bandwidth per UE (UL) | 3 | 3 |
| The number of active UE (DL) (Note 1) | 1 | 1 |
| The number of active UE (UL) (Note 1) | 3 | 3 |
| Traffic model | full buffer | full buffer |
| DL power control | No | No |
| UL power control | Yes | Yes |
| TN BS-UE min distance in meters | 30m | 30m |
| TN BS max TX power in dBm | 43dBm | 49dBm |
| TN UE max TX power in dBm | 23dBm | 23dBm |
| TN UE min TX power in dBm | -40dBm | -40dBm |
| TN BS Noise figure in dB | 5dB | 5dB |
| TN UE Noise figure in dB | 9dB | 9dB |
| Handover margin | 3dB | 3dB |
| Note 1 Same as the number of BS beam(s); Note 2: TN BS max TX power is defined per polarization |  |  |

6.2.3 Antenna and beamforming pattern modelling

6.2.3.1 ATG BS antenna model

6.2.3.2 ATG UE antenna model

6.2.3.3 TN BS antenna model

6,2,3,4 TN UE antenna model

The following assumption for TN UE antenna is shown as below.

Table 6.2.3.4-1: TN handheld UE antenna charateristic

| Characteristics | Handheld |
| --- | --- |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Tx/Rx Antenna gain | 0 dBi per element |
| the number of Tx and Rx | 1T2R |

<<End of TP for TR 38.876>>