3GPP TSG-RAN WG4 Meeting #105                                R4-2219630

Toulouse, France, 14 November– 18 November, 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-18 NTN coexistence study assumption

Agenda Item:	8.23.2

Document for:	Approval

Introduction

In RAN4#104bis-e meeting, a WF [1] on NR_NTN_enh_Part2 was approved with some open issues for further discussion. Frameworks and simulation assumptions for NTN co-existence study in bands above 10GHz were captured into the contribution [2]. In this paper, we’d like to discuss these open issues and share our views.

Link budget calculation

In clause 6.1.1.1 of TR 38.821 [3], the following tables representing two sets of satellite parameters are considered as the baseline for system level simulator calibration:

Table 1: Set-1 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| Note 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. Note 2: This beam size refers to the Nadir pointing of the satellite Note 3: All these satellite parameters are applied per beam. Note 4: The EIRP density values are considered identical for all frequency re-use factor options. Note 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Table 2: Set-2 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 2 m | 0.2 m | 0.2 m |
| Satellite EIRP density |  | 32 dBW/MHz | 2 dBW/MHz | -4 dBW/MHz |
| Satellite Tx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi |
| 3dB beamwidth |  | 0.4412 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter (Note 2) |  | 280 km | 90 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 1.33 m | 0.13 m | 0.13 m |
| G/T |  | 20 dB K-1 | 5 dB K-1 | 5 dB K-1 |
| Satellite Rx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi |
| Note 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. Note 2: This beam size refers to the Nadir pointing of the satellite Note 3: All these satellite parameters are applied per beam. Note 4: The EIRP density values are considered identical for all frequency re-use factor options. |  |  |  |  |

The following table is agreed for UE characteristics for System Level Simulations

Table 3: Ka band VSAT characteristics for system level simulations

| Characteristics | VSAT (Note 2) | Other (Note 1) |
| --- | --- | --- |
| Frequency band | Ka band(i.e. 30 GHz UL and 20 GHz DL) | Ka band(i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) |
| Polarisation | circular | Linear : +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | TBD dBi per element |
| Antenna temperature | 150 K | TBD K |
| Noise figure | 1.2 dB | TBD dB |
| Tx transmit power | 2 W (33 dBm) | [TBD W (TBD dBm)] |
| Tx antenna gain | 43.2 dBi | TBD dBi per element |

Based on the parameters above, we calculate the link budget by using the following formula.

SNR = EIRP – pathloss + G/T - 10*log10(kB)

For LEO-600, LEO-1200, GSO, we summarized the following link budget calculation.

Table 4: DL link budget for each scenario

|  | Units | LEO-600 DL | LEO-1200 DL | GSO DL |
| --- | --- | --- | --- | --- |
| Satellite EIRP density | dBm/MHz | 34.0 | 40.0 | 70.0 |
| Channel bandwidth | MHz | 200.0 | 200.0 | 200.0 |
| Satellite EIRP | dBm | 57.0 | 63.0 | 93.0 |
| Tx antenna gain | dBi | 38.5 | 38.5 | 58.5 |
| satellite Maximum TRP | dBm | 18.5 | 24.5 | 34.5 |
| center frequency | GHz | 20.0 | 20.0 | 20.0 |
| Disctance | meter | 600000.0 | 1200000.0 | 36000000.0 |
| Free space path loss | dB | 174.0 | 180.1 | 209.6 |
| Received power level at VSAT OTA | dBm | -117.0 | -117.0 | -116.6 |
| Rx antenna Gain for VSAT | dBi | 39.7 | 39.7 | 39.7 |
| antenna temperature | K | 150.0 | 150.0 | 150.0 |
| Thermal noise temperature | K | 290.0 | 290.0 | 290.0 |
| Noise figure for VSAT receiver | dB | 1.2 | 1.2 | 1.2 |
| G/T for VSAT | dB/K | 15.9 | 15.9 | 15.9 |
| SNR | dB | 14.4 | 14.4 | 14.9 |

Table 5: UL link budget for each scenario

|  | Units | LEO-600 UL | LEO-1200 UL | GSO UL |
| --- | --- | --- | --- | --- |
| VSAT EIRP density | dBm/MHz | 53.2 | 53.2 | 53.1 |
| Channel bandwidth | MHz | 200.0 | 201.0 | 202.0 |
| VSAT EIRP | dBm | 76.2 | 76.2 | 76.2 |
| Tx antenna gain | dBi | 43.2 | 43.2 | 43.2 |
| VSAT Maximum TRP | dBm | 33.0 | 33.0 | 33.0 |
| center frequency | GHz | 30.0 | 30.0 | 30.0 |
| Disctance | meter | 600000.0 | 1200000.0 | 36000000.0 |
| Free space path loss | dB | 177.6 | 183.6 | 213.1 |
| Received power level at SAN OTA | dBm | -101.4 | -107.4 | -136.9 |
| Rx antenna Gain for satellite | dBi | 38.5 | 38.5 | 58.5 |
| Rx antenna temperature | K | 40.0 | 40.0 | 40.0 |
| Thermal noise temperature | K | 290.0 | 290.0 | 290.0 |
| Noise figure for satellite receiver | dB | 3.2 | 3.2 | 6.7 |
| G/T for satellite | dB/K | 13.0 | 13.0 | 28.0 |
| SNR | dB | 27.2 | 21.2 | 6.6 |

It seems that the performance of VSAT is better than LEO satellite. DL SNR is about 14.4dB for LEO, but UL SNR is 27.2dB and 21.2dB for LEO-600 and LEO-1200. Thus, RAN4 should revisit these parameters before performing co-existence simulation.

Proposal 1: RAN4 should revisit these parameters for LEO-600 and LEO-1200 scenarios, e.g. Tx/Rx antenna gain, Satellite EIRP density and G/T.

In addition, one set of parameters for VSAT is not enough to cover both LEO and GSO scenarios. Thus, it’s necessary to develop a new set of parameters for VSAT in LEO-600 and LEO-1200 scenarios.

Proposal 2: it’s necessary to develop a new set of parameters for VSAT in LEO-600 and LEO-1200 scenarios.

Currently, all the parameters and antenna pattern are provided based on dish antennas for both SAN and VSAT. Not sure whether RAN4 need to simulate the coexistence considering both dish antennas and phased array antennas. However, from coexistence study perspective, RAN4 need to discuss whether we only assume dish antennas for both SAN and VSAT or we can assume phased array antennas. At least, we don’t have enough TU or resources to simulate these two kinds of device.

Proposal 3: There is no enough TU or resources to assume both dish antenna and phased array antenna for SAN and UE. RAN4 need to make a decision which kind of antenna is the first priority for coexistence simulation.

G/T calculation

In last meeting, companies proposed the following noise figure based on G/T assumption.

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 28 | 13 | 13 |
| G_Rx (dBi) | 58.5 | 38.5 | 38.5 |
| NF (dB) | 5.9 | 0.9 | 0.9 |

However, referring to companies’ contribution [4], following formula applies for the computation of G/T or figure of merit, in dB:

G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF))

Where:

-     Antenna Gain : Ga in dBi

-     Ambient Temperature : T0 (usually 290 K)

-     Antenna temperature : Ta

-     Noise Figure: NF in dB including feeder loss

If we assume Ta = 40K, the NF in including feeder loss is about 3.2dB for LEO and 6.7dB for GSO. However, 2dB is assumed for the cable loss, so 3.2dB~6.7dB NF are still too ideal and are higher than the assumption in FR1 NTN.

Proposal 4: 1.1dB/K is proposed for LEO SAN G/T for simulation study.

Propagation model between NTN UE and TN UE

In last meeting, we have an open issue about the propagation model between NTN UE and TN UE. The proposal is listed as below.

  - Option 1: Propagation model between NTN UE and TN UE should reference to the following propagation model,

- UE-to-UE: Umi (h_BS=1.5 m ~ 22.5 m)

+ penetration loss see TR 38.803

It’s need to clarify whether UE-to-UE: Umi (h_BS=1.5 m ~ 22.5 m) refer to the following propagation model. If so, it’s proposed to draft the assumption explicitly.

Proposal 5: it’s proposed to explicitly draft propagation model between NTN UE and TN UE as below.

| Scenario | Pathloss [dB], fc is in GHz and d is in meters (6) | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- |
| UMi - Street Canyon LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m |
| UMi – Street Canyon NLOS |  | σSF=7.82 | 10 m < d2D < 5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m Explanations: see note 4 |
| Note 1: d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In UMi scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – 1.0 m, h'UT = hUT–1.0 m, where hBS and hUT are the actual antenna heights, and the effective environment height is assumed to be equal to 1.0 m. In UMa scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. Note 2: The applicable frequency range of the PL formula in this table is 0.8 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified formatand and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |

Summary

Based on the discussion above, the following observations and proposals are listed.

Proposal 1: RAN4 should revisit these parameters for LEO-600 and LEO-1200 scenarios, e.g. Tx/Rx antenna gain, Satellite EIRP density and G/T.

Proposal 2: it’s necessary to develop a new set of parameters for VSAT in LEO-600 and LEO-1200 scenarios.

Proposal 3: There is no enough TU or resources to assume both dish antenna and phased array antenna for SAN and UE. RAN4 need to make a decision which kind of antenna is the first priority for coexistence simulation.

Proposal 4: 1.1dB/K is proposed for LEO SAN G/T for simulation study.

Proposal 5: it’s proposed to explicitly draft propagation model between NTN UE and TN UE as below.

| Scenario | Pathloss [dB], fc is in GHz and d is in meters (6) | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- |
| UMi - Street Canyon LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m |
| UMi – Street Canyon NLOS |  | σSF=7.82 | 10 m < d2D < 5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m Explanations: see note 4 |
| Note 1: d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In UMi scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – 1.0 m, h'UT = hUT–1.0 m, where hBS and hUT are the actual antenna heights, and the effective environment height is assumed to be equal to 1.0 m. In UMa scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. Note 2: The applicable frequency range of the PL formula in this table is 0.8 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified formatand and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |

- References

[1] R4-2217468, WF on [313] NR_NTN_enh_Part2, Samsung

[2] R4-2217469, Simulation assumptions for NTN co-existence study in bands above 10GHz, Samsung

[3] TR 38.821, Solutions for NR to support non-terrestrial networks (NTN)

[4] R4-2215348, VSAT UE Characteristics and Initial Simulation Parameters, THALES