3GPP TSG-RAN WG4 Meeting #105	R4-2219699

Toulouse, France, 14 November – 18 November 2022

Source:	Skyworks Solutions, Inc.

Title:	MSD test point simplification for EN-DC

Agenda item:			8.3.3 Simplification of specification and reduction of test burden

Document for:	Approval

# 1. Introduction

In the late development stages of the Release 17, a new MSD table template was adopted in TS 38.101-1 that helped achieved a significant reduction of TS complexity, improved readability, simplified future TS maintenance while addressing concerns on RAN4 workload due to the new BCS4 concept [1,2]. It took RAN4 several meetings and WF to reach this agreement. Due to its late implementation, several test points were simply copied and pasted from the old template into the new template. This resulted in introducing some test points that do not entirely comply with the agreed WF [4]. The lack of time also prevented porting from the new template to EN-DC TS 38.101-3. In this contribution, we believe that the path to a successful migration to the new template would first require reviewing and correcting the remaining Release 17 NR-CA test points. Then, for a given band combination, whenever a difference in MSD specifications between EN-DC and its corresponding NR-CA combination occurs, RAN 4 would need to reach agreement on how to resolve these differences. In this initial work, we propose additional simplification guidelines based on an attempt to migrate the EN-DC uplink (UL) harmonic MSD test points to the new template.

# 2. Discussion

Considering that Reference sensitivity exception requirements for intra-band contiguous, intra-band non-contiguous, and MSD due to dual uplink interference are already in a compact form, the scope of EN-DC MSD table simplification is reduced to MSD due to harmonic interference (both Tx harmonic and Rx harmonic mixing), and MSD due to cross-band isolation.

Observation 1: The scope of MSD test point simplification for TS 38.101-3 is restricted to the following 6 tables:

- Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1 (Table 7.3B.2.3.1-1),

- Reference sensitivity QPSK PREFSENS (EN-DC with n46) (Table 7.3B.2.3.1-3),

- Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1 (Table 7.3B.2.3.2-1),

- Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1 (Table 7.3B.2.3.2-1a),

- Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1 (Table 7.3B.2.3.4-1),

- Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 EN-DC in NR FR1 (Table 7.3B.2.3.4-1a).

## 2.1 Required Corrections in Rel-17 to Deliver a Stable R18 Baseline

The new MSD template was ported to TS 38.101-1 via one CR for MSD due to harmonic interference and another CR for MSD due to cross-band isolation [1,2]. Due to the late introduction into the TS and priority of RAN4 priorityfocus on Rel-18, we note that the latest Release 17.7.0 TS requires further maintenance work to ensure a solid baseline that can be used to work on R18 optimization. The corrections fall into three categories:

- Alignment of NR-CA receiver (Rx) harmonic mixing test points with legacy agreements reached for EN-DC. For EN-DC, it was agreed to remove UL3/DL2 Rx harmonic mixing test points due to DL interference level being substantially lower than receiver thermal noise [3],

- Corrections to cross-band isolation test points. In CR [1], and to reach quick consensus, the test points were simply copied from the previous template into the new template. This resulted in several test points that were not compliant with the agreed WF [4].  We provided some corrections based on measurements in [5], but since then, no agenda item allowed companies to comment on these corrections.

- Corrections needed due to agreed power class (PC) 2 MSD test points missing in RAN4#98 bigCR.

Observation 2: Several essential corrections to the Release 17.7.0 NR-CA (TS 38.101-1) MSD test points due to harmonic interference and cross band isolation are required before any optimization can start on TS 38.101-3.

These three issues are addressed at this meeting via CRs [6,7]. However, it is possible that in future meetings further corrections will be necessary to Release 17.

We consider the migration of EN-DC Release 18 test points to the new template should be done across all tables simultaneously, like it was done for NR-CA Release 17. This requires that NR-CA Release 17 test points are first corrected.

Proposal 1: The migration of EN-DC MSD test points due to harmonic interference and cross-band isolation interference should be implemented simultaneously across all tables as was done for Release 17. To do so, the NR-CA Release 17 MSD test points should be corrected first.

Assuming the objectives summarized in proposal 1 are met, further work may be required. For a given pair of frequency bands:

- Is it agreeable that in case an MSD test point is missing in one TS (say 38.101-1), but the MSD level has been agreed in the other TS (say 38.101-3), can the agreed test point be automatically adopted as a common test point to both TS? For example, n71 MSD Rx harmonic mixing is specified for DC_2_n71 but it is not for CA_n2-n71. Can the CA_n2-n71 adopt the DC_2_n71 test point? Counter example: DC_3_n77 UL harmonic MSD test points are missing, yet they are specified for CA_n3-n77.

- In case test points were present in both TS, but MSD levels are different, can the worst-case MSD level be adopted as a common test point to both TS?

Proposal 2: For a given pair of frequency bands and for identical UL/DL CBW configurations, in case an MSD test point is missing in one TS (say 38.101-1), but has been agreed in the other TS, automatically port the agreed test point to the TS where the specification is missing.

Proposal 3: For a given pair of frequency bands and for identical UL/DL CBW configurations, in case the MSD levels differ between the TS 38.101-1 and TS 38.101-3, adopt the highest MSD level and align both TS test points.

## 2.2 Example of EN-DC MSD Simplification for MSD due to Harmonic Interference.

Table 1 shows an attempt of porting the NR-CA Rel-17 table template to EN-DC Table 7.3B.2.3.1-1 “MSD due to UL harmonic.”  Although the number of test points is reduced, the table size remains considerable due to the 2 test points per band combination and repeating test points for swapping of NR and LTE. For example, a pair of test points is specified with the exact same MSD levels for both DC_12_n66 and for DC_66_n12. To further simplify EN-DC we propose:

Proposal 4: Only 1 test point per band combination is specified, that which defines the affected DL band smallest CBW.

Proposal 5: For a given pair of frequency bands and in case the smallest CBW is common to both the LTE and the NR DL cell groups, a single test point is specified for combinations where the LTE and NR cell groups are swapped, like DC_12_n66 / DC_66_n12.  It is sufficient to capture only the MSD of the DL affected band smallest CBW. Using the example of DC_12_n66/DC_66_n12, we propose to reduce the table complexity from 4 entries to only 1 entry as shown below.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | Difference to NR-CA | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 12 n12 | n66 66 | 5 | 15 | 8 (RBstart=0) | 5 | 10 |  | UL3/DL1 direct-hit |

Table 1: Porting the Release 17 NR-CA table template to EN-DC Table 7.3B.2.3.1-1 for MSD due to UL harmonic

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | Difference to NR-CA | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 1 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 |  | UL2/DL1 direct-hit |
| 1 | n77 | 20 | 15 | 100 (RBstart=0) | 100 | 13.8 |  | UL2/DL1 direct-hit |
| 1 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 |  | UL2/DL1 near-miss |
| 2 | n48 | 5 | 15 | 25 (RBstart=0) | 5 | 27.3 | +0.2 dB | UL2/DL1 direct-hit |
| 2 | n48 | 10 | 15 | 50 (RBstart=0) | 1007 | 14 | +0.2 dB | UL2/DL1 direct-hit |
| 2 | n48 | 5 | 15 | 25 (RBstart=0) | 10 | 1.9 |  | UL2/DL1 near-miss |
| n2 | 48 | 5 | 15 | 25 (RBstart=0) | 5 | 27.3 | +0.2 dB | UL2/DL1 direct-hit |
| n2 | 48 | 10 | 15 | 50 (RBstart=0) | 1007 | 14 | +0.2 dB | UL2/DL1 direct-hit |
| n2 | 48 | 5 | 15 | 25 (RBstart=0) | 10 | 1.9 |  | UL2/DL1 near-miss |
| 2 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 |  | UL2/DL1 direct-hit |
| 2 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 |  | UL2/DL1 direct-hit |
| 2 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 |  | UL2/DL1 near-miss |
| 2 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 |  | UL2/DL1 direct-hit |
| 2 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 |  | UL2/DL1 direct-hit |
| 2 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 |  | UL2/DL1 near-miss |
| 3 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | missing – ported from NR-CA | UL2/DL1 direct-hit |
| 3 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | missing – ported from NR-CA | UL2/DL1 direct-hit |
| 3 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | missing – ported from NR-CA | UL2/DL1 near-miss |
| 3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 |  | UL2/DL1 direct-hit |
| 3 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 |  | UL2/DL1 direct-hit |
| 3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 |  | UL2/DL1 near-miss |
| n3 | 42 | 5 | 15 | 25 (RBstart=0) | 5 | 27.3 |  | UL2/DL1 direct-hit |
| n3 | 42 | 5 | 15 | 25 (RBstart=0) | 20 | 21.3 |  | UL2/DL1 direct-hit |
| n3 | 42 | 5 | 15 | 25 (RBstart=0) | 5 | 1.9 |  | UL2/DL1 near-miss |
| 4 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 |  | UL2/DL1 direct-hit |
| 4 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 |  | UL2/DL1 direct-hit |
| 4 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 |  | UL2/DL1 near-miss |
| 5 | n778 | 5 | 15 | 16 (RBstart=0) | 10 | 10.5 |  | UL4/DL1 direct-hit |
| 5 | n778 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 |  | UL4/DL1 direct-hit |
| 5 | n778 | 5 | 15 | 16 (RBstart=0) | 10 | 10.4 |  | UL5/DL1 direct-hit |
| 5 | n778 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 |  | UL5/DL1 direct-hit |
| 5 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.5 |  | UL4/DL1 direct-hit |
| 5 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 |  | UL4/DL1 direct-hit |
| 7 | n79 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 |  | UL2/DL1 near-miss |
| 8 | n313 | N/A | N/A | N/A | N/A | N/A |  | UL2/DL1 direct-hit |
| 8 | n7 | 5 | 15 | 8 (RBstart=0) | 5 | 10 |  | UL3/DL1 direct-hit |
| 8 | n7 | 5 | 15 | 25 (RBstart=0) | 50 | 1.3 | +0.2 dB | UL3/DL1 direct-hit |
| 8 | n41 | 5 | 15 | 16 (RBstart=0) | 10 | 13 |  | UL3/DL1 direct-hit |
| 8 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 3.5 |  | UL3/DL1 direct-hit |
| 8 | n77 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 |  | UL4/DL1 direct-hit |
| 8 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 |  | UL4/DL1 direct-hit |
| 8 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 |  | UL4/DL1 direct-hit |
| 8 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 |  | UL4/DL1 direct-hit |
| 8 | n79 | 5 | 15 | 16 (RBstart=0) | 10 | 12.0 | missing – ported from NR-CA | UL5/DL1 direct-hit |
| 8 | n79 | 5 | 15 | 25 (RBstart=0) | 100 | 4.4 |  | UL5/DL1 direct-hit |
| n8 | n313 | N/A | N/A | N/A | N/A | N/A |  | UL2/DL1 direct-hit |
| n8 | 7 | 5 | 15 | 8 (RBstart=0) | 5 | 10 |  | UL3/DL1 direct-hit |
| n8 | 7 | 5 | 15 | 25 (RBstart=0) | 20 | 5.3 |  | UL3/DL1 direct-hit |
| 12 | n66 | 5 | 15 | 8 (RBstart=0) | 5 | 10 | 10dB for 10MHz for NR-CA | UL3/DL1 direct-hit |
| 12 | n66 | 5 | 15 | 20 (RBstart=0) | 40 | 2.4 |  | UL3/DL1 direct-hit |
| 12 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 |  | UL5/DL1 direct-hit |
| 12 | n77 | 5 | 15 | 20 (RBstart=0) | 100 | 0.7 |  | UL5/DL1 direct-hit |
| 12 | n78 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 |  | UL5/DL1 direct-hit |
| 12 | n78 | 5 | 15 | 20 (RBstart=0) | 100 | 0.7 |  | UL5/DL1 direct-hit |
| n12 | 48 | 5 | 15 | 10 (RBstart=0) | 5 | 13 | NR-CA smallest CBW=10MHz | UL5/DL1 direct-hit |
| n12 | 48 | 5 | 15 | 25 (RBstart=0) | 20 | 7.8 | NR-CA highest CBW=40MHz | UL5/DL1 direct-hit |
| n12 | 66 | 5 | 15 | 8 (RBstart=0) | 5 | 10 | 10dB for 10MHz for NR-CA | UL3/DL1 direct-hit |
| n12 | 66 | 5 | 15 | 20 (RBstart=0) | 20 | 5.5 | NR-CA highest CBW=40MHz | UL3/DL1 direct-hit |
| 13 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 |  | UL5/DL1 direct-hit |
| 13 | n77 | 5 | 15 | 20 (RBstart=0) | 100 | 0.7 |  | UL5/DL1 direct-hit |
| 14 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 |  | UL5/DL1 direct-hit |
| 14 | n77 | 5 | 15 | 20 (RBstart=0) | 100 | 0.7 |  | UL5/DL1 direct-hit |
| 18 | n77 | 5 | 15 | 16 (RBstart=0) | 10 | 10.4 |  | UL4/DL1 direct-hit |
| 18 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 |  | UL4/DL1 direct-hit |
| 19 | n77 | 5 | 15 | 16 (RBstart=0) | 10 | 10.4 |  | UL4/DL1 direct-hit |
| 19 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 |  | UL4/DL1 direct-hit |
| n28 | 32 | 5 | 15 | 12 (RBstart=0) | 5 | 28.1 |  | UL2/DL1 direct-hit |
| n28 | 32 | 5 | 15 | 25 (RBstart=0) | 20 | 22.8 |  | UL2/DL1 direct-hit |
| 28 | n50 | 5 | 15 | 12 (RBstart=0) | 5 | 27.8 | + 4.8dB | UL2/DL1 direct-hit |
| 28 | n50 | 5 | 15 | 25 (RBstart=0) | 80 | 10.8 | +4.6dB | UL2/DL1 direct-hit |
| 28 | n50 | 5 | 15 | 12 (RBstart=0) | 5 | 1.9 |  | UL2/DL1 near-miss |
| 28 | n51 | 5 | 15 | 12 (RBstart=0) | 5 | 27.8 |  | UL2/DL1 direct-hit |
| 28 | n66 | 5 | 15 | 8 (RBstart=0) | 5 | 10.2 |  | UL3/DL1 near-miss |
| 28 | n66 | 5 | 15 | 25 (RBstart=0) | 40 | 2 |  | UL./DL1 direct-hit |
| 28 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 |  | UL5/DL1 direct-hit |
| 28 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 |  | UL5/DL1 direct-hit |
| 28 | n78 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 |  | UL5/DL1 direct-hit |
| 28 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 |  | UL5/DL1 direct-hit |
| 20 | n38 | 5 | 15 | 16 (RBstart=0) | 5 | 12.9 |  | UL3/DL1 direct-hit |
| 20 | n38 | 5 | 15 | 25 (RBstart=0) | 40 | 5 |  | UL3/DL1 direct-hit |
| 20 | n41 | 5 | 15 | 16 (RBstart=0) | 6 | 12.9 |  | UL3/DL1 direct-hit |
| 20 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 2.1 |  | UL3/DL1 direct-hit |
| 20 | n77 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 |  | UL4/DL1 direct-hit |
| 20 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 1.0 | -0.4 dB | UL4/DL1 direct-hit |
| 20 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 |  | UL4/DL1 direct-hit |
| 20 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.0 | -0.4 dB | UL4/DL1 direct-hit |
| 25 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 |  | UL2/DL1 direct-hit |
| 25 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 |  | UL2/DL1 direct-hit |
| 25 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 |  | UL2/DL1 near-miss |
| 25 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 |  | UL2/DL1 direct-hit |
| 25 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 |  | UL2/DL1 direct-hit |
| 25 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 |  | UL2/DL1 near-miss |
| 28 | 21 | N/A | N/A | N/A | N/A | N/A |  | UL2/DL1 direct-hit |
| n25 | 48 | 5 | 15 | 25 (RBstart=0) | 5 | 27.3 |  | UL2/DL1 direct-hit |
| n25 | 48 | 5 | 15 | 25 (RBstart=0) | 20 | 21.2 |  | UL2/DL1 direct-hit |
| n25 | 48 | 5 | 15 | 25 (RBstart=0) | 5 | 1.9 |  | UL2/DL1 near-miss |
| 26 | n41 | 5 | 15 | 16 (RBstart=0) | 10 | 10.3 |  | UL3/DL1 direct-hit |
| 26 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 2.7 |  | UL3/DL1 direct-hit |
| 26 | n77 | 5 | 15 | 16 (RBstart=0) | 10 | 10.3 |  | UL4/DL1 direct-hit |
| 26 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 2.7 |  | UL4/DL1 direct-hit |
| 26 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.3 |  | UL4/DL1 direct-hit |
| 26 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 2.7 |  | UL4/DL1 direct-hit |
| 28 | n1 | 5 | 15 | 8 (RBstart=0) | 5 | 10.2 |  | UL3/DL1 direct-hit |
| 28 | n1 | 5 | 15 | 25 (RBstart=0) | 50 | 1.0 | -0.1 dB | UL3/DL1 direct-hit |
| n28 | 1 | 5 | 15 | 8 (RBstart=0) | 5 | 10.2 |  | UL3/DL1 direct-hit |
| n28 | 1 | 5 | 15 | 25 (RBstart=0) | 20 | 5.3 |  | UL3/DL1 direct-hit |
| n28 | 4 | 5 | 15 | 8 (RBstart=0) | 5 | 10.2 |  | UL3/DL1 direct-hit |
| n28 | 4 | 5 | 15 | 25 (RBstart=0) | 20 | 5.3 |  | UL3/DL1 direct-hit |
| 28 | n75 | 5 | 15 | 12 (RBstart=0) | 5 | 28.1 |  | UL2/DL1 direct-hit |
| 28 | n75 | 10 | 15 | 50 (RBstart=0) | 50 | 18.0 |  | UL2/DL1 direct-hit |
| n28 | 11 | 5 | 15 | 12 (RBstart=0) | 5 | 24.8 |  | UL2/DL1 direct-hit |
| n28 | 11 | 5 | 15 | 25 (RBstart=0) | 20 | 21.8 |  | UL2/DL1 direct-hit |
| n28 | 42 | 5 | 15 | 16 (RBstart=0) | 5 | 14.1 |  | UL4/DL1 direct-hit |
| n28 | 42 | 5 | 15 | 25 (RBstart=0) | 20 | 7.9 |  | UL4/DL1 direct-hit |
| n28 | 66 | 5 | 15 | 8 (RBstart=0) | 5 | 10.2 |  | UL3/DL1 direct-hit |
| n28 | 66 | 5 | 15 | 25 (RBstart=0) | 20 | 5.3 |  | UL3/DL1 direct-hit |
| 71 | n2 | 5 | 15 | 25 (RBstart=0) | 5 | 4.6 |  | UL3/DL1 near-miss |
| 71 | n2 | 5 | 15 | 20 (RBstart=0) | 20 | 0.6 |  | UL3/DL1 near-miss |
| n71 | 2 | 5 | 15 | 25 (RBstart=0) | 5 | 4.6 |  | UL3/DL1 near-miss |
| n71 | 2 | 5 | 15 | 20 (RBstart=0) | 20 | 0.6 |  | UL3/DL1 near-miss |
| n71 | 7 | 5 | 15 | 25 (RBstart=0) | 5 | 14.6 |  | UL4/DL1 direct-hit |
| n71 | 7 | 5 | 15 | 25 (RBstart=0) | 20 | 9 |  | UL4/DL1 direct-hit |
| 71 | n41 | 5 | 15 | 25 (RBstart=0) | 5 | 10.8 |  | UL4/DL1 direct-hit |
| 71 | n41 | 5 | 15 | 25 (RBstart=0) | 20 | 1.4 |  | UL4/DL1 direct-hit |
| 66 | n48 | 5 | 15 | 12 (RBstart=0) | 5 | 27.3 | +0.2 dB | UL2/DL1 direct-hit |
| 66 | n48 | 40 | 15 | 200 (RBstart=0) | 1007 | 14.0 | +0.2 dB | UL2/DL1 direct-hit |
| 66 | n48 | 5 | 15 | 12 (RBstart=0) | 10 | 1.9 |  | UL2/DL1 near-miss |
| 66 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 |  | UL2/DL1 direct-hit |
| 66 | n77 | 20 | 15 | 100 (RBstart=0) | 100 | 13.8 |  | UL2/DL1 direct-hit |
| 66 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 |  | UL2/DL1 near-miss |
| 66 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 |  | UL2/DL1 direct-hit |
| 66 | n78 | 20 | 15 | 100 (RBstart=0) | 100 | 13.8 |  | UL2/DL1 direct-hit |
| 66 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 |  | UL2/DL1 near-miss |
| n66 | 48 | 5 | 15 | 12 (RBstart=0) | 5 | 27.3 | +0.2 dB | UL2/DL1 direct-hit |
| n66 | 48 | 40 | 15 | 200 (RBstart=0) | 20 | 21.2 |  | UL2/DL1 direct-hit |
| 71 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 |  | UL5/DL1 direct-hit |
| 71 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 8.9 |  | UL5/DL1 direct-hit |
| 71 | n78 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 |  | UL5/DL1 direct-hit |
| 71 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 8.9 |  | UL5/DL1 direct-hit |

# 3. Conclusion

This contribution collects our views towards simplifying the EN-DC MSD specifications for MSD due to interference and MSD due to cross-band isolation.

Observation 1: The scope of MSD test point simplification for TS 38.101-3 is restricted to the following 6 tables:

- Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1 (Table 7.3B.2.3.1-1),

- Reference sensitivity QPSK PREFSENS (EN-DC with n46) (Table 7.3B.2.3.1-3),

- Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1 (Table 7.3B.2.3.2-1),

- Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1 (Table 7.3B.2.3.2-1a),

- Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1 (Table 7.3B.2.3.4-1),

- Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 EN-DC in NR FR1 (Table 7.3B.2.3.4-1a).

Observation 2: Several essential corrections to the Release 17.7.0 NR-CA (TS 38.101-1) MSD test points due to harmonic interference and cross band isolation are required before any optimization can start on TS 38.101-3.

Proposal 1: The migration of EN-DC MSD test points due to harmonic interference and cross-band isolation interference should be implemented simultaneously across all tables as was done for Release 17. To do so, the NR-CA Release 17 MSD test points should be corrected first.

Proposal 2: For a given pair of frequency bands and for identical UL/DL CBW configurations, in case an MSD test point is missing in one TS (say 38.101-1) but has been agreed in the other TS, automatically port the agreed test point to the TS where the specification is missing.

Proposal 3: For a given pair of frequency bands and for identical UL/DL CBW configurations, in case the MSD levels differ between the TS 38.101-1 and TS 38.101-3, adopt the highest MSD level and align both TS test points.

Proposal 4: Only 1 test point per band combination is specified, that which defines the affected DL band smallest CBW.

Proposal 5: For a given pair of frequency bands, and in case the smallest CBW is common to both the LTE and the NR DL cell groups, a single test point is specified for combinations where the LTE and NR cell groups are swapped, like DC_12_n66 / DC_66_n12.  It is sufficient to capture only the MSD of the DL affected band smallest CBW. Using the example of DC_12_n66/DC_66_n12, we propose to reduce the table complexity from 4 entries to only 1 entry as shown below.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | Difference to NR-CA | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 12 n12 | n66 66 | 5 | 15 | 8 (RBstart=0) | 5 | 10 |  | UL3/DL1 direct-hit |

# References

[1]	R4-2210566, Draft CR for 38.101-1 to introduce new table format for MSD due to cross-band isolation, 3GPP TSG-RAN WG4 Meeting #103-e, Skyworks Solutions, Inc.

[2] 	R4-2210567, Draft CR for 38.101-1 to introduce new table format for MSD due to harmonic interference and for SUL, 3GPP TSG-RAN WG4 Meeting #103-e, Huawei, HiSilicon.

[3] 	R4-1912855, Draft CR for TS 38.101-3: Removing MSD requirements for EN-DC combinations due to receiver even order harmonic mixing with UL 3rd harmonic, 3GPP TSG-RAN WG4 Meeting #92-bis Chongqing China, MediaTek Inc., Qualcomm Inc.

[4] 	R4-2210565, WF on criteria on Rel-17 enhanced MSD table format, 3GPP TSG-RAN WG4 Meeting #103-e, Skyworks Solutions, Inc.

[5] 	R4-2214004, Corrections to NR-CA Cross-band Isolation MSD Test Points, 3GPP TSG-RAN WG4 Meeting # 104-e, Skyworks Solutions, Inc.

[6]	R4-211xxxx, Corrections to Cross-band isolation and Rx harmonic mixing MSD, 3GPP TSG-RAN WG4 Meeting # 105, Toulouse, France, Skyworks Solutions, Inc., ZTE Corporation.

[7] 		R4-2218038, CR for 38.101-3 to Add Missing MSD Items for PC2 EN-DC combos, 3GPP TSG-RAN WG4 Meeting # 105, Toulouse, France, AT&T, Verizon.