3GPP TSG-RAN WG4 Meeting #105	R4-2219701

Toulouse, France, 14 November – 18 November 2022

Source:	Skyworks Solutions, Inc.

Title:	Triple beat MSD for DC_3C_n1A-n75A and DC_3C-32A_n1A

Agenda item:			7.1.1.1 Band combinations with UL configurations including intra-band ULCA with IMD or triple beat issues

Document for:	Approval

# 1. Introduction

At meeting RAN4 #104-bis-e, a text proposal (TP) for DC_3C_n1A-n75A was flagged due to missing band n75 triple beat maximum sensitivity degradation (MSD) analysis due to uplink (UL) DC_3C_n1A transmission [1]. This contribution presents measurement data to evaluate the MSD due to triple-beat intermodulation (IMD) distortion generated by the respective primary and diversity Low Noise Amplifiers (LNAs).

# 2. Discussion

The proposed MSD test point for DC_3_n1-n75 reproduced in Table 1 [1] shows that a triple-beat product due to UL DC_3C_n1A is expected at 1481.5MHz. This IMD product may therefore also affect B32, accordingly this document presents a common MSD analysis to both DC_3C_n1A-n75A and DC_3C-32A_n1A.

Table 1 Proposed MSD test point for DC_3C_n1A-n75A.

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3C_n1A-n75A | 3 | 1720 | 20 | 1(RBstart=0) | 1815 | N/A | FDD | N/A |
|  |  | 1739.8 | 20 | 1(RBstart=99) | 1834.8 | N/A |  |  |
|  | n1 | 1977.5 | 5 | 25(RBstart=0) | 2167.5 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1481.5 | [FFS] | SDL | IMD3 |

The MSD analysis assumes the pentaplexer RF-FE architecture of Figure 1. Due to time constraints, only the primary/diversity LNA contributions are considered as sole contributors to IMD interference through coupling paths #1 to #7. Paths 1,2,3 are conducted paths, Paths 4,5,6,7 are PCB isolation coupling paths.

Figure 1: DC_3-32_n1/DC_3_n1-n75 Triple Beat coupling paths for band n75/B32 LNA contributions

The RF-FE assumptions are shown in Table 2, with isolations for diversity branch assumed 10dB worse than for main branch.

Table 2: RF-FE parameters for DC_3C_n26A MSD evaluation

| Attenuation and Isolation Parameter | This contribution (dB) |
| --- | --- |
| Antenna ISO | 10 |
| Diplexer ISO | 10 |
| PCB isolation | 60 |
| B1-3-7 pentaplexer/triplexer Rx rejection at band B3 Tx | pentaplexer (main): 50 triplexer (diversity): 40 |
| B1-3-7 pentaplexer/triplexer Rx rejection at band n1 Tx | pentaplexer (main): 50 triplexer (diversity): 40 |

In Figure 4, the IMD levels are based on jammer input power levels calculated using Table 2 assumptions, and LNA IMD measurements of [2]. The band n75/B32 MSD is calculated based on the 5MHz CBW REFSENS level of -100dBm.

Figure 4: LNA triple beat IMD interference levels referred to the primary and diversity antenna ports and corresponding MSD for Band n75/B32 5MHz CBW.

The MRC MSD assuming correlated interference is 21.6dB.

Proposal 1: For DC_3C-32A_n1A, adopt the band B32 5MHz CBW triple beat MSD test point of Table 3

Table 3: Band B75 5MHz MSD for DC_3C_n1A-n75A

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3C_n1A-n75A | 3 | 1720 | 20 | 1(RBstart=0) | 1815 | N/A | FDD | N/A |
|  |  | 1739.8 | 20 | 1(RBstart=99) | 1834.8 | N/A |  |  |
|  | n1 | 1977.5 | 5 | 25(RBstart=0) | 2167.5 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1481.5 | 21.6 | SDL | IMD3 |

Proposal 2: For DC_3C-32A_n1A, adopt the band B32 5MHz CBW triple beat MSD test point of Table 4

Table 4: Band B32 5MHz MSD for DC_3C-32A_n1A

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3C-32A_n1A | 3 | 1720 | 20 | 1(RBstart=0) | 1815 | N/A | FDD | N/A |
|  |  | 1739.8 | 20 | 1(RBstart=99) | 1834.8 | N/A |  |  |
|  | 32 | N/A | 5 | N/A | 1481.5 | 21.6 | SDL | IMD3 |
|  | n1 | 1977.5 | 5 | 25(RBstart=0) | 2167.5 | N/A | FDD | N/A |

# 3. Conclusion

This contribution evaluates the band n75/B32 5MHz MSD for DC_3C_n1A-n75A and DC_3C-32A-n1A respectively. Due to time constraints, the contribution of power amplifier IMD could not be evaluated, we therefore make the following proposals based solely on the LNAs contribution.

Proposal 1: For DC_3C-32A_n1A, adopt the band B32 5MHz CBW triple beat MSD test point of Table 3

Table 3: Band B75 5MHz MSD for DC_3C_n1A-n75A

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3C_n1A-n75A | 3 | 1720 | 20 | 1(RBstart=0) | 1815 | N/A | FDD | N/A |
|  |  | 1739.8 | 20 | 1(RBstart=99) | 1834.8 | N/A |  |  |
|  | n1 | 1977.5 | 5 | 25(RBstart=0) | 2167.5 | N/A | FDD | N/A |
|  | n75 | N/A | 5 | N/A | 1481.5 | 21.6 | SDL | IMD3 |

Proposal 2: For DC_3C-32A_n1A, adopt the band B32 5MHz CBW triple beat MSD test point of Table 4

Table 4: Band B32 5MHz MSD for DC_3C-32A_n1A

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3C-32A_n1A | 3 | 1720 | 20 | 1(RBstart=0) | 1815 | N/A | FDD | N/A |
|  |  | 1739.8 | 20 | 1(RBstart=99) | 1834.8 | N/A |  |  |
|  | 32 | N/A | 5 | N/A | 1481.5 | 21.6 | SDL | IMD3 |
|  | n1 | 1977.5 | 5 | 25(RBstart=0) | 2167.5 | N/A | FDD | N/A |

# Reference

[1]	R4-2217765, email discussion summary for [104-bis-e][105] NR_Baskets_Part_1, 3GPP TSG-RAN WG4 Meeting #104-bis-e, Skyworks Solutions, Inc.

[2] 	R4-2215528, Triple beat MSD for DC_3C_n28A, 3GPP TSG-RAN WG4 Meeting #104-bis-e, Skyworks Solutions, Inc.