3GPP TSG-RAN WG4 Meeting #105	R4-2219705

Toulouse, France, 14 November – 18 November 2022

Source:	Skyworks Solutions, Inc.

Title:	NR-U 100MHz A-MPR

Agenda item:			7.28.2 Common requirements (channel raster, A-MPR for 100MHz CBW)

Document for:	Approval

# 1. Introduction

This document brings raw Power Amplifier (PA) back-off measurements for NS_53, NS_54 and NS_58 for 100MHz single-carrier operation.

# 2. Discussion

## 2.1		100MHz NR-U A-MPR Measurement Results

The test conditions are set as follows:

- A 5GHz Wi-Fi PA is calibrated to meet 1dB MPR for power class (PC) 5 using the agreed 20MHz 100RB3 QPSK DFT-s-OFDM SCS 15kHz reference waveform.

- Local Oscillator (LO) leakage has been disabled to avoid erroneous NR-U SEM scaling and therefore erroneous back-off gating levels.

- IQ Image rejection: -28dB.

- Gating measurements: only MPR gating is evaluated using:

- PC 5 ACLR: -27dBc; and

- Agreed Spectrum Emission Mask (SEM) in WF [1]. The spectrum emission mask for non-transmitted channels is floored at -28dBr.

Due to lack of time we can only present preliminary raw PA back-off data. The back-off is evaluated against NS_53, NS_54 and NS_58 emission requirements for WB configurations 10000, 11100, 11110, 11111. The measurement is repeated for 4 types of QPSK modulated waveforms: Fully allocated CP-OFDM, denoted “CP_Full”, fully allocated DFT-s-OFDM, denoted “DFT_Full”, interlaced CP-OFDM “CP_Inter” and interlaced DFT-s-OFDM “DFT_Inter.” The raw PA back-off results are captured in Table 1.

Table 1: raw PA back-off results

| Waveform type | WB configuration | SEM | ACLR | NS_53/NS_54 OOB | NS_58 OOB |
| --- | --- | --- | --- | --- | --- |
| CP_Full | 11100 | 4.25 | 2.85 | 2.85 | 2.15 |
| CP_Full | 11111 | 2.9 | 2.75 | 2.75 | 1.85 |
| CP_Inter | 10000 | 2.15 | 2.65 | 0.75 | 2.35 |
| CP_Inter | 11111 | 2.55 | 2.85 | 3.35 | 2.45 |
| DFT_Full | 11100 | 2.15 | 1.05 | 1.35 | 0.75 |
| DFT_Full | 11111 | 1.95 | 1.75 | 1.45 | 0.55 |
| DFT_Inter | 10000 | 1.25 | 1.65 | 0.35 | 1.65 |
| DFT_Inter | 11100 | 1.15 | 1.65 | 2.45 | 1.75 |
| DFT_Inter | 11111 | 2.15 | 1.95 | 2.55 | 1.75 |

Observation 1: The preliminary measurement data presented in Table 1 indicate that MPR is sufficient to meet out of band (OOB) emission requirements. Table 1 does not re-evaluate in-band PSD back-off that are assumed equal to those previously presented in contribution [1].

These preliminary results need to be confirmed with a more exhaustive campaign. Based on this observation, we propose to consider the following NS_53 table as input for future studies.

Observation 2: Preliminary measurement results indicate that NR-U 100MHz single carrier MPR requirement are sufficient to NS_53 requirements. Companies are encouraged to further evaluate the validity of these preliminary findings.

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 | ≤ 1.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 | ≤ 1.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 2.0 | ≤ 3.0 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.0 | ≤ 7.0 | ≤ 5.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

# 3. Conclusion

This contribution presents preliminary measurement results for NR-U 100MHz single carrier A-MPR.

Observation 1: The preliminary measurement data presented in Table 1 indicate that MPR is sufficient to meet out of band (OOB) emission requirements. Table 1 does not re-evaluate in-band PSD back-off that are assumed equal to those previously presented in contribution [1].

These preliminary results need to be confirmed with a more exhaustive campaign. Based on this observation, we propose to consider the following NS_53 table as input for future studies.

Observation 2: Preliminary measurement results indicate that NR-U 100MHz single carrier MPR requirement are sufficient to NS_53 requirements. Companies are encouraged to further evaluate the validity of these preliminary findings.

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 | ≤ 1.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 | ≤ 1.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 2.0 | ≤ 3.0 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.0 | ≤ 7.0 | ≤ 5.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

# Reference

[1]	R4-2211606, Single carrier NR-U A-MPR for in-band emission limited and 100MHz channels, 3GPP TSG-RAN WG4 Meeting # 104-e, Skyworks Solutions Inc.