3GPP TSG-RAN4 Meeting #105 														R4-2220051

Toulouse, France, 14th Nov. – 18th Nov., 2022

Agenda item:			6.2.7

Source:	Moderator (Xiaomi)

Title:	Email discussion summary for [105][205] NR_NTN_solutions_RRM_2

Document for:	Information

Introduction

The scope of this email discussion contains NTN RRM performance requirements (6.2.5). All the submitted TDocs in this agenda were reviewed and the relevant observations and proposals are included in this email discussion.

The timeline for 1st and 2nd round email discussions can be referred in TDoc of “RAN4#105 E-meeting Arrangements and Guidelines”

In providing comments, companies are encouraged to:

- Be concise

- Provide comments on all topics/sub-topics of interest to them

- Ensure that their comments are inserted in the latest version of the document by checking the folder before uploading

- Use “Track changes” to help identify added comments/changes

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: Performance part for NTN RMM

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2218407 | Qualcomm Incorporated | We presented our views on the following four open issues. UE GNSS location change in test cases for location-based intra/inter-frequency cell reselection Proposal 1: For location-based cell reselection test cases, UE GNSS location change shall be no less than distanceThresh + 50m. SIB19 Reading Time in Cell Reselection Test Cases Proposal 2: SIB19 scheduling period shall not be larger than 160ms. Satellite Configurations Proposal 3: Satellite trajectory to be used in RRM test cases shall meet the following characteristics. - For GEO at an altitude of 35,786km, an elevation angle relative to a UE position shall not be smaller than 30 deg during entire test time - For LEO at an altitude of 600km on a circular orbit, an elevation angle relative to a UE position shall not be smaller than 45 deg during entire test time Reference Propagator Model Observation 1: A basic Kepler equation-based LEO satellite position estimation can achieve an accuracy of 6.7m error, which is 22.3% of the assumption considered for UE UL transmission timing requirements, when the satellite is on a circular orbit at 600km altitude, and a frequency of ephemeris information update is assumed once every 10.24s. And the estimation error of the satellite velocity is insignificant. Proposal 4: RAN4 to adopt a Keplerian propagator model as a reference satellite positioning model and capture it in Annex B for RAN5 and TE vendors’ reference. Reference Time Instances for UL Timing Accuracy Requirements Proposal 5: In RRM test cases, when a test equipment adjusts downlink transmission frame boundary/Doppler shift and UL reception timing, asymmetric propagation delays on DL and UL for the same slot index shall be taken into account. To model the round trip delay over service link (N_{TA,UE-specific}), the following definitions of reference slot for S3 and S4 (based in Fig. 3) are adopted and captured in Annex B for RAN5 and TE vendors’ reference. - The S3 represents a one-way propagation delay of a UE UL transmission over the service link from the UE to the satellite. The UL transmission arrives at the satellite at T3, which is the time instance when the UL transmission is supposed to arrive at the target satellite based on provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model. - The S4 represents a one-way propagation delay of a DL transmission over the service link from the satellite to the UE. The satellite relays the DL transmission from the GW to UE at T4, which is the time instance when the DL transmission corresponding to the same slot index as the UL transmission is supposed to arrive at the target satellite based on provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model. Figure 3. Timing Relation between UE DL Reception and UE UL Transmission |
| R4-2218428 | CATT | Proposal 1: No need to introduce the reference propagator model in the NTN RRM test cases. Proposal 2: Update two test cases for inter-frequency time/distance based conditional handover with fulfilling power based events and time/location based events at different time instances, and the test requirements will be updated as “The UE shall start to transmit the PRACH to Cell 2 later than [xxx] ms and less than [yyy] ms from the beginning of time period T2”. Proposal 3: Agree option 1, i.e. UE location is acquired by GNSS positioning, and the test parameter for GNSS signal power levels defined in B.4.1 is reused. Proposal 4: If using AT command approach to acquire UE location, the timing test requirements should be Te_NTN – 10.24TS. Proposal 5: The test requirement for UE initial transmit timing is Te_NTN, and Reference timing for uplink transmission in test cases is (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc, here - The DL timing shall be adjusted according to the distance change between serving satellite and UE based on a reference orbit for the serving satellite. - NTA,common is the common TA for reference timing is calculated based on the definition in TS38.211. - NTA,UE-specific is the UE specific TA for reference timing derived from the satellite location and UE location, and equal to (D1 + D2) / (c×Tc) . - D1 is distance between UE and satellite when receiving the UE transmit signal slot. - D2 is distance between UE and satellite when at time of NTA,common×Tc after receiving the UE transmit signal slot. Proposal 6: We have no strong view, and prefer not to define test case for FO gaps. |
| R4-2218660 | CMCC | Proposal 1: Set SIB19 scheduling period smaller than 1280ms. For example, 20ms, which is same with SSB period in the test case. Proposal 2: Choose test case 2-4 and 2-5 to apply the test configuration update, or test case 2-3 and 2-6 to apply the test configuration update. Proposal 3: - Set the time instant fulfilling t1-Threshold-r17 at (T2+2*Tmeasure), and set the time instant fulfilling duration-r17 at (T2+ 3*Tmeasure). - Test requirement should be 2*Tmeasure + Tinterrupt + TCHO_execution from the start of T2, others shall follow legacy Proposal 4: UE location is acquired by GNSS positioning, and the test parameter for GNSS signal power levels defined in B.4.1 is reused. Proposal 5: Either define FO test case or not define FO test case is fine for us. |
| R4-2219068 | Ericsson | Proposal 1: No need to introduce the reference propagator model. Proposal 2: Proposal 3: Since only high-level sharing rule is defined, we intend to no test case for FO gaps. |
| R4-2219243 | Huawei, HiSilicon | Proposal 1: For NTN UE timing test cases, it is suggested to use AT command approach to acquire UE location in order to simplify the test configuration. Proposal 2: For NTN UE timing testing, it is suggested to define a reference orbit for the serving satellite, and the DL timing shall be adjusted according to the distance change between serving satellite and UE. Proposal 3: For NTN UE timing test cases, the additional timing error for the translation from reference orbit to ephemeris information can be considered. |
| R4-2219481 | Nokia, Nokia Shanghai Bell | Proposal 1: For the transmit timing test cases, the configuration of the reference motion trajectory to be chosen must be the one that provides the lowest elevation angle between UE and satellite, i.e., where timing advance varies faster. Proposal 2: The reference motion trajectory configuration must cover the lowest angle where UEs might reasonably expect coverage from NTN cells, i.e., elevation angles > [30] degrees. Proposal 3: Introduce a reference propagator model in specifications to define the reference values for N"N" _"TA,adj" ^"UE" " " and N_"TA,adj" ^"common" . The propagator model at UE side is left for implementation, as long as the transmit timing accuracy requirements are observed. Proposal 4: The reference propagator model to be selected is Eckstein-Hechler and the duration of the prediction corresponds to the interval between consecutive epoch times, as the satellite assistance information is renewed at epoch time. Proposal 5: The common delay parameters provided in SIB19 for the RRM test cases can be obtained as follows: for each satellite motion trajectory configuration vector, define one GNSS location on Earth to represent the uplink time synchronization reference point. The common delay is then calculated from the distance between the satellite position defined by the satellite motion trajectory and the time synchronization reference point. |
| R4-2219482 | Nokia, Nokia Shanghai Bell | Proposal 1: Remove the sentence defining the SIB1 and SIB19 periodicity equal to 20ms in the following test cases: A.14.1.1, A.14.1.2, A.14.1.3, A.14.1.4, A.14.1.7, A.14.1.8 Observation 1: After UE acquire SIB19, it must await until the epoch time is reached before performing timing alignment (application of timing advance) before initiating a Random Access procedure toward the target cell. Observation 2: There is no consensus yet whether the UE is allowed to iniate T430 before the epoch time is reached in RAN 1. Proposal 2: For the cell reselection test-cases, consider to include in TSI-NR the time between the reception of SIB19 and the epoch time. Proposal 3: Define a configuration for the SIB19 where the epoch-time is implicitly conveyed and the si-WindowLength is set to [s80]. |
| R4-2219483 | Nokia, Nokia Shanghai Bell | Observation 1: For other RRM features, RAN4 has compromised developing test cases where the UE acquires its position from the test equipment. Observation 2: For the transmit timing test, the usage of the AT command to convey UE position does not fully test the new functionality introduced in NTN. Proposal 1: For the acquisition of UE location in UE transmit timing test cases, we prefer Option 1: UE location is acquired by GNSS positioning, and the test parameter for GNSS signal power levels defined in B.4.1 is reused. |
| R4-2219540 | Huawei, HiSilicon | Proposal 1: Adopt the following steps to define satellite RMC for NTN RRM tests. - Step 1: Generate a detailed reference motion trajectory which describes the location of the satellite during an orbital period - Step 2: Generate ephemeris information based on reference motion trajectory Proposal 2: RAN4 not to define reference propagator for NTN RRM test. Proposal 3: As baseline, no location margin needs to be added in the test cases where “Update UE Location Information” procedure is used to set the GNSS location. Proposal 4: SIB19 is scheduled with [80]ms period in RRM tests. Proposal 5: RAN4 not to define test cases for fully overlapping MG. |

Open issues summary and Companies views’ collection for 1st round

Test case coverage, design and configuration

Issue 1-1: Satellite configuration.

- Option 1: (QC)

  - Satellite trajectory to be used in RRM test cases shall meet the following characteristics.

    - For GEO at an altitude of 35,786km, an elevation angle relative to a UE position shall not be smaller than 30 deg during entire test time

    - For LEO at an altitude of 600km on a circular orbit, an elevation angle relative to a UE position shall not be smaller than 45 deg during entire test time

- Option 2: (Nokia)

  - For the transmit timing test cases, the configuration of the reference motion trajectory to be chosen must be the one that provides the lowest elevation angle between UE and satellite, i.e., where timing advance varies faster.

  - The reference motion trajectory configuration must cover the lowest angle where UEs might reasonably expect coverage from NTN cells, i.e., elevation angles > [30] degrees.

- Option 3: (Huawei)

  - Adopt the following steps to define satellite RMC for NTN RRM tests.

    - Step 1: Generate a detailed reference motion trajectory which describes the location of the satellite during an orbital period

    - Step 2: Generate ephemeris information based on reference motion trajectory

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| XXX |  |

Issue 1-2: Reference propagator model.

- Option 1: (CATT, Ericsson, Huawei)

  - No need to introduce the reference propagator model in the NTN RRM test cases.

- Option 2: (QC)

  - RAN4 to adopt a Keplerian propagator model as a reference satellite positioning model and capture it in Annex B for RAN5 and TE vendors’ reference.

- Option 3: (Nokia)

  - Introduce a reference propagator model in specifications to define the reference values for N"N" _"TA,adj" ^"UE"  " "  and N_"TA,adj" ^"common"  . The propagator model at UE side is left for implementation, as long as the transmit timing accuracy requirements are observed.

  - The reference propagator model to be selected is Eckstein-Hechler and the duration of the prediction corresponds to the interval between consecutive epoch times, as the satellite assistance information is renewed at epoch time.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| XXX |  |

Issue 1-3: The configuration of the common delay parameters.

- Option 1: (Nokia)

  - The common delay parameters provided in SIB19 for the RRM test cases can be obtained as follows: for each satellite motion trajectory configuration vector, define one GNSS location on Earth to represent the uplink time synchronization reference point. The common delay is then calculated from the distance between the satellite position defined by the satellite motion trajectory and the time synchronization reference point.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| XXX |  |

Test case design for Cell reselection

Issue 2-1: UE GNSS location change in location-based intra/inter-frequency cell reselection test.

- Option 1: (QC)

  - For location-based cell reselection test cases, UE GNSS location change shall be no less than distanceThresh + 50m.

- Option 2: (Huawei)

  - As baseline, no location margin needs to be added in the test cases where “Update UE Location Information” procedure is used to set the GNSS location.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| XXX |  |

Issue 2-2: TSI-NR in test requirement.

- Option 1: (Nokia)

  - For the cell reselection test-cases, consider to include in TSI-NR the time between the reception of SIB19 and the epoch time..

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| XXX |  |

Issue 2-3: SIB19 scheduling time in test requirement.

- Option 1: (QC)

  - SIB19 scheduling period shall not be larger than 160ms.

- Option 2: (CMCC)

  - Set SIB19 scheduling period smaller than 1280ms. For example, 20ms, which is same with SSB period in the test case.

- Option 3: (Huawei)

  - SIB19 is scheduled with [80]ms period in RRM tests.

- Option 4: (Nokia)

  - Remove the sentence defining the SIB1 and SIB19 periodicity equal to 20ms in the following test cases: A.14.1.1, A.14.1.2, A.14.1.3, A.14.1.4, A.14.1.7, A.14.1.8.

  - Define the SIB19 configuration where the epoch-time is implicitly conveyed and the si-WindowLength is set to [s80].

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| XXX |  |

Test case design for handover

Issue 3-1: Test configuration for inter-frequency time/distance based conditional handover test with fulfilling power based events and time/location based events at different time instances.

- Option 1: (CATT)

  - Update two test cases for inter-frequency time/distance based conditional handover with fulfilling power based events and time/location based events at different time instances, and the test requirements will be updated as “The UE shall start to transmit the PRACH to Cell 2 later than [xxx] ms and less than [yyy]  ms from the beginning of time period T2”.

- Option 2: (CMCC)

  - Choose test case 2-4 and 2-5 to apply the test configuration update, or test case 2-3 and 2-6 to apply the test configuration update.

  - Set the time instant fulfilling t1-Threshold-r17 at (T2+2*Tmeasure), and set the time instant fulfilling duration-r17 at (T2+ 3*Tmeasure).

  - Test requirement should be 2*Tmeasure + Tinterrupt + TCHO_execution from the start of T2, others shall follow legacy

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| XXX |  |

Test case design for UE timing requirements

Issue 4-1: Acquisition of UE location in UE timing test cases.

- Option 1: (CATT, CMCC, Nokia)

  - UE location is acquired by GNSS positioning, and the test parameter for GNSS signal power levels defined in B.4.1 is reused.

- Option 2: (Huawei)

  - For NTN UE timing test cases, it is suggested to use AT command approach to acquire UE location in order to simplify the test configuration.

- Option 2a: (CATT)

  - If using AT command approach to acquire UE location, the timing test requirements should be Te_NTN – 10.24Ts.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| XXX |  |

Issue 4-2: Reference Time Instances for UL Timing Accuracy Requirements.

- Option 1: (QC)

  - In RRM test cases, when a test equipment adjusts downlink transmission frame boundary/Doppler shift and UL reception timing, asymmetric propagation delays on DL and UL for the same slot index shall be taken into account. To model the round trip delay over service link (N_{TA,UE-specific}), the following definitions of reference slot for S3 and S4 (based in Fig. 3) are adopted and captured in Annex B for RAN5 and TE vendors’ reference.

    - The S3 represents a one-way propagation delay of a UE UL transmission over the service link from the UE to the satellite. The UL transmission arrives at the satellite at T3, which is the time instance when the UL transmission is supposed to arrive at the target satellite based on provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model.

    - The S4 represents a one-way propagation delay of a DL transmission over the service link from the satellite to the UE. The satellite relays the DL transmission from the GW to UE at T4, which is the time instance when the DL transmission corresponding to the same slot index as the UL transmission is supposed to arrive at the target satellite based on provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model.

Figure 3. Timing Relation between UE DL Reception and UE UL Transmission

- Option 2: (CATT)

  - The test requirement for UE initial transmit timing is Te_NTN, and Reference timing for uplink transmission in test cases is (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc, here

    - The DL timing shall be adjusted according to the distance change between serving satellite and UE based on a reference orbit for the serving satellite.

    - NTA,common is the common TA for reference timing is calculated based on the definition in TS38.211.

    - NTA,UE-specific is the UE specific TA for reference timing derived from the satellite location and UE location, and equal to (D1 + D2) / (c×Tc) .

    - D1 is distance between UE and satellite when receiving the UE transmit signal slot.

    - D2 is distance between UE and satellite when at time of NTA,common×Tc after receiving the UE transmit signal slot.

- Option 3: (Huawei)

  - For NTN UE timing testing, it is suggested to define a reference orbit for the serving satellite, and the DL timing shall be adjusted according to the distance change between serving satellite and UE.

  - For NTN UE timing test cases, the additional timing error for the translation from reference orbit to ephemeris information can be considered.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| XXX |  |

Test case design for measurement requirements

Issue 5-1: Test case for FO MGs.

- Option 1: (CATT, Ericsson, Huawei)

  - RAN4 not to define test cases for fully overlapping MG.

    - We have no strong view, and prefer not to define test case for FO gaps. (CATT)

    - Since only high-level sharing rule is defined, we intend to no test case for FO gaps. (Ericsson)

- Option 2: (CMCC)

  - Either define FO test case or not define FO test case is fine for us.

- Recommended WF

  - Agree on option 1.

| Company | Comments |
| --- | --- |
| XXX |  |

CRs comments collection for 1st round

| CRs | Company | Comments collection |
| --- | --- | --- |
| R4-2218227 | MediaTek inc. | Company A |
| R4-2218228 | MediaTek inc. | Company A |
| R4-2218429 | CATT | Company A |
| R4-2218661 | CMCC | Company A |
| R4-2218662 | CMCC | Company A |
| R4-2218912 | LG Electronics UK | Company A |
| R4-2219069 | Ericsson | Company A |
| R4-2219244 | Huawei, HiSilicon | Company A |
| R4-2219541 | Huawei, HiSilicon | Company A |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Discussion on 2nd round (if applicable)

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents