3GPP TSG-RAN WG4 Meeting #105					      	     R4-2220244

Toulouse, France, November 14th – November 18th, 2022

Agenda item:			8.18.4

Source:	Samsung

Title:	WF for the feasibility from BS aspect

Document for:	Approval

# Introduction

This document provides way forwards SBFD feasibility study and RF impact from BS aspects, based on the discussion on the thread [105][310] FS_NR_duplex_evo_Part1.

# Way Forward on Self-Interference Model

## Residual Self-Interference Cancellation (RSIC) Analysis Framework

Agreement (based on Wed. evening Ad-Hoc):

- Companies are encouraged to provide values based on the following RSIC analysis framework table.

- Companies are encouraged to provide the information on how the intermediate results are derived.

| FR1 (or FR2-1) |  |  |  | Company-A |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS | Medium Range BS | Local Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | xxx dBm |  |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | xxx dBc |  |  |
|  |  | Frequency isolation techniques used |  | e.g., DPD, sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in TX |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | xxx dBc |  |  |
|  |  | Spatial isolation techniques used |  | e.g., spatial separation between TX/RX panel; cross polarization; circulator; shielding case; metal fences, etc. Note: List all relevant techniques used in the evaluation |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | xxx dBc |  |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  |  |  |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | xxx dBc |  |  |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | xxx dBc |  |  |
|  |  | RF IC techniques and other tech. (before LNA) |  | e.g., RF IC, sub-band filtering etc. Note: List all relevant techniques used in RX (before LNA) |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | xxx dBc |  |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  |  |  |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc |  |  |
|  |  | Frequency isolation techniques |  | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) |  |  |  |
|  |  |  | Rx IM3 contribution (dBm) |  |  |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  |  |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | xxx dBm |  |  |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | xxx dBc |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | xxx dBc |  |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | xxx dBc |  |  |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | xxx dBc |  |  |
| Noise floor ⑩dBm |  |  |  | xxx dBm/CBW |  |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | xxx dBm |  |  |
| Required RSIC budget (①-⑪dBc) |  |  |  | xxx dBc |  |  |
| SBFD configuration |  |  |  |  |  |  |
| Guardband assumption (if exist) |  |  |  |  |  |  |
| bandwidth over which suppression is achieved |  |  |  |  |  |  |
| Others |  |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |

## Assumption on site deployment aspects

Agreement:

- FFS the effect of clutter on achievable RSIC performance.

## Impact of multi-carrier support at BS

Agreement:

- Study single-carrier firstly, and secondly study the impact of multi-carrier support at BS during this SI:

  - Case-1: SBFD carrier and other TDD carriers operating in the same BS (prioritized case)

  - Case-2: SBFD carrier and other SBFD carriers operating in the same BS

# Way Forward on RSI dependency on blocking, AGC and ADC

## Assumption for input power metric to LNA

Agreement:

- FFS gNB receiver saturation, non-linearity, and AGC model is based on peak input power.

## Analysis on LNA non-linearity and blocking level

Agreement:

- RAN4 further study on LNA saturation/non-linearity:

  - FFS the value as the maximum blocking level to ensure the receiver of UL sub-band is not blocked and maintain an acceptable reference sensitivity, for FR1 WA BS.

    - Option 1: -43dBm (baseline)

    - Option 2: Other improvements are not precluded

  - The IM3 product level is encouraged to be provided in the RSIC analysis framework

- RAN4 further study on ADC dynamic range:

  - Take into account the maximum blocking level (baseline) to study the required ADC dynamic range.

- RAN4 further study on AGC impact:

  - Option 1: AGC is not need to be analysed for BS Rx path because ADC dynamic is not a problem.

  - Option 2: FFS the AGC model’s impact

    - AGC model examples:

      - Example-1: the effect of non-linearities at the gNB receiver suffered can be modelled as a linear increase (with slope SL1 and SL2) of the base station noise figure as a function of the RF peak input power at each Rx chain once such peak input power exceeds a first and a second threshold a and b.

Figure. Behaviour of noise figure as a function of Peak input power.

Table: Example parametrization of proposed model

| Snf | Small signal noise figure | 5 | dB |
| --- | --- | --- | --- |
| a | Peak input power threshold 1 | [-35] | dBm |
| b | Peak input power threshold 2 | [-16] | dBm |
| SL1 | Noise figure slope 1 | [0.35] |  |
| SL2 | Noise figure slope 2 | [1.9] |  |

NF = Snf for Peak input power < a NF = Snf – a * SL1 + SL1* Peak input power for: a < Peak input power < b NF = Snf – a*SL2 + b*(SL1 – SL2) + SL2*( Peak input power) for Peak input power > b

      - Example-2: No AGC if the signal level is below the maximum blocking level.

# Way Forward on Co-channel Inter-subband gNB-gNB CLI Model

## Co-channel co-site inter-sector gNB-gNB CLI modelling – Coupling Loss

Background: Agreement from RAN4#104-Bis-e:

On the feasibility and how to model co-site inter-sector gNB-gNB CLI modelling: similar modelling as for self-interference (RSI) can be applied but may with different parameters especially on antenna isolation - FFS on possibility to apply digital IC for this case - For co-channel co-site inter-sector gNB-gNB CLI modelling, it is encouraged to provide the numerical value for: - The achievable coupling loss in the case of co-site inter-sector gNB-gNB - Compared to self-interference, FFS the antenna isolation (with the achievable coupling loss). - Practical issues to achieve antenna isolation can be considered: e.g. increasing sector separation, mounting isolating materials on the site and the physical characteristics of such materials (size, weight etc.) - Clarification on the value discussed here: - the co-channel co-site inter-sector gNB-gNB CLI discussed here is for the sum contributions from all co-site sectors.

Agreement:

- FFS the analysis framework co-channel co-site inter-sector gNB-gNB CLI

  - FFS the RSIC analysis framework can be reused or not.

- For co-channel co-site inter-sector gNB-gNB CLI modelling, it is encouraged to provide the numerical value for:

  - The achievable coupling loss in the case of co-site inter-sector gNB-gNB

  - Compared to self-interference, FFS the antenna isolation (with the achievable coupling loss).

  - Information about the following aspects can be provided:

    - Operating band

    - BS class

    - Inter-sector distance

    - Details about isolation structure

    - Other site considerations

## Co-channel inter-site gNB-gNB CLI modelling

Agreement:

- For co-channel inter-site gNB-gNB CLI modeling, gNB ACS shall be used as baseline for system level simulation and feasibility study.

  - Further study on the possibility of improved receiver impairment performance compared to gNB ACS shall not be precluded in future RAN4 works.

# Way Forward on RF Requirement Impact

## Potentially impacted RF requirement for SBFD capable gNB

Agreement:

- In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking and adjacent subband selectivity:

  - Option 1: No such requirement needed. RAN4 consider the SBFD performance requirement for receiver sensitivity with the simultaneous TX in the SBFD time slot, in which the in-channel adjacent subblock leakage ratio and in-channel adjacent subblock blocking requirements can be guaranteed implicitly.

  - Option 2: New requirements are needed for In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking + Adjacent subband selectivity.

- OTA sensitivity:

  - New requirements are needed

- ACLR, ACS, in-band blocking, intermodulation:

  - FFS.

- Other requirements not precluded

## Existing requirement without impact for SBFD capable gNB

Agreement:

- The existing RF requirements with respect to wanted signal as below are still applicable for gNB capable of SBFD:

| Conducted RF requirement | Radiated RF requirement |
| --- | --- |
| BS output power Output power dynamics Transmit ON/OFF power Transmitted signal quality Occupied bandwidth Dynamic range | Radiated transmit power OTA base station output power OTA output power dynamics OTA transmitted signal quality OTA occupied bandwidth OTA dynamic range |

- The receiver out-of-band blocking and receiver spurious emission requirement in TS38.104 are still applicable for gNB capable of SBFD.

  - FFS the wanted signal could be REFSENS+7dB instead of REFSENS+6dB.