3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2220286

# Electronic Meeting, November 14 – 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draft | rev | 0 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General test applicability of FR2-2 UE demodulation requirements introduction and PBCH Demodulation requirements introduction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz |  |  |  |  |  | Date: |  |  | 2022-11-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel.17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | draftCR for corrections to R4-2216011 Big CR for introduction of FR2-2 UE demodulation and CSI requirements in 38.101-4 – General and PBCH Requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update TBD values for PBCH requirements; Update TBD tests references for PDSCH regarding the applicability of FR2-2 requirements; Update applicability requirements for PDSCH for operating bands and optional capabilities according to agreements in RAN4#105 (see WF R4-2220187); |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2-2 UE Demodulation requirements applicability rules are not introduced |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3, 7.1.1.6, 7.1.1.8; 7.4.2.2; 9.1.1.1, 9.1.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 7	Demodulation performance requirements (Radiated requirements)

## 7.1	General

### 7.1.1	Applicability of requirements

~~~~~~~ Start of Change #1 (new change, not included in the Big CR)~~~~~~~

#### 7.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 7.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 7.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR2-1 TDD | PDSCH | Clause 7.2.2.2.1 (Test 3-1) |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) | NR CA | SDR | Clause 7.5A.1 | 1) Up to 16 DL carriers 2) Same numerology across carrier for data/control channel at a given time |
| PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots) | FR2-1 TDD | PDSCH | Clause 7.2.2.2.2 |  |
| DRX Adaptation (drx-Adaptation-r16) | FR2-1 TDD | PDCCH | Clause 7.3.2.2.3 | If the Test 3-1 in Clause 7.3.2.2.3 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 7.3.2.2.1. |
| 256QAM for PDSCH (pdsch-256QAM-FR2) | FR2-1 TDD | PDSCH | Clause 7.2.2.2.1 (Test 1-4) |  |
| 256QAM for PDSCH (pdsch-256QAM-FR2) | FR2-1TDD | SDR | Clause 7.5A.1 | For UE capable of pdsch-256QAM-FR2 for certain band(s), mcs-Table is configured to ‘64QAM’ for SDR test. |
| Support of FR2 HST operation [(FR2 UE power class PC6 signalling is used to indicate support of feature group)] | FR2-1 TDD | PDSCH | [Clause 7.2.2.2.4] |  |
| Support of 480kHz SCS for FR2-2 (dl-FR2-2-SCS-480kHz-r17) | FR2-2 TDD | PDSCH | Clause 7.2.2.2.1 (Table 7.2.2.2.1-6: Test 4-6, 4-7) |  |
|  |  | PDCCH | Clause 7.3.2.2 (Table 7.3.2.2.1-2: Test 1a-4) (Table 7.3.2.2.2-2, Test 3-3) |  |
|  |  | PBCH | Clause 7.4.2.2 (Table 7.4.2.2-2: Test 4) |  |

~~~~~~~ End of Change #1 ~~~~~~~

~~~~~~~ Start of Change #2 ~~~~~~~

#### 7.1.1.6	Applicability of requirements for operating bands in FR2-1

The applicability rules for FR2 operating bands are specified in Table 7.1.1.6-1.

Table 7.1.1.6-1: Requirements applicability for operating bands in FR2-1

| Test type |  | Test list |  | Applicability notes |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| FR2-1 TDD | PDSCH | Clause 7.2.2.2.1 (Test 1-4) |  | The requirements are applicable for bands with FDL_high higher than 40000 MHz and lower than 48200 MHz with additional margin as 1.5 dB. |  |  |
|  | PDSCH | Clause 7.2.2.2.1 (Test 2-6) Clause 7.2.2.2.1 (Test 3-1) |  | The requirements are applicable for bands with FDL_high higher than 40000 MHz and lower than 48200 MHz with additional margin as 0.5 dB. |  |  |
| FR2-2 TDD | PDSCH |  | If UE supports Clause 7.2.2.2.1 (All Tests in Table 7.2.2.2.1-6, except for Test 4-4,4-5) |  |  | The requirements are applicable for bands with FDL_high higher than 52600 MHz and lower than 71000 MHz |
|  |  |  | Clause 7.2.2.2.1 (Test 4-4 and 4-5) |  | Test 4-5 is applicable only if the Test Equiment maximum achievable SNR is not sufficient to execute Test 4-4 |  |
|  | PDCCH |  | Clause 7.3.2.2.2 (All Tests in Table 7.3.2.2.1-2 and Table 7.3.2.2.2-2) |  |  |  |
|  | PBCH |  | Clause 7.4.2.2 (Tests 3, 4 in Table 7.4.2.2-2) |  |  |  |

~~~~~~~ End of Change #2 ~~~~~~~

~~~~~~~ Start of Change #3 (new change, not included in the Big CR)~~~~~~~

#### 7.1.1.8	Applicability of requirements for operating bands in FR2-2

The requirements in Table 7.1.1.8-1 are applicable for bands with FDL_high higher than 52600 MHz and lower than 71000 MHz;

Other performance requirements mandatory for UE supporting NR operation defined in Section 7 but not included in Table 7.1.1.8-1 should not be considered applicable to FR2-2 bands;

Table 7.1.1.8-1: Requirements applicability for operating bands in FR2-2

| Test type |  | Test list |  | Applicability notes |  |
| --- | --- | --- | --- | --- | --- |
| FR2-2 TDD | PDSCH | Clause 7.2.2.2.1 (All Tests in Table 7.2.2.2.1-6, except for Test 4-4, 4-5) |  |  | The requirements apply if the device supports Single Carrier operations in FR2-2, or if it supports both Single Carrier and CA_AX (FR1+FR2-2) operations; |
|  |  | Clause 7.2.2.2.1 (Table 7.2.2.2.1-6: Test 4-4, 4-5) | Test 4-5 is applicable only if the TE max testable SNR is not sufficient for Test 4-4 |  |  |
|  | PDCCH | Clause 7.3.2.2 (All Tests in Table 7.3.2.2.1-2) (All tests in Table 7.3.2.2.2-2) |  |  |  |
|  | PBCH | Clause 7.4.2.2 (Table 7.4.2.2-2: Tests 3, 4) |  |  |  |

~~~~~~~ End of Change #3 ~~~~~~~

## 7.4	PBCH demodulation requirements

### 7.4.2	2RX requirements

#### 7.4.2.2	TDD

~~~~~~~ Start of Change #4 ~~~~~~~

Table 7.4.2.2-1: Test parameters for PBCH

| Parameter | Unit | Single antenna port |
| --- | --- | --- |
| Physical Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index Note1 |  | 0 |
| SS/PBCH block periodicity | ms | 20 |
| TDD UL-DL pattern |  | FR2.120-1 for Tests 1,2,3 in Table 7.4.2.2-2 and Tests 1, 2 in Table 7.4.2.2.3 FR2.480-1 for Test 4 in Table 7.4.2.2-2 |
| Note 1: as specified in clause 4.1 of TS 38.213 [11] Note 2: as specified in clause 11.1 of TS 38.213 [11] |  |  |

For the parameters specified in Table 7.4.2.2-1 the average probability of a miss-detected PBCH (Pm-bch) shall be below the specified values in Table 7.4.2.2-2 in case SS/PBCH block index is not known and below the specified values in Table.7.4.2.2-3 in case SS/PBCH block index is known. The downlink physical setup is in accordance with Annex C.5.1.

Table 7.4.2.2-2: Minimum performance PBCH in case SS/PBCH block index is not known

| Test number | Bandwidth (MHz) / Subcarrier spacing (kHz) | Reference channel | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNRBB (dB) |
| 1 | 100 / 120 | R.PBCH.5 | TDLA30-300 | 1 x 2 Low | 1 | -6.3 |
| 2 | 100 / 240 | R.PBCH.6 | TDLA30-75 | 1 x 2 Low | 1 | -6.1 |
| 3 | 100 / 120 | R.PBCH.5 | TDLA30-650 | 1 x 2 Low | 1 | [-4.1] |
| 4 | 400 / 480 | R.PBCH.7 | TDLA10-200 | 1 x 2 Low | 1 | [-3.9] |

~~~~~~~ End of Change #4 ~~~~~~~

~~~~~~~ Start of Change #5 (new change, not included in the Big CR)~~~~~~~

#### 9.1.1.1 	Applicability of requirements for optional UE features

Table 9.1.1.1-1: Void

The applicability rule defined in Clause 5.1.1.3 shall be applied for performance requirements in Clauses 9.2B.1.1 and 9.4B.1.1.

The applicability rule defined in Clause 7.1.1.3 shall be applied for performance requirements in Clauses 9.2B.1.2, 9.4A.1, 9.4B.1.2 and 9.4B.1.3.

The performance requirements in Table 9.1.1.1-2 shall apply for UEs which support optional UE features only.

Table 9.1.1.1-2 Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Support of 480kHz SCS for FR2-2 (dl-FR2-2-SCS-480kHz-r17) | FR2-2 TDD | PDSCH | Clause 9.2A.1 (Table 9.2A.1-3: Test 4-4, 4-7) |  |
|  |  | PDCCH | Clause [9.3A.1] (Table [TDD]: Test TBD) |  |

#### 9.1.1.3	Applicability of requirements for operating bands in FR2-2

The requirements in Table 9.1.1.3-1 are applicable for bands with FDL_high higher than 52600 MHz and lower than 71000 MHz;

Other performance requirements mandatory for UE supporting NR operation defined in Section 9 but not included in Table 9.1.1.3-1 should not be considered applicable to FR2-2 bands;

Table 9.1.1.3-1: Requirements applicability for operating bands in FR2-2

| Test type |  | Test list |  | Applicability notes |  |
| --- | --- | --- | --- | --- | --- |
| FR2-2 TDD | PDSCH | Clause 9.2A.1 (Table 9.2A.1-3: Test 1-2) | The requirements apply if the device supports Single Carrier operations in FR2-2, or if it supports both Single Carrier and CA_AX (FR1+FR2-2) operations; |  |
|  |  | Clause 9.2A.1 (All Tests in Table 9.2A.1-3, except for Test 1-4, 1-5) |  | The requirements apply if the device supports CA_AX (FR1+FR2-2) operations in FR2-2 and not Single Carrier operations in FR2-2 |
|  |  | Clause 9.2A.1 (Test 1-4, 1-5) | Test 1-5 applies only if the TE max testable SNR is not sufficient for Test 1-4 |  |
|  | PDCCH | Clause [9.3A.1] (Test in Table [TDD]) |  |  |

~~~~~~~ End of Change #5 ~~~~~~~