3GPP TSG-RAN WG4 Meeting#105-e	R4-2220587

# Toulouse, France, 14 – 18 November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1238 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the CA_NS indication the values for n77 in the US and Canada |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-11-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clarify that the CA_NS and CA_NC_NS indicate additional unwanted emissions requirements with one and the same NS value indicated (SI) or configured (for Scells) for all uplink serving cells of the CA configuration. CA_NS values are also relevant for NS_55 and NS_57 for configurations that are fully contained within the ranges and 3450-3550 MHz and 3650-3980 MHz applicable for NS_55 and NS_57, respectively. CA_NS_47 and CA_NC_NS_47 for CA_n41 are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.2A.3.1.1: clarify that the labels CA_NS indicated additional emission requirements and the associated A-MPR allowance. The (general) requirements for CA_NS_01 are also added to the table. The information on the default indication CA_NS_01 is removed, this is specified in 38.331. Clause 6.2A.3.1.2: the corresponding changes for non-contiguous configurations. CA_NS_47 and CA_NC_NS_47 for CA_n41 are listed for CA_n41.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definitions of the CA_NS and CA_NS values may be misinterpreted. CA_NS values relevant for NS_55 and NS_57 for configuration that are fully contained within the respective ranges 3450-3550 MHz and 3650-3980 MHz are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3.1.1, 6.2A.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: the table rows for CA_NS_55/57 and CA_NC_NS_55/77 corrected, NS_47 added |  |  |  |  |  |  |  |  |

< start of changes >

### 6.2A.3	UE additional maximum output power reduction for CA

#### 6.2A.3.1.1	UE additional maximum output power reduction for Intra-band contiguous CA

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7]. Relation between NR CA band and NR frequency band is specified in Table 5.2A.1-1.

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2A.1.5-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2A.2.4. In absense of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2A.3.1.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable CA band(s) for each CA_NS value. The CA_NS_xy value indicates the additional unwanted emissions requirements that apply for intra-band contiguous CA bands with NS_xy indicated or configured in multiple uplink serving cells, except CA_NS_01 that indicates the general emission requirements for intra-band contiguous CA bands. The mapping of NR CA band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2A.3.1.1-2. For any NR CA band not listed in Table 6.2A.3.1.1-2 the network signalling label CA_NS_01 applies.

Table 6.2A.3.1.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR CA Band | Aggregated channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_NS_01 | 6.5A.2.2.1 6.5A.3.2.1 | Table 5.2A.1-1 | All applicaple NR CA bands | All applicaple NR CA configurations | N/A |
| CA_NS_04, CA_NS_47 | 6.5A.2.3.1.1 6.5A.3.3.1.1 | CA_n41 | Table 5.5A.1-1 | 6.2A.3.1.1.1 | 6.2A.3.1.1.1 |
| CA_NS_27 | 6.5A.2.3.1.2 6.5A.3.3.1.2 | CA_n48 | Table 5.5A.1-1 | 6.2A.3.1.1.2 | 6.2A.3.1.1.2 |
| CA_NS_46 | 6.5A.3.3.1.3 | CA_n7 | Table 5.5A.1-1 | 6.2A.3.1.1.3 | 6.2A.3.1.1.3 |
| CA_NS_55, CA_NS_57 | See CA_NS_01 | CA_n77 | Table 5.5A.1-1 | 6.2A.2.1 | See CA_NS_01 |

For UEs configured with intra-band contiguous CA in n77 and if NS_01 is indicated for an uplink component carrier in the range 3450-3650 MHz and NS_01 or NS_57 for another uplink component carrier in the range 3650-3980 MHz, the allowed additional spurious emission and maximum output power reduction requirements are according to CA_NS_01.

Table 6.2A.3.1.1-2: Mapping of network signaling label

| NR CA band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| CA_n41 | CA_NS_01 | CA_NS_04 | CA_NS_47 |  |  |  |  |  |
| CA_n48 | CA_NS_01 | CA_NS_27 |  |  |  |  |  |  |
| CA_n7 | CA_NS_01 | CA_NS_46 |  |  |  |  |  |  |
| CA_n77 | CA_NS_01 | CA_NS_55 | CA_NS_57 |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

##### 6.2A.3.1.1.1	A-MPR for CA_NS_04

< text omitted >

##### 6.2A.3.1.2	UE additional maximum output power reduction for Intra-band non-contiguous CA

6.2A.3.1.2.0	General

Table 6.2A.3.1.2-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable CA band(s) for each CA_NC_NS value. The CA_NC_NS_xy value indicates the additional unwanted emissions requirements that apply for intra-band non-contiguous CA bands with NS_xy indicated or configured in multiple uplink serving cells, except CA_NC_NS_01 that indicates the general emission requirements for intra-band non-contiguous CA bands. The mapping of NR CA band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2A.3.1.2-2. For any NR CA band not listed in Table 6.2A.3.1.2-2 the network signalling label CA_NC_NS_01 applies.

Table 6.2A.3.1.2-1: Additional Maximum Power Reduction (A-MPR) for intra-band non-contiguous CA

| CA Network Signalling value | Requirements (clause) | Uplink CA Configuration | A-MPR for sub-blocks in order of increasing uplink carrier frequency |
| --- | --- | --- | --- |
|  |  |  | A-MPR [dB] (clause) |
| CA_NC_NS_01 | 6.5A.2.2.2 6.5A.3.2.2 | All applicaple NR CA configurations | N/A |
| CA_NC_NS_04, CA_NC_NS_47 | 6.5A.2.3.2.1 6.5A.3.3.2.1 | CA_n41(2A) | 6.2A.3.1.2.1 |
| CA_NC_NS_55 CA_NC_NS_57 | See CA_NC_NS_01 | CA_n77(2A) | See CA_NC_NS_01 |

For UEs configured with intra-band non-contiguous CA in n77 and if NS_01 is indicated for an uplink component carrier in the range 3700-3980 MHz and NS_01 or NS_55 for another uplink component carrier in the range 3450-3550 MHz, the allowed additional spurious emission and maximum output power reduction requirements are according to CA_NC_NS_01.

Table 6.2A.3.1.2-2: Mapping of network signaling label

| NR CA band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| CA_n41 | CA_NC_NS_01 | CA_NC_NS_04 | CA_NC_NS_47 |  |  |  |  |  |
| CA_n77 | CA_NC_NS_01 | CA_NC_NS_55 | CA_NC_NS_57 |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

6.2A.3.1.2.1	AMPR for CA_NC_NS_04 (CA_n41(2A))

< end of changes >