3GPP TSG-RAN WG4 Meeting # 105 -e	R4-2220787

Electronic Meeting, November 14th – 18th, 2022

Title:	TP to TR 38.717-02-01: Addition of CA_n77-n85

Source:	Nokia, T-Mobile

Agenda item:	7.10.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.717-02-01 to add CA_n77A-n85A with 2UL.

************************************* Start of TP*****************************************

## 5.x		CA_n77-n85

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1 Operating bands for CA

Table 5.X.1.1-1: CA band combination of band n77+n85

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| n85 | 698 MHz | – | 716 MHz | 728 MHz | – | 746 MHz | FDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n77+n85

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n77A-n85A | CA_n77A-n85A | n77 | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n77(2A)-n85A | CA_n77A-n85A | n77 | CA_n77(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |

#### 5.x.1.3	Co-existence studies

Table 5.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n77-n85.

Table 5.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |
| n85 | 698 | 716 | 728 | 746 | 1396 | 1432 | 2094 | 2148 | 2792 | 2864 | 3490 | 3580 |

Based on above table, the 5th UL harmonic of n85 causes interference in RX of n77.

Table 5.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |
| n85 | 698 | 716 | 728 | 746 | 1456 | 1492 | 2184 | 2238 | 2912 | 2984 | 3640 | 3730 |

Based on above table, the fundamental of n77 causes harmonic mixing at the 5th DL harmonic of n85 for CA_n77-n85.

#### 5.x.1.4	∆TIB and ∆RIB values

For CA_n77-n85, the TIB,c and RIB values are based on CA_n71-n77.

Table 5.x.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n77-n85 | 0.7 | 0.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n77-n85 | 0.5 | 0.2 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.x.1.5	REFSENs requirements

As can be seen in the co-existence studies in 5.x.1.3 there are both UL harmonic and harmonic mixing issues.

Based on the co-existence studies there is 5th uplink harmonic from band n85 UL into band n77 DL. MSD value proposed based on similar conditions in CA_n12A-n77A as there is UL spectrum overlap (n12 UL: 699MHz - 716MHz ; n85 UL: 698MHz – 716MHz).

Table 5.x.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n85 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n85 | n77 | 5 | 15 | 20 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Based on the co-existence studies there are 5th harmonic mixing from band n77 UL into band n85 DL. MSD value proposed based on similar conditions in CA_n12A-n77A as there is UL spectrum overlap (n12 UL: 699MHz - 716MHz ; n85 UL: 698MHz – 716MHz).

Table 5.x.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n85 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n85 | 15 | 15 | 75 (RBstart=0) | 15 | 26.2 | NOTE 5 | UL1/DL5 |
| NOTE 5: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

#### 5.x.1.6	OOB blocking exception requirements

Exceptions added for CA_n77-n85

Table 5.x.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n77-n85

### 5.x.2		Specific for 2 bands UL CA

#### 5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2A.1.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n77A-n85A | 23 | +2/-3 |

#### 5.x.2.2		UE co-existence

Table 5.x.2.2-1 gives IMD interference analysis for CA_ n77-n85 with 2 ULs.

Table 5.x.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 3300 | 4200 | 698 | 716 |
| DL Frequency [MHz] | 3300 | 4200 | 728 | 746 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3502 | 2584 | 3998 | 4916 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5884 | 7702 | 2804 | 1868 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7298 | 9116 | 4696 | 5632 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 9184 | 11902 | 2106 | 1152 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5168 | 7004 | 7996 | 9832 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10598 | 13316 | 5394 | 6348 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 436 | 1408 | 16102 | 12484 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4452 | 6306 | 11204 | 8468 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6092 | 7064 | 13898 | 17516 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8694 | 10548 | 11296 | 14032 |

Based on the table 6.x.2.2-1, the 2nd order IMD product of CA_n77-n85 falls inside the RX of n77, but n77 is a TDD band, so there’s no RX during 2UL.

Based on the table 6.x.2.2-1, the 5th order IMD product of CA_n77-n85 falls inside the RX of n85.

Table 5.x.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.x.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) |  |  | MBW (MHz) | NOTE |
| CA_n77-n85 | E-UTRA Band 3, 5, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 30, 39, 40, 45, 53, 73, 74, 103 | FDL_low | - | FDL_high |  | -50 | 1 |  |  |
|  | Frequency range | 1884.5 |  | 1915.7 |  | -41 | 0.3 |  | 3 |
|  | E-UTRA Band 1, 2, 4, 7, 10, 25, 41, 34, 50, 51, 65, 66, 70 | FDL_low | - | FDL_high |  | -50 | 1 |  | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high |  | -38 | 1 |  | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high |  | -50 | 1 |  | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |  |  |

#### 5.x.2.3		REFSENS requirements

As seen in the coexistence study 5th IMD product might fall in own Rx band why MSD is needed for 2 band UL. Values from CA_n71A-n77A is reused due to similar frequency range and IMD order.

Table 5.x.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n77-n856 | n77 | 3590 | 10 | 25 | 3590 | N/A | TDD | N/A |
|  | n85 | 712 | 10 | 25 | 742 | 5.5 | FDD | IMD5 |
| NOTE 6: Considering the spectrum holdings of the operator for CA_n77(2A) (when one uplink sub block is assigned within 3300-3400MHz, the other uplink sub block is not assigned within 4000-4200MHz or vice versa), no IMD5 result will fall in Rx frequency range of band n3. Therefore, no MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). |  |  |  |  |  |  |  |  |

************************************* End of TP*****************************************