3GPP TSG RAN WG1 Meeting #105	R4-2220842

Toulouse, France, November 14 – November 18, 2022

Title:	LS on RF simulation parameters to be used in RAN4 for power domain enhancements

Response to:

Release:	Rel-18

Work Item:	NR_cov_enh2

Source:	RAN4

To:	RAN1

Cc:

Contact person:		Hiromasa Umeda, hiromasa (dot) umeda (at) nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:		None

# 1	Overall description

RAN4 discussed work split principles to be adopted in RAN4 for power domain enhancements throughout Rel-18 and made a following agreement in R4-2217745 in RAN4#104-bis-e:

- RAN4 follows below RAN1 agreements and focus on prepare for RF simulations

  - Establish evaluation parameters and side-conditions if any for both transparent and non-transparent schemes

    - The parameters and side-conditions will be updated if needed according to RAN1 input

    - Share the agreements with RAN1 that could affect RAN1 link level simulation

  - RAN4 can perform evaluations without RAN1 input for both transparent and non-transparent schemes

    - No discussion on simulation results of non-transparent scheme at least in RAN4#105

According to the above agreement, RAN4 would like to share following RF parameters and some side-conditions which may impact on RAN1 link level simulation for coverage enhancement.

Table 1 Simulation parameters for FR1

| Carrier frequency | 4GHz 700 MHz (optional) |
| --- | --- |
| Channel BW | Case 1: 20 MHz Case 2: 100 MHZ |
| SCS | Case 1: 15/30/60 kHz Case 2: 30 kHz |
| DMRS config | ZC, 2 symbols |
| Modulation | - Pi/2 BPSK - QPSK |
| Waveform | DFT-S-OFDM |
| Allocation type | Sweep over the channel |
| Extension factors | 0-0.375 |
| Channel | PUSCH, 14 OFDM symbols |
| Spectral shaping filter | - 3-tap, FD implementation - (0.335 1 0.335) - (0.28 1 0.28) - 2-tap: (1, 0.28) for FDSS QPSK w/o SE - Truncated RRC - No filter (reference case) NOTE1 |
| Power class | PC 3 |
| NOTE 1: A reference waveform) which meet the existing requirements for gain evaluation |  |

Table 3 Simulation parameters for FR2

| Carrier frequency | 28GHz |
| --- | --- |
| Channel BW | 400MHz |
| SCS | 120kHz |
| DMRS config | ZC, 2 symbols |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Allocation type | Sweep over the channel |
| Extension factors | 0- 0.375 |
| Channel | PUSCH, 14 OFDM symbols |
| Spectral shaping filter | - 3-tap, FD implementation - (0.335 1 0.335) - (0.28 1 0.28) - 2-tap: (1, 0.28) for FDSS QPSK w/o SE - Truncated RRC - No filter (reference case) NOTE1 |
| Power class | PC 3 |
| NOTE 1: A reference waveform which meet the existing requirements for gain evaluation |  |

# 2	Actions

To RAN1:

ACTION: 	RAN4 respectfully requests RAN1 to take the above into account in their future work.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #106	February 27th –March 03rd 2023		Athens, Greece

TSG-RAN WG4 Meeting #106-bis-e	April 17th – April 26th 2023		Electronic meeting