3GPP TSG-RAN WG4 Meeting # 106	R4-2300104

# Athens, Greece, February 27 – March 3, 2022

Source: 		Qualcomm Incorporated

Title: 	SSB based scheduling and measurement restrictions

Agenda item:	9.8.3.4

Document for:		Approval

# Introduction

In this paper, we present our view on the following aspects:

- Measurement Restriction

- Scheduling Availability

# Discussion

Measurement Restriction

When an L1 measurement resource is associated with one of active TCI states for group-based simultaneous mTRP support, UE may be able to receive and process another resource, e.g. CSI-RS for RLM/BFD, from the other TRP.

Proposal 1: A measurement restriction for mTRP can be defined such that the following can be supported:

- UE can receive SSB for L1-RSRP measurement from one TRP while performing CSI-RS based RLM, BFD, [CBD] from the other TRP if the resources are associated with active TCI states for simultaneous reception from the two TRPs

- UE can receive CSI-RS for L1-RSRP measurement from one TRP while performing CSI-RS based RLM, BFD, [CBD] from the other TRP if the resources are associated with active TCI states for simultaneous reception from the two TRPs

- The CSI-RSs are not in a resource set configured with repetition ON

Scheduling Availability

According to RAN1 spec (relevant statements are quoted in Figure 1), PDSCHs can be scheduled to a UE on OFDM symbols overlapping with SSB if the SSB is associated with one of the PDSCHs, which is illustrated in Figure 1.

Fig. 1 Scheduling availability in FR2 mTRP

Observation 1: The following two statements from TS38.214 allow a simultaneous reception of PDSCHs on SSB symbols if one of the PDSCHs is associated with the SSB:

- If at least one TCI codepoint indicates two TCI states and the UE receives the DM-RS for PDSCH and an SS/PBCH block in the same OFDM symbol(s), then the UE may assume that at least one DM-RS port for the PDSCH and SS/PBCH block are quasi co-located with 'QCL-TypeD', if 'QCL-TypeD' is applicable.

- If the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in different ControlResourceSets, and the UE receives the DM-RS for PDSCH(s) and an SS/PBCH block in the same OFDM symbol(s), then the UE may assume that at least one DM-RS port for the PDSCH(s) and SS/PBCH block are quasi co-located with 'QCL-TypeD', if 'QCL-TypeD' is applicable.

With Observation 1, the legacy scheduling restriction rule can be relaxed such that a simultaneous reception of PDSCHs from two TRPs on the SSB symbols can be supported. And for such scheduling relaxations, a proper set of side conditions needs to be defined, e.g. the relaxation is applied only when UE is in a simultaneous dual beam based mTRP mode.

Proposal 2: A scheduling availability during SSB based L1 measurements from two TRPs can be defined such that a simultaneous reception of PDSCHs from two TRPs on the SSB symbols can be supported if one of the PDSCHs is associated with the SSB, and the SSB configured for L1 measurements is a source of one of the activated TCI states. FFS on side conditions, e.g.

- UE is configured with active TCI states from two TRPs, and the association between the TCI states and the TRPs is explicitly known to the UE, i.e.

  - (single DCI based mTRP) at least one of the codepoints in the active TCI list for PDSCH includes two reference resources for qcl-TypeD from respective TRPs

  - (multi DCI based mTRP) two CORESETs QCL’ed with two reference resources for qcl-TypeD are configured

  - SNR > XdB for each TRP, where rank > 2 is expected

- Group-based L1-RSRP measurement is configured based on L3 measurements for the same measurement resources

# Conclusions

In this paper, we proposed the following.

Measurement Restriction

Proposal 1: A measurement restriction for mTRP can be defined such that the following can be supported:

- UE can receive SSB for L1-RSRP measurement from one TRP while performing CSI-RS based RLM, BFD, [CBD] from the other TRP if the resources are associated with active TCI states for simultaneous reception from the two TRPs

- UE can receive CSI-RS for L1-RSRP measurement from one TRP while performing CSI-RS based RLM, BFD, [CBD] from the other TRP if the resources are associated with active TCI states for simultaneous reception from the two TRPs

- The CSI-RSs are not in a resource set configured with repetition ON

Scheduling Availability

Observation 1: The following two statements from TS38.214 allow a simultaneous reception of PDSCHs on SSB symbols if one of the PDSCHs is associated with the SSB:

- If at least one TCI codepoint indicates two TCI states and the UE receives the DM-RS for PDSCH and an SS/PBCH block in the same OFDM symbol(s), then the UE may assume that at least one DM-RS port for the PDSCH and SS/PBCH block are quasi co-located with 'QCL-TypeD', if 'QCL-TypeD' is applicable.

- If the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in different ControlResourceSets, and the UE receives the DM-RS for PDSCH(s) and an SS/PBCH block in the same OFDM symbol(s), then the UE may assume that at least one DM-RS port for the PDSCH(s) and SS/PBCH block are quasi co-located with 'QCL-TypeD', if 'QCL-TypeD' is applicable.

Proposal 2: A scheduling availability during SSB based L1 measurements from two TRPs can be defined such that a simultaneous reception of PDSCHs from two TRPs on the SSB symbols can be supported if one of the PDSCHs is associated with the SSB, and the SSB configured for L1 measurements is a source of one of the activated TCI states. FFS on side conditions, e.g.

- UE is configured with active TCI states from two TRPs, and the association between the TCI states and the TRPs is explicitly known to the UE, i.e.

  - (single DCI based mTRP) at least one of the codepoints in the active TCI list for PDSCH includes two reference resources for qcl-TypeD from respective TRPs

  - (multi DCI based mTRP) two CORESETs QCL’ed with two reference resources for qcl-TypeD are configured

  - SNR > XdB for each TRP, where rank > 2 is expected

- Group-based L1-RSRP measurement is configured based on L3 measurements for the same measurement resources