3GPP TSG-RAN WG4 Meeting # 106	R4-2300252

# Athens, Greece, February 27 – March 3, 2022

Agenda item:	9.28.3.2

Source:	Apple

Title:	On RRM requirements for NR MIMO evolution for downlink and uplink

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN#94-e a new WI for MIMO Evolution for Downlink and Uplink was approved and the WID was further revised in [1].  In this first meeting for R18 MIMO in RAN4, we discuss the RRM requirement impact for this WI .

# 2. Discussion

The WI for MIMO evolution on downlink and uplink covers enhancements for several features as captured in the objectives of the WID.

The detailed objectives are as follows: RAN1: - Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM, - Striving for a common design between DL and UL DMRS - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32 - Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study. - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

The enhancements being studied and introduced in RAN1 can be classified under the following categories:

- Multi-TRP enhancements

- CSI Enhancement

- Reference Signal Enhancement

- Enhanced Uplink Transmission

Under multi-TRP enhancements RAN1 is defining extension to unified TCI framework for multi-TRP. In Rel-17 RAN4 defined requirements for TCI state switching for unified DL, UL and joint TCI.  Although the current requirements in Rel-17 are not specific to single TRP or multi-TRP, we think that there could be potential requirements impact to RRM requirements.

Observation #1: The unified TCI framework extension for multi-TRP could have potential RRM impact.

Proposal #1: RAN4 discuss impact to RRM requirements with extension of unified TCI framework to multi-TRP

Another topic under multi-TRP enhancements is Two TAs for multi-DCI. This topic has impact to RRM requirements for defining the timing related requirements for two TA. RAN4 has a dedicated AI for discussion of this topic.

Observation #2: For two TA for multi-DCI RAN4 has dedicated AI for RRM.

Under CSI enhancements RAN1 is introducing enhancements to CSI reporting in high/medium velocities, and enhancements for CSI acquisition for coherent-JT. Both these enhancements would not have any impact to RRM requirements.

Observation #3: The CSI enhancements in Rel-18 are not expected to have impact to RRM requirements.

For Reference signal enhancement RAN1 is introducing enhancements to specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO and to support SRS enhancement for 8TX operation and TDD coherent joint transmission. Both these enhancements are not expected to have any RRM impact.

Observation #4: No RRM impact is identified for reference signal enhancement in R18.

Under enhanced uplink transmission, there are updates for UL precoding indication for multi-panel transmission and SRI/TPMI enhancement for enabling 8 TX UL transmission targeting CPE/FWA/vehicle/industrial devices. We don’t see any impact to RRRM requirements for enhanced uplink transmission.

Observation #5: The enhanced uplink transmission is not expected to have RRM impact.

The table below consolidates our observations on the potential RRM impact for topics covered in R18 MIMO evolution.

Table 1: Identified RRM Impact for R18 MIMO Evo topics

| Topic | RRM Impact |
| --- | --- |
| Multi-TRP Enhancement |  |
| Unified TCI Framework | Requirements for unified TCI for multi-TRP transmission |
| Two TA for multi-DCI | RRM requirements for 2 TA |
| CSI Enhancement |  |
| CSI Enhancement | No RRM impact identified |
| Reference Signal Enhancement |  |
| Increased number of orthogonal DMRS ports | No RRM impact identified |
| SRS enhancement | No RRM impact identified |
| Enhanced Uplink Transmission |  |
| UL precoding indication for multi-panel transmission | No RRM impact identified |
| SRI/TPMI enhancement | No RRM impact identified |

# 3. Conclusion

In this paper, we provide our views on potential RRM impact for R18 MIMO evolution topics. Our observations and proposals are captured below:

| Topic | RRM Impact |
| --- | --- |
| Multi-TRP Enhancement |  |
| Unified TCI Framework | Requirements for unified TCI for multi-TRP transmission |
| Two TA for multi-DCI | RRM requirements for 2 TA |
| CSI Enhancement |  |
| CSI Enhancement | No RRM impact identified |
| Reference Signal Enhancement |  |
| Increased number of orthogonal DMRS ports | No RRM impact identified |
| SRS enhancement | No RRM impact identified |
| Enhanced Uplink Transmission |  |
| UL precoding indication for multi-panel transmission | No RRM impact identified |
| SRI/TPMI enhancement | No RRM impact identified |

Observation #1: The unified TCI framework extension for multi-TRP could have potential RRM impact.

Proposal #1: RAN4 discuss impact to RRM requirements with extension of unified TCI framework to multi-TRP

Observation #2: For two TA for multi-DCI RAN4 has dedicated AI for RRM.

Observation #3: The CSI enhancements in Rel-18 are not expected to have impact to RRM requirements.

Observation #4: No RRM impact is identified for reference signal enhancement in R18.

Observation #5: The enhanced uplink transmission is not expected to have RRM impact.

# Reference

- RP-223276, “WID Update: MIMO Evolution for Downlink and Uplink”, Samsung.