3GPP TSG-RAN WG4 Meeting #106	R4- 2300715

Athens, Greece, 27th Feb – 3rd Mar, 2023

Source: 	Huawei, HiSilicon

Title:	MSD for PC2 FDD bands

Agenda Item:	8.23.2

Document for:	Approval

# 1	Introduction

This paper presents the PC2 MSD analysis for the FDD bands as per the WF in [1].

# 2	Discussion

### 2.1 PC2 MSD for 2Tx

The same methodology [2] as used for Rel-17 PC2 FDD WI is followed to derive the MSD estimation for 2Tx based on the existing PC3 REFSENS requirements. The results are shown in Table 1 below.

Table 1: REFSENS degradation for 2Tx (SCS=15kHz)

| Band | Channel BW (MHz) / MSD (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| n2 | 2.0 | 2.0 | 2.0 | 2.0 | 2.0 | 6.2 | 6.2 | 6.5 | - | - |
| n5 | 2.0 | 2.0 | 2.0 | 5.9 | 6.2 | - | - | - | - | - |
| n8 | 3.1 | 3.2 | 3.8 | 6.2 | - | - | 7.1 | - | - | - |
| n25 | 3.6 | 3.6 | 3.6 | 3.6 | 3.6 | 6.7 | 6.7 | 6.9 | 7.0 | - |
| n26 | 2.6 | 2.4 | 2.3 | 5.6 | 6.3 | 6.8 | - | - | - | - |
| n28 | 1.4 | 1.4 | 1.4 | 3.1 | 6.4 | 7.1 | - | - | - | - |
| n71 | 2.9 | 2.9 | 3.6 | 6.2 | 6.4 | 6.6 | 6.8 | - | - | - |
| n85 | 3.1 | 3.2 | 6.9 | - | - | - | - | - | - | - |

### 2.2 PC2 MSD for 1Tx

Some measurement results on Tx leakage were provided in [3] by Skyworks in the last meeting. When comparing PC2 1Tx with PC3 1Tx, it can be seen that the Tx leakage power difference is around 3dB for relatively small BWs such as 5, 10 or 15MHz. And the difference is 4~5 dB for relatively large BWs such as 35, 30, 25 or 20MHz.

In the former case, the dominant Tx leakage is PA wideband noise or IMD products of very large orders (9 or above). In the latter case, the dominant Tx leakage is IMD5 or IMD7 products. Based on this observation, the MSD estimation for PC2 1Tx are derived, and the results are shown in Table 2 below.

Table 2: REFSENS degradation for 1Tx (SCS=15kHz)

| Band | Channel BW (MHz) / MSD (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| n2 | 0.9 | 0.9 | 1.0 | 1.3 | 1.3 | 3.2 | 3.2 | 3.4 | - | - |
| n5 | 0.9 | 0.9 | 1.0 | 3.1 | 3.3 | - | - | - | - | - |
| n8 | 1.2 | 1.2 | 1.4 | 3.3 | - | - | 3.8 | - | - | - |
| n25 | 1.4 | 1.4 | 1.4 | 1.9 | 1.9 | 3.5 | 3.5 | 3.7 | 3.8 | - |
| n26 | 1.0 | 1.0 | 1.0 | 2.9 | 3.3 | 3.6 | - | - | - | - |
| n28 | 0.8 | 0.8 | 0.8 | 1.7 | 3.4 | 3.8 | - | - | - | - |
| n71 | 1.2 | 1.2 | 1.4 | 3.2 | 3.4 | 3.5 | 3.6 | - | - | - |
| n85 | 1.2 | 1.2 | 2.7 | - | - | - | - | - | - | - |

# 3	Conclusion

In this paper, we have presented our analysis on the REFSENS degradation for both PC2 1Tx and PC2 2Tx. And we have the following proposal:

Proposal 1: Consider the results in Table 1 and Table 2 when defining PC2 REFSENS degradation requirements for the listed FDD bands.

# References

- R4-2217121 WF on requirements for HPUE_Basket_FDD, China Unicom, RAN4#104bis-e

- R4-2204079 On remaining issues for PC2 FDD bands, Huawei, HiSilicon, RAN4#102-e

- R4-2219723 Preliminary REFSENS results for 1Tx and 2Tx PC2 FDD bands, Skyworks Solutions Inc., RAN4#105