3GPP TSG-RAN WG4 Meeting # 106      		R4-2300784

Athens, GR, Feb 27 - Mar 03, 2023

Agenda Item:	9.13.5.3

Source: 	CMCC

Title: 			 Discussion on BS demodulation requirements for ATG scenario

Document for:	Discussion

- Introduction

The following objectives has been included in WID on Further enhancement on Air-to-ground network for NR [1]:

The performance part of the work item includes - Specify corresponding RRM performance requirements and test cases for ATG UE [RAN4] - Specify corresponding demodulation performance requirements for ATG BS [RAN4] - Specify corresponding demodulation performance and CSI reporting requirements for ATG UE/BS [RAN4] - Specify test procedures for ATG BS conformance testing and conformance requirements [RAN4]

According to the work plan [2], RAN4 will start to discuss the BS demodulation/conformance test performance requirements for ATG BS from this meeting. In this contribution, we give our views on test scope, network scenario, channel model, simulation assumption and so on.

- Test scope

For uplink, PUSCH demodulation, PUCCH demodulation and PRACH detection requirements should be taken into consideration.

Whether to reuse legacy requirements or define new requirements for ATG could be discussed after propagation condition is agreed.

Proposal 1: Investigate PUSCH demodulation, PUCCH demodulation and PRACH detection requirements for R18 ATG.

- Network scenario

SCS and channel bandwidth

For SCS and bandwidth, we prefer to consider 5MHz, 20MHz and 40MHz for 15kHz SCS, 40MHz, 60MHz and 100MHz for 30kHz SCS as ATG scenario should consider high data rate demand.

Proposal 2: Define PUSCH requirements for following SCS and bandwidth combination:

- 5MHz, 20MHz and 40MHz for 15kHz SCS

- 40MHz, 60MHz and 100MHz for 30kHz SCS

TDD pattern

Considering of the distance of CPE and ATG BS may achieve 250km, a large guard period may be needed to prevent interference from DL to UL, the length should at least cover 2*propagation delay ≈ 1.7ms. The original TDD pattern which guard period short than 1 slot cannot be applied to ATG in practical scenario. Therefore, we propose to define a new TDD pattern 30D4S6U, which is applied for ATG scenario only.

Proposal 3: Define a new TDD pattern 30D4S6U which only applied for ATG scenario.

- Channel model

Doppler

One special feature of ATG scenario is that UE move at extremely high speed. The maximum UE speed is about 1200km/h, which will introduce severe doppler frequency shift, about ±2.22ppm for UL. In order to calculate the doppler value, for carrier frequency, we use the same assumption of RF co-existence evaluation, 2GHz for FDD and 4GHz for TDD. The calculation result is shown in table below:

| Maximum Doppler (±1.11ppm) | UL |
| --- | --- |
| FDD 2GHz; 15kHz SCS; 1200km/h | ±4.44kHz |
| TDD 4GHz; 30kHz SCS; 1200km/h | ±8.88kHz |

Observation 1: The maximum doppler in ATG scenario is about ±2.22ppm for UL.

- For Doppler frequency shift in FDD 2GHz, ±4.44kHz for UL.

- For Doppler frequency shift in TDD 4GHz, ±8.88kHz for UL.

Frequency offset tracking

For uplink, if the gap between adjacent RS could be 1 symbol, then BS is able to perform the UL frequency offset tracking in ATG scenario.

Observation 2: The current NR solution is sufficient for frequency offset tracking.

- For UL, guarantee that the gap between adjacent RS could be 1 symbol.

UE transmit frequency pre-compensation

The UE transmit frequency pre-compensation solution is used in NTN system, to address the high doppler issue. This solution can be applied provided that UE could acquire its location, speed and BS’s location.

Currently, we have agreed that UE could acquire its location by GNSS, and acquire BS’s location by PVT information. However, how does UE acquire its speed and the accuracy is unclear.

If UE could acquire the speed, and use the result in frequency compensation, then we are fine to also consider the UE transmit frequency pre-compensation solution in ATG, which could increase the system throughput with less RS configured.

Observation 3: UE transmit frequency pre-compensation solution can be applied provided that UE could acquire its location, speed and BS’s location.

- UE could acquire its location by GNSS, and acquire BS’s location by PVT information

- how doss UE acquire its speed is unclear

With all analysis above, there are two ways to address the doppler issue

- Alt 1: Use legacy frequency offset tracking solution with proper RS configuration

- Alt 2: Use UE transmit/receive frequency pre-compensation

These two ways will lead to different channel model, considering there is almost no diffraction, reflection and other clutter between BS and UE:

For Alt 1, a new channel model similar to HST single-tap should be defined with higher doppler value, the doppler value depends on carrier frequency and maximum UE speed.

For Alt 2, if frequency offset can be perfectly compensated by UE, then we can use the AWGN as channel model. If there is still residual frequency offset after frequency pre-compensation, then a new channel model should be defined, which is like AWGN with relatively low doppler, the doppler value depends on residual frequency offset after pre-compensation.

We can take these two methods as the baseline for further discussion.

Proposal 4: Further discuss following two frequency tracking/compensation method and corresponding channel model:

- Alt 1:

  - Use legacy frequency offset tracking solution with proper RS configuration

  - Define a new channel model which similar to HST single-tap, the maximum doppler value depends on carrier frequency and maximum UE speed.

- Alt 2:

  - Use UE transmit/receive frequency pre-compensation

    - If frequency offset can be perfectly compensated by UE, then use the AWGN as channel model

    - If there is still residual frequency offset after frequency pre-compensation, then define a new channel model, which is like AWGN with relatively low doppler. The doppler value depends on residual frequency offset after pre-compensation

- PRACH format

For PRACH, there are format 0/1/2/3/Ax/Bx/Cx. The SCS of format 0/1/2 is 1.25 kHz, the SCS of format 3 is 5 kHz and the SCS of format Ax/Bx/Cx is the multiple of 15 kHz. With the constrains of ∆fd≪∆fRA , if UE don’t perform transmit frequency pre-compensation, then only format Ax/Bx/Cx can be applied to ATG. If UE perform transmit frequency pre-compensation of PRACH, then considering of large cell range, format 0/1/2/3 is preferred.

Proposal 5: For ATG PRACH format,

- if RAN4 assume UE don’t perform transmit frequency pre-compensation, then format Ax/Bx/Cx can be applied to ATG

- if RAN4 assume UE perform transmit frequency pre-compensation as baseline, then format 0/1/2/3 is preferred.

- PUSCH parameters

PUSCH configuration

Proposal 6: If PUSCH demodulation requirements should be defined, we propose following parameters for PUSCH configuration:

- Mapping type: type A

- Starting symbol: 0

- Length: 14

- PUSCH aggregation factor: 1 and 2

Antenna configuration

For Tx antenna number, we prefer to consider both 1Tx and 2Tx at this stage. For the Rx antenna number, we think 2Rx, 4Rx and 8Rx should all be covered.

Proposal 7: Consider antenna configuration of 1x2, 1x4, 1x8, 2x2, 2x4, and 2x8.

RANK and MCS

For RANK, we propose to simulate both RANK 1 and RANK 2. For MCS, we can take 16QAM, 64QAM and 256QAM as simulation assumption.  Further selection of Rank and MCS can be done according to SNR in simulation results.

Proposal 8: For rank, cover both rank 1 and rank 2 in simulation, and do the further selection according to simulation results.

Proposal 9: For MCS, consider 16QAM, 64QAM and 256QAM in simulation, and do the further selection according to simulation results.

For UL 2Tx and 2-layer transmission, the existing codebook-based transmission with TPMI index=0 can be reused as the starting point.

Proposal 10: For UL 2Tx and 2-layer transmission scheme, the existing codebook-based transmission with TPMI index=0 can be reused as the starting point.

Transform precoding

Considering large coverage demand and UL channel capacity demand in ATG, both Transform precoding disabled and enabled can be considered in simulation.

Proposal 11: Consider CP-OFDM and DFT-S-OFDM waveform for requirement definition.

DMRS configuration

DMRS configuration may related to channel model. For example, if BS perform the frequency offset tracking as legacy, additional 2 DMRS should be configured. If UE perform UE transmit/receive frequency pre-compensation, then 1 additional DMRS or no additional DMRS may also ok.

Proposal 12: If BS perform the frequency offset tracking as legacy, additional 2 DMRS should be configured. If UE perform UE transmit/receive frequency pre-compensation, then 1 additional DMRS or no additional DMRS may also ok.

PTRS and SRS configuration

PTRS and SRS configuration also related to channel model. If BS should perform the frequency offset tracking, to deal with extremely high Doppler, PTRS and SRS may need to be configured to enhance the frequency offset tracking performance.

Proposal 13: Whether and how to configure PTRS and SRS is related with propagator model, which should be discussed after propagation model is agreed.

Other parameters

For other parameters, we propose to reuse the configuration of FR1 High speed train.

| Parameter |  | Value |
| --- | --- | --- |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Test metric |  | 70% maximum throughput |

Proposal 14: Reuse the configuration of FR1 High speed train for the following parameters:

| Parameter |  | Value |
| --- | --- | --- |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Test metric |  | 70% maximum throughput |

- Conclusion

Based on the discussion above, the following and proposals are concluded.

Proposal 1: Investigate PUSCH demodulation, PUCCH demodulation and PRACH detection requirements for R18 ATG.

Proposal 2: Define PUSCH requirements for following SCS and bandwidth combination:

- 5MHz, 20MHz and 40MHz for 15kHz SCS

- 40MHz, 60MHz and 100MHz for 30kHz SCS

Proposal 3: Define a new TDD pattern 30D4S6U which only applied for ATG scenario.

Observation 1: The maximum doppler in ATG scenario is about ±2.22ppm for UL.

- For Doppler frequency shift in FDD 2GHz, ±4.44kHz for UL.

- For Doppler frequency shift in TDD 4GHz, ±8.88kHz for UL.

Observation 2: The current NR solution is sufficient for frequency offset tracking.

- For UL, guarantee that the gap between adjacent RS could be 1 symbol.

Observation 3: UE transmit frequency pre-compensation solution can be applied provided that UE could acquire its location, speed and BS’s location.

- UE could acquire its location by GNSS, and acquire BS’s location by PVT information

- how doss UE acquire its speed is unclear

Proposal 4: Further discuss following two frequency tracking/compensation method and corresponding channel model:

- Alt 1:

  - Use legacy frequency offset tracking solution with proper RS configuration

  - Define a new channel model which similar to HST single-tap, the maximum doppler value depends on carrier frequency and maximum UE speed.

- Alt 2:

  - Use UE transmit/receive frequency pre-compensation

    - If frequency offset can be perfectly compensated by UE, then use the AWGN as channel model

    - If there is still residual frequency offset after frequency pre-compensation, then define a new channel model, which is like AWGN with relatively low doppler. The doppler value depends on residual frequency offset after pre-compensation

Proposal 5: For ATG PRACH format,

- if RAN4 assume UE don’t perform transmit frequency pre-compensation, then format Ax/Bx/Cx can be applied to ATG

- if RAN4 assume UE perform transmit frequency pre-compensation as baseline, then format 0/1/2/3 is preferred.

Proposal 6: If PUSCH demodulation requirements should be defined, we propose following parameters for PUSCH configuration:

- Mapping type: type A

- Starting symbol: 0

- Length: 14

- PUSCH aggregation factor: 1 and 2

Proposal 7: Consider antenna configuration of 1x2, 1x4, 1x8, 2x2, 2x4, and 2x8.

Proposal 8: For rank, cover both rank 1 and rank 2 in simulation, and do the further selection according to simulation results.

Proposal 9: For MCS, consider 16QAM, 64QAM and 256QAM in simulation, and do the further selection according to simulation results.

Proposal 10: For UL 2Tx and 2-layer transmission scheme, the existing codebook-based transmission with TPMI index=0 can be reused as the starting point.

Proposal 11: Consider CP-OFDM and DFT-S-OFDM waveform for requirement definition.

Proposal 12: If BS perform the frequency offset tracking as legacy, additional 2 DMRS should be configured. If UE perform UE transmit/receive frequency pre-compensation, then 1 additional DMRS or no additional DMRS may also ok.

Proposal 13: Whether and how to configure PTRS and SRS is related with propagator model, which should be discussed after propagation model is agreed.

Proposal 14: Reuse the configuration of FR1 High speed train for the following parameters:

| Parameter |  | Value |
| --- | --- | --- |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Test metric |  | 70% maximum throughput |

- Reference

[1] RP-221369, Revised WID on Air-to-ground network for NR, CMCC.

[2] R4-2211953, Work plan on Rel-18 ATG, CMCC.