3GPP TSG-RAN WG4 Meeting # 106	R4-2301045

Athens, Greece, February 27 – March 3, 2023

Agenda Item: 9.8.3.6

Source: MediaTek Inc.

Title: Discussion on receive timing difference for multi-Rx UE

Document for: Discussion

# Introduction

According to WF [1] and discussion summary [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. In this meeting, this WI is divided into six agenda items to be discussed: (1) general issues, (2) L1 measurement, (3) RLM/BFD/CBD, (4) scheduling/measurement restriction, (5) dual TCI state switching and (6) receive timing difference. The discussion in this paper focus on the “receive timing difference”.

# Discussion

For this issue, we think the key point of whether UE can measure the signals with timing difference larger than on CP is not only related to the number of the UE Rx antenna modules. It also involves the number of FFT module and timing estimation/tracking/compensation modules in the baseband. If UE wants to measure the signals by different antenna modules with timing difference larger than one CP, then the number of modules mentioned above should be doubled. Furthermore, it is not clear whether 4-layer MIMO can still be supported when receive timing difference is larger than CP.

Observation 1: For receiving signals with timing difference > CP by 2 antenna modules, the number of the FFT modules, timing estimation/tracking/compensation also need be doubled.

Proposal 1: The timing difference between different antenna modules is at least within one CP.

# Summary

In this paper, the discussion of FR2 multiple panels is provided. We have the following proposal:

Observation 1: For receiving signals with timing difference > CP by 2 antenna modules, the number of the FFT modules, timing estimation/tracking/compensation also need be doubled.

Proposal 1: The timing difference between different antenna modules is at least within one CP.

# Reference

- R4-2220402, WF on NR FR2 multi-Rx chain DL reception – Part 3, Ericsson, RAN4 #105

- R4-2220063, Topic summary for [105][217] FR2_multiRx_RRM_part3, Ericsson, RAN4 #105