# 3GPP TSG-RAN WG4 Meeting # 106		  R4-2301226

# Athens, Greece, Feb. 27-Mar. 03, 2023

# Source:	ZTE Corporation

# Title:	Discussion on UE RF requirements for 900 MHz LTE Band

# Agenda Item:	10.4.3

# Document for:	Approval

- Introduction

During RAN#98-e meeting, a new Work Item [1] has been approved on introduction of 900 MHz LTE Band in the US. In this contribution, we want to share some initial views on UE RF requirements.

- Specify a new E-UTRA/LTE FDD operating band operating with

  - UL: 896 – 901 MHz and DL 935 – 940 MHz

  - To support 5 MHz, 3 MHz, 1.4 MHz and NB-IoT channel bandwidths with 15 kHz SCS and 3.75kHz SCS.

- Study and specify the co-existence requirements

  - Support FCC OOB emission mask in FCC-20-67A1 for all the defined channel bandwidths in the proposed band to ensure co-existence with adjacent narrowband and broadband networks to avoid interference.

  - Study and if needed, specify the protection for B5 and B26 including MPR.

  - Assume reusing band B8 duplexer

- Specify UE RF core requirements to operate in North America with primary operations in the US.

  - Assume power class 3 (23 dBm).

- Specify necessary BS RF core requirement.

- Discussion

The summary of the corrections on TS 36.101 are given in Table 1.

Table 1. Summary of related RF requirement

| Tx RF requirements part |  |
| --- | --- |
| UE maximum output power | 23dBm maximum output power (i.e. PC3) for the new added FDD band. |
| MPR | No requirement impacts. |
| Configured transmitted power | No requirement impacts. |
| Output power dynamics | No requirement impacts. |
| Transmit signal quality | No requirement impacts. |
| Output RF spectrum emissions |  |
| - Out of band emission | No requirement impacts. |
| - Additional emission mask | The Report and Order establishes an out-of-band emissions (OOBE) limit outside a licensee’s frequency band of operation to be attenuated by at least 43 + 10 log (P) dB for uplink operations in the 897.5-900.5 MHz band. |
| - ACLR | No requirement impacts. |
| Spurious emissions |  |
| - Spurious emissions for UE co-existence | To add the new added FDD band as the protected band or co-existence with other operating bands. |
| - Additional spurious emissions | To add the additional spurious emissions requirements for the new added FDD band. |
| Receiver characteristics |  |
| Reference sensitivity | If we reuse band B8 duplexer, we can follow the reference sensitivity requirement for B8 defined in TS 36.101. |
| Maximum input level | No requirement impacts. |
| ACS | No requirement impacts. |
| Blocking characteristics | No requirement impacts. |
| Spurious response | No requirement impacts. |
| Intermodulation | No requirement impacts. |
| Spurious emissions | No requirement impacts. |

- Conclusion

In this contribution, we give some initial discussion on UE RF requirements for 900 MHz LTE band and the proposal is made as following:

Proposal 1: To use the proposals in Table 1 for 900 MHz LTE band UE RF requirements.

- Reference

RP-223530, New WID on Introduction of 900 MHz LTE Band in the US, Anterix