# 3GPP TSG-RAN WG4 Meeting #106	R4-2301530

Athens, Greece, February 27 – March 3, 2023

Source: 	vivo

Title: 	Discussion on frequency range definition update for TS 38.101-1/2/3

Agenda Item:	6.2.1

Document for:	Approval

Introduction

In Rel-17 NR_ext_to_71GHz, the definition for FR2 was extended to 71GHz and sub-frequency ranges FR2-1 (24250 MHz – 52600 MHz) and FR2-2 (52600 MHz – 71000 MHz) were introduced. This frequency range update has been introduced in TS38.101-2 in Rel-17 and Rel-18. In this contribution, we discuss the frequency range definition update for TS 38.101-1/2/3.

Discussion

In the general part (section 5.1) of TS 38.101-1/3, the frequency range is still defined as follows:

Table 5.1-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | 24250 MHz – 52600 MHz |

However, the FR2 definition has been updated in NR_ext_to_71GHz. From current understanding, the FR2 definition in TS 38.101-1 and TS38.101-3 would be wrong. Thus, we suggest to update the FR2 definition for TS 38.101-1 and TS 38.101-3.

Proposal 1: Update the FR2 definition for TS 38.101-1 and TS 38.101-3.

Another issue is to update the FR2 definition from which release of the specification TS 38.101-1/2/3. One understanding is that frequency range definition would be general and release independent and this frequency range should be updated from Rel-15. Another understanding is that FR2 definition update was introduced in Rel-17, the change request should only be introduced from Rel-17. In this case, FR2 definition would be different from Rel-17/Rel-18 and Rel-15/Rel-16. Thus, we suggest to discuss from which release to introduce FR2 definition for TS 38.101-1/2/3.

Proposal 2：Discuss from which release to introduce FR2 definition for TS 38.101-1/2/3：

- Rel-15

- Rel-17

Conclusion

This contribution discusses the frequency range update for TS 38.101-1/2/3. The following proposals are made:

Proposal 1: Update the FR2 definition for TS 38.101-1 and TS 38.101-3.

Proposal 2：Discuss from which release to introduce FR2 definition for TS 38.101-1/2/3：

- Rel-15

- Rel-17