3GPP TSG-RAN WG4 Meeting#106                               R4-2301729

Athens, Greece,27th Feb – 3rd Mar, 2023

Agenda item:	9.13.3

Source: 	ZTE Corporation

Title: 	Further discussion on ATG BS RF requirements

Document for:	Approval

Introduction

In RAN#95e meeting, the work item [RP-221369] on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package. During the previous RAN4 meetings, we have reached good progress for ATG BS in [6], however there are still some open issues left for further discussion pending the outcome of ATG coexistence study as following. In this contribution, we want to share further views on ATG BS RF requirements based on the initial evaluation results obtained so far.

Table 6.1-1: Simulation scenarios for ATG coexistence study

| No. | Combination | Aggressor |  | Victim |  | Simulation frequency | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode |  |  |  |
| 1 | TN with ATG | ATG DL | 100MHz TDD | TN rural DL | 100MHz /TDD | 3.5 GHz |  | Phase 1 |
| 2 | TN with ATG | ATG UL | 100MHz TDD | TN rural UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 3 | TN with ATG | TN rural DL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 4 | TN with ATG | TN rural UL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 5 | TN with ATG | ATG DL | 100MHz TDD | TN rural UL | 100MHz /TDD | 3.5GHz |  | FFS |
| 6 | TN with ATG | ATG UL | 100MHz TDD | TN rural DL | 100MHz TDD | 3.5GHz |  | FFS |
| 7 | TN with ATG | TN rural DL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | FFS |
| 8 | TN with ATG | TN rural UL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | FFS |
| 9 | TN with ATG | ATG DL | 20MHz FDD | TN rural DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 10 | TN with ATG | ATG UL | 20MHz FDD | TN rural UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 11 | TN with ATG | TN rural DL | 20MHz FDD | ATG DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 12 | TN with ATG | TN rural UL | 20MHz FDD | ATG UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 13 | TN with ATG | ATG UL | 20MHz FDD | TN rural DL | 20MHz TDD | 2 GHz | n1/n39 | FFS |
| 14 | TN with ATG | TN rural DL | 20MHz TDD | ATG UL | 20MHz FDD | 2 GHz | n39/n1 | FFS |

Discussion

## 2.1. ACLR requirement of ATG BS

Based on the initial simulation results obtained so far for Case 1 and Case 9 as shown in Figure 1 and Figure 2, ACI caused by ATG DL is quite limited due to both the sparse deployment sites of ATG BS and its uptilt beamforming towards the aircraft in the sky. In short, based on the initial simulation results in Case 1 and Case 9 dominating the ATG BS DL ACLR requirement, it should be sufficient to reuse the legacy FR1 ACLR 45dBc requirement for ATG BS.

Figure 1. simulation results for Case 1: ATG DL interfering TN DL @4GHz

Figure 2. simulation results for Case 9: ATG DL interfering TN DL @2GHz

Observation 1: based on the initial simulation results in Case 1 and Case 9, it’s sufficient to reuse the legacy FR1 ACLR 45dBc requirement for ATG BS.

## 2.2. ACS requirement of ATG BS

Based on the initial simulation results obtained so far for Case 4 and Case 12 as shown in Figure 3 and Figure 4, regardless of co-located deployment between ATG BS and the legacy TN BS, it should be sufficient to reuse the legacy FR1 ACS 46dBc requirement for ATG BS.

-

Figure 3a. simulation results for Case 4: TN UL interfering ATG UL @4GHz [non co-located deployment]

Figure 3b. simulation results for Case 4: TN UL interfering ATG UL @4GHz [co-located deployment]

Figure 4a. simulation results for Case 12: TN UL interfering ATG UL @2GHz [non co-located deployment]

- Figure 4b. simulation results for Case 12: TN UL interfering ATG UL @2GHz [co-located deployment]

Observation 2: based on the initial simulation results in Case 4 and Case 12, it’s sufficient to reuse the legacy FR1 ACS 46dBc requirement for ATG BS.

Conclusions

In this contribution, we shared some further views on ATG BS RF requirement and observations are made as following:

Observation 1: based on the initial simulation results in Case 1 and Case 9, it’s sufficient to reuse the legacy FR1 ACLR 45dBc requirement for ATG BS.

Observation 2: based on the initial simulation results in Case 4 and Case 12, it’s sufficient to reuse the legacy FR1 ACS 46dBc requirement for ATG BS.

References

- RP-221369, Core part: Air-to-ground network for NR, CMCC, Approved.

- R4-2214461,WF on BS requirements for ATG, ZTE, Approved.

- R4-2215147,WF on UE requirements for ATG, Apple, Approved.

- R4-2217454 WF on ATG BS RF requirements, ZTE, Approved.

- R4-2220542,WF on ATG co-existence evaluation, CMCC, Approved.

- R4-2220250, WF for ATG BS RF requirements, ZTE, Approved.

- R4-2220540,TP for TR 38.876 to capture scenarios and network layout, Ericsson, approved.