3GPP TSG-RAN WG4 Meeting#107	R4-2307046

Incheon, Korea, May 22nd – 26th, 2023

Source: 	NYCU

Title: 	Discussion on system parameters for above 10 GHz

Agenda item:	 8.27.1.1

Document for:	Discussion

# 1 Introduction

During the most recent RAN4#106bis-e meeting, bands definition and frequency ranges were discussed. In this document, we present our perspective on frequency range based on the Way Forward [1].

- Discussion

There were still some issues that needed to be discussed regarding the agreed Way Forward for defining NTN frequency ranges during the last meeting.

Sub-topic 1-1 (Issues 1-1-1, 1-1-2, 1-1-3, 1-1-4), updated: Above 10 GHz NTN band definition and related parameters - Starting with option below and further work on the text proposal of “note 1 and note 2”. LS to RAN1/RAN2 can be considered. - It’s not precluded to further extend the frequency ranges in the future if needed and justified. - Agree the definition of NTN frequency ranges as shown in the Proposed Table 5.1-1 to be introduced to TS 38.101-5 and TS 38.108. Proposed Table 5.1-1: Definition of NTN frequency ranges Frequency range designation; Corresponding frequency range. FR1-NTN1; 410 MHz – 7125 MHz. FR2-NTN2; 17300 MHz – 30000 MHz. NOTE 1: [NTN bands within this frequency range are regarded as a FR1 band when references from other specifications.] NOTE 2: [NTN bands within this frequency range are regarded as a FR2 band when references from other specifications.].

  - Text Proposal of “note 1 and note 2”

In our view, NOTE1 is reasonable, but NOTE2 is unclear because the range of FR2-NTN is not fully contained within the terrestrial FR2 frequency band. Therefore, we suggest emphasizing that it belongs to the FR2 band when referenced from other specifications.

Proposal 1: Remove square bracket of Note 1. That is “NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications.”

Proposal 2: Change NOTE 2 to “NOTE 2: NTN bands in this frequency range are considered as belonging to the FR2 band when referenced from other specifications.”

  - Frequency Range

Our observation indicates that specific services can benefit from utilizing both Ku and Ka bands, and we suggest that RAN4 expand the frequency ranges.

Proposal 3: We support the expansion of frequency ranges and we do not object to including a sub-bullet that specifies the frequency bands.

# 3 Conclusion

Summarized the proposal:

Proposal 1: Remove square bracket of Note 1. That is “NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications.”

Proposal 2: Change NOTE 2 to “NOTE 2: NTN bands in this frequency range are considered as belonging to the FR2 band when referenced from other specifications.”

Proposal 3: We support the expansion of frequency ranges and we do not object to including a sub-bullet that specifies the frequency bands.

# 4 References

- R4-2305925, WF for system parameters of above 10 GHz NTN band, THALES