3GPP TSG-RAN WG4 # 107	R4-2307113

Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.5.3.1.2

Source:	MediaTek Inc.

Title:	Discussion on PDSCH requirements for 8Rx UE

Document for:	Discussion

- Introduction

In RAN4#106bis-e meeting, a WF related to 8Rx UE PDSCH requirements was agreed [1]. In this contribution, we provide simulation results and discuss outstanding issues.

- Discussion

For 8Rx UE PDCSH requirements, there are still some options about propagation channel and MCS after RAN4#106bis-e.

Issue 2-3: Propagation conditions for Rank 2 test - Option 1: TDLC300-100 ULA Medium B and TDLA30-10 ULA Medium B - Option 2: Only TDLC300-100 ULA Medium B Issue 2-4: MCS and antenna correlation for Rank 2 test - Proposals for TDLC300-100 ULA Medium B - Option 1: MCS 2, 7 (Table 2) - Option 2: MCS 19 (Table 1) - Proposals for TDLA30-10 ULA Medium B (if agreed) - Option 1: MCS 13, 26 (Table 2) - Option 2: MCS 19 (Table 1) Issue 2-5: MCS for Rank 4 test - Option 1: MCS 13 and MCS26 (Table 1) - Option 2: Only MCS17 (Table 1) Issue 2-7: Single MCS configuration for Rank 8 test - Option 1: MCS13 - Option 2: MCS17

We provide our simulation results in Table 1 based on the assumptions from Table 2 in Appendix.

Table 1. SNR point of 70% maximum throughput for rank 2, 4, 8

| Rank | MCS | Propagation Condition | Channel Correlation | SNR (dB) |
| --- | --- | --- | --- | --- |
| 2 | 19 (64 QAM, MCS Table 1) | TDLC 300-100 | ULA Medium B | 11.1 |
|  | 7 (16QAM, MCS Table 2) |  |  | 5.5 |
|  | 2 (QPSK, MCS Table 2) |  |  | -2.7 |
|  | 26 (256 QAM, MCS Table 2) | TDLA 30-10 |  | 26.8 |
|  | 19 (64 QAM, MCS Table 1) |  |  | 9.6 |
|  | 13 (64 QAM, MCS Table 2) |  |  | 10.8 |
| 4 | 26 (64 QAM, MCS Table 1) | ULA Low | TDLA 30-10 | 20.9 |
|  | 17 (64 QAM, MCS Table 1) |  |  | 10.3 |
|  | 13 (16 QAM, MCS Table 1) |  |  | 6.6 |
| 8 | 17 (64 QAM, MCS Table 1) | ULA Low | TDLA 30-10 | 21.2 |
|  | 13 (16 QAM, MCS Table 1) |  |  | 14.8 |

From simulation results, it can be shown that the SNR values to achieve 70% of maximum throughput for considered MCS are within a reasonable range. However, to reduce test effort, we prefer to pick one MCS for each rank configuration to define requirements for 8Rx UE.

Proposal 1: Introduce PDSCH requirements for 8Rx UE with the following MCS:

- MCS19 (rank2, ULA Medium B, TDLC 300-100)

- MCS17 (rank4, ULA low, TDLA 30-10)

- MCS13 (rank8, ULA low, TDLA 30-10)

- Conclusion

In this contribution, we provide simulation results for different MCS based on agreed simulation assumptions in the last meeting. According to simulation results, we have following proposals for MCS selection to define 8Rx UE PDSCH requirements.

Proposal 1: Introduce PDSCH requirements for 8Rx UE with the following MCS:

- MCS19 (rank2, ULA Medium B, TDLC 300-100)

- MCS17 (rank4, ULA low, TDLA 30-10)

- MCS13 (rank8, ULA low, TDLA 30-10)

- Reference

- R4-2305888, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon

- Appendix

Table 1. Simulation assumptions for PDSCH requirements

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U (FR1.30-1) |
| SCS(kHz)/Bandwidth (MHz) |  |  | 30/40 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 for rank 2 and 4 (@symbols#2,11) 2 for rank 8 (@symbols#2, 3, 10, 11) |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | for rank 2 and 4 l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 for rank 8 l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  | CSI-RS periodicity | Slots | 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4. |
|  | Frequency Occupation |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
| NZP CSI-RS for CSI acquisition | Row index |  | 3 for rank 2 5 for rank 4 6 for rank 8 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | Same as number of transmit antenna |
|  | CDM Type |  | 'FD-CDM2' |
|  | Density (ρ) |  | 1 |
|  | CSI-RS periodicity | Slots | 40 |
|  | CSI-RS offset |  | 0 |
|  | Frequency Occupation |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
| Number of HARQ Processes |  |  | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Tx EVM (Explicitly modeled in the simulation) |  |  | 64QAM:6% 256QAM:3% |