3GPP TSG-RAN WG4 # 107	R4-2307115

Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.5.3.1.4

Source:	MediaTek Inc.

Title:	Discussion on CQI requirements for 8Rx UE

Document for:	Discussion

- Introduction

In RAN4#106bis-e meeting, a WF related to 8Rx UE CQI requirements was agreed [1]. In this contribution, we provide our views about PMI configuration for 8Rx UE CQI test. Also, we propose SNR points for CQI test under static channels according to simulation results.

- Discussion

CodebookSubsetRestriction

For the legacy RAN4 CQI requirements, at most 2Tx is considered and 6 bits CodebookSubsetRestriction is used to restrict UE for reporting some specific PMI. For the newly introduced 8Rx UE CQI requirements, RAN4 agreed to use 4Tx. Therefore, according to TS38.214, both i1 and i2 need to be indicated for UE to restrict PMI reporting and RAN4 achieved the below agreements in RAN4#106.

- Antenna configuration: XP 4T8R - (N1,N2)=(2,1); (O1,O2)=(4,1) - ReportQuantity: CRI/Ri/i1/CQI - Two-one-TypeI-SinglePanel-Restriction: 0b00000001 - TypeI-SinglePanel-codebookSubsetRestriction-i2: 0x0000000000000001

However, according to TS38.306, UE is not mandatory to support the report quantity 'CRI/RI/i1/CQI '.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| semiOpenLoopCSI Indicates whether UE supports CSI reporting with report quantity set to 'CRI/RI/i1/CQI ' as defined in clause 5.2.1.4 of TS 38.214 [12]. | UE | No | No | Yes |

Therefore, in RAN4#106bis-e, RAN4 agreed to use legacy reportQuantity cri-RI-PMI-CQI instead of CRI/Ri/i1/CQI to avoid introducing additional UE capability in 8Rx UE CQI requirements. However, there were still three options for i2.

Report quantity configuration - Use the legacy configuration, report quantity 'cri-RI-PMI-CQI ' - Use two-one-TypeI-SinglePanel-Restriction = 00000001 - Following options for i2 will be discussed for next RAN4 meeting: - Option 1: TE sets random i2 (1 out of 2 possibilities) during the test - Option 2: TE uses fixed value i2 = 0 or 1 during the test - Option 3: Leave TE implementation to use random i2 or fixed i2 = 0 or 1

According to TS38.214 (Table5.2.2.2.1-8), i2 only impacts the phase of precoder. With two-one-TypeI-SinglePanel-Restriction = 00000001, the post SNR for both i2 = 0 and i2 = 1 will be the same and UE will report the same CQI correspondingly. However, to avoid the randomness behaviour in TE, we support Option 2 and propose to use fixed value i2 = 0 during the test.

Proposal 1: We support Option 2. RAN4 defines 8Rx CQI requirements with fixed value i2 = 0 during the test.

Simulation results

Our simulation result in Figure 1 is based on parameters in Table 1.

Table 1. Simulation assumptions for 8Rx UE CQI requirement

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | 7D1S2U S=6D+4G+4U |  |  |  |
| SNR |  |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  |  | static channel model with 4T8R |  |  |  |
| Antenna configuration |  |  |  | XP 4×8 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 in 38.101-4 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,(0) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | CodebookSubsetRestriction |  | two-one-TypeI-SinglePanel-Restriction = 00000001 Note: fixed i2 = 0 |  |  |  |
|  |  | RI Restriction |  | 00001000 |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-3, TBS.3-4 |  |  |  |

Figure 1. Simulation results for median CQI

In the last meeting, RAN4 discussed the SNR points for CQI requirements and there was no conclusion.

SNR points - Option 1: [4,5] dB and [10,11] dB - Option 2: [7,8] dB for 64QAM, [1,2] dB for 16QAM - Other options are not precluded

According to results in Figure 1, we propose to use “SNR [4,5] dB for 16QAM” and “SNR [10,11] dB for 64QAM” for 8Rx CQI testing.

Proposal 2: For 8Rx UE CQI requirements under static channels, RAN4 can use “SNR [4,5] dB for 16QAM” and “SNR [10,11] dB for 64QAM”.

- Conclusion

In this contribution, we provide our views about i2 configuration during the 8Rx UE CQI test. Also, we propose SNR points for CQI test under static channels based on our simulation results.

Proposal 1: We support Option 2. RAN4 defines 8Rx CQI requirements with fixed value i2 = 0 during the test.

Proposal 2: For 8Rx UE CQI requirements under static channels, RAN4 can use “SNR [4,5] dB for 16QAM” and “SNR [10,11] dB for 64QAM”.

- Reference

- R4-2305888, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon