3GPP TSG-RAN WG4 Meeting # 107		R4-2307273

Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.11.2.4

Source:			Qualcomm Incorporated

Title:				Impact of Option B-1-2

Document for:	Approval

# Introduction

In this paper we present our view on UE interruption requirements for Option B-1-2.

# Discussion

Interruption length

From the previous RAN4 #106bis e-meeting, we observed the following two flavours:

- 2 times RF retuning time plus SSB length, i.e. RF fully tuning away during L1 measurements/evaluations during based on SSB outside UE active BWP

- 2 times RF retuning time, i.e. wideband RF based L1 measurements/evaluations

Between the two options, the first option does not make much sense to us. And it does not fit into the original spirit of Option B-1-2 “Using larger BW covering SSB outside active BWP with interruptions.”

Proposal 1: For Option B-1-2, interruption length upon L1 measurement/evaluation based on SSB outside UE active BWP is defined as 2 times RF retuning time before and after the L1 measurement/evaluation, i.e. wideband RF based L1 measurement/evaluation.

Interruption ratio

It was observed that interruption ratio requirements proposed by companies were spread over quite a divergent range, e.g. from 0.5% to every L1 measurement/evaluation occasions. We do not think either of the two extreme requirements make much sense. For instance, the requirement of 0.5% would make Option B-1-2 more or less them same as Option B-1-1. And the other extreme interruption ratio requirement would make the feature similar to the original Option B-2-2, which is excluded from the scope of WI. If NW scheduling ambiguity is concerned, we believe introducing UE capability on the ratio of interruption would be one of the compromises. For example, if the capability is smaller than [X]%, depending on NW decision, NW may keep scheduling DL/UL to the UE.

Proposal 2: For Option B-1-2, interruption ratio upon L1 measurement/evaluation based on SSB outside UE active BWP is subject to UE capability. The capability can be from 0.5% to [5]%, and the granularity of the capability can be decided based on NW vendors’ inputs, e.g. 0.5, 1, 2, and 5. FFS on whether and how to differentiate L1 measurement/evaluation period, e.g. DRX on/off, DRX cycle, periodicity of SSB, etc.

# Conclusion

In this paper we proposed the following.

Interruption length

Proposal 1: For Option B-1-2, interruption length upon L1 measurement/evaluation based on SSB outside UE active BWP is defined as 2 times RF retuning time before and after the L1 measurement/evaluation, i.e. wideband RF based L1 measurement/evaluation.

Interruption ratio

Proposal 2: For Option B-1-2, interruption ratio upon L1 measurement/evaluation based on SSB outside UE active BWP is subject to UE capability. The capability can be from 0.5% to [5]%, and the granularity of the capability can be decided based on NW vendors’ inputs, e.g. 0.5, 1, 2, and 5. FFS on whether and how to differentiate L1 measurement/evaluation period, e.g. DRX on/off, DRX cycle, periodicity of SSB, etc.