3GPP TSG-RAN WG4 Meeting # 107                                                       R4- 2307389

# Incheon, KR, May 22 – May 26, 2023

Title: 	TP for TR 38.858:“Feasibility of FR1 Wide Area BS aspects”

Source: 	CATT

Agenda item:	8.20.2.2.1

Document for:	Approval

Background

In WF[1], The TR section “10.2 Feasibility of FR1 Wide Area BS aspects” has been further broken-down, Template for capturing co-site inter-sector co-channel interference impact has been provided in WF[2]. This TP provides our company’s analysis.

Reference

[1] R4-2305916, “WF on implementation feasibility of SBFD”, RAN4#106bis-e, Samsung

[2] R4-2305917, “WF on implementation feasibility of SBFD: Co-channel Co-site/inter-site interference”, RAN4#106bis-e, Ericsson

TP for TR 38.858

## 10.2 Feasibility of FR1 Wide Area BS aspects

### 10.2.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

### 10.2.1.1	Summary table for self-interference analysis

Editor's note: This section captures the summary table which is based on self-interference analysis framework.

### 10.2.1.2	Feasibility study on self-interference

SI feasibility study for Wide area BS can be made as following.

Table 10.2.1.2-1: SI feasibility study for Wide Area BS

| FR1 |  |  |  | Calculation formula | CATT |
| --- | --- | --- | --- | --- | --- |
| BS class |  |  |  |  | WA |
| BS TX Power dB(P_TX ) = ① dBm |  |  |  |  | 49 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dB(α_(SI-frequency,TX) ) = ② dBc |  |  | 45 |
|  |  | Frequency isolation techniques used |  |  | CFR、DPD |
|  |  | Spatial isolation capability dB(α_(SI-spatial) ) = ③ dBc |  |  | 70 |
|  |  | Spatial isolation techniques used |  |  | TX/RX panel separation、 isolation structures、isolation material、cross polarization |
|  | TX Beam nulling /isolation in TX sub-band dB(α_(SI-beam) ) = ④ dBc |  |  |  | 10 |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBm(P_(SI_leakage) ) (Note 1) |  |  | ①-②- ③- ⑨-⑦-10*log10(40/20) | -89.01 |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band sub-band dB(α_(SI-RFIC) ) = ⑤ dBc |  |  | 0 |
|  |  | RF IC capability and other tech. in RX sub-band sub-band dB(α_(SI-RFIC) ) = ⑧ dBc |  |  | 0 |
|  |  | RF IC techniques and other tech.(before LNA) |  |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  |  | 0 |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBm(P_(SI_TXSB) ) (Note 1) |  |  | ①-③-④ | -31 Rx is blocked |
|  | Blocker Suppression at RX | Frequency isolation capability dB(α_(CSSI-frequency,RX) )=⑥dBc |  |  | 65 |
|  |  | Frequency isolation techniques |  |  | Digital filter |
|  |  | RX IMD | Rx IIP3 capability (dBm) |  | -10 |
|  |  |  | Rx IM3 contribution (dBm) | (①-③-④)-2*(IIP3-(①-③-④)) | -73 |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  | -116 |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalizeddB(P_(SI_Selectivity) )(Note 1, 2) |  |  | ①-③-④-⑥ | -96 |
|  | RX Beam nulling /isolation in RX sub-band dB(α_(SI-beam) ) = ⑨ dBc |  |  |  | 10 |
|  | RX sensitivity degradation caused by RX beam nulling |  |  |  |  |
|  | Digital IC dB(α_(SI-digtial) ) = ⑦ dBc |  |  |  | 10 |
| Overall RSIC capability dB(RSIC) (Note 1) |  |  |  | ①-(P_(SI_leakage)+Rx IM3+Other RX +P_(SI_Selectivity)) Calculate with power linear value | 121.86 |
| Noise floor ⑩dBm |  |  |  | -174+10*LOG10(20*10^6)+NF | -95.99 |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  |  | -101.99 |
| Required RSIC budget (①-⑪dBc) |  |  |  |  | 150.99 |
| SBFD configuration |  |  |  |  | 40-20-40 |
| Guardband assumption (if exist) |  |  |  |  |  |
| bandwidth over which suppression is achieved |  |  |  |  |  |
| Others |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. |  |  |  |  |  |
| Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. |  |  |  |  |  |
| Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |

10.2.1.2.1	CATT analysis

For FR1 Wide Area BS SI analysis, the following assumptions are used.

Table 10.2.1.2.1-1: SI analysis assumptions for Wide area BS

| Parameters | Wide Area BS |
| --- | --- |
| Reference sensitivity level degradation due to SI | 1dB |
| Channel bandwidth | 100MHz |
| Subband configuration | {DUD} |
| DL subband width | 40MHz*2 |
| UL subband width | 20MHz |
| Tx output power over whole channel | 49dBm |
| Adjacent subband Leakage Power Ratio | 45dB |
| Noise Figure | 5dB |

In SI capability analysis, the following techniques are used,

- PAR of wanted signal is high, so CFR is used to improve equipment efficiency, DPD is used for high power equipment to optimize ACLR.

- Tx antennas panel and Rx antennas panel are separate, there are also some isolation material between them, and cross polarization is also used.

- Rx signals include wanted signal and interference signals. Interference signals far from Rx operating frequency range can be rejected by Rx RF and IF filters, but DL-subband and UL-subband are adjacent, RF filters except metal cavity filter are difficult to reject the adjacent interference signals, at the same time, the filter also needs to be changed when subband configuration changes. Tx RF filters have the same performance as Rx RF filter.

- Digital filter is used to resolve the adjacent subband( i.e. TX subband )interference issue.

- Beam nulling is used to improve isolation between Tx and RX.

From the analysis in Table 10.2.1.2-1, the receiver may be blocked if the spatial isolation is not so high. Some specific techniques need to be used to improve the spatial ISO. Requirement of IIP3 is relatively high and the ACS requirement is improved largely compared with the legacy BS. It seems wide area SBFD BS design may need much improvement from several aspects to make it feasible.