3GPP TSG-RAN WG4 Meeting #107				              	         R4-2307607

Incheon, Korea, May 22 – 26, 2023

Title:	[draft] Reply LS on time gap between a PDCCH order and the corresponding PRACH transmission for LTM

Response to:	R1-2304276

Release:	Release - 18

Work Item:	NR_Mob_enh2-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

Name:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN4 thanks RAN1 for the LS R1-2304276. As requested, RAN4 discussed how to update the time gap between a PDCCH order and the corresponding PRACH transmission for LTM in this meeting and would like to provide the following feedback:

- Delay requirements

- As requested, RAN4 would provide required changes from RAN4’s point to RAN1. Final delay requirements will be captured in RAN1 spec as legacy.

- Additional time for DL synchronization should be added in the legacy time gap specified for PDCCH ordered RACH transmission on serving cell in RAN1. The value is TRS+ Tmargin, where TRS is the SSB/SMTC periodicity of the target NR cell for gap-less cases and max{SSB/SMTC periodicity, MGRP} for gap-assisted case, Tmargin=2ms. Whether to use SSB periodicity or SMTC periodicity depends on whether L1-RSRP measurement is configured or not. The detail scenarios for gap-less and gap-assisted cases will be defined in RAN4 spec

- RAN4 will further discuss whether additional time for RACH configuration decoding after RAN2 concludes on how NW configures the RACH configuration of the neighbour cell(s).

- Additional time for UL timing adjustment should be added in the legacy time gap specified for PDCCH ordered RACH transmission on serving cell in RAN1. FFS: The value.

- If RO of the target cell is not in the active UL BWP, RF retuning time is needed. It is feasible to reuse the legacy values, i.e., 0.5ms for FR1 and 0.25ms for FR2.

- BWP switching delay ( ∆BWPSwitching ) is not needed from RAN4’s point of view.

- For the other three components (NT,2 and Tswitch), as they are defined in RAN1, RAN4 would rely on RAN1 to check the need for any update.

- Scheduling restriction

- Scheduling restriction due to DL synchronization after reception of PDCCH order is needed.

- When RO of the target cell is not in the active UL BWP, there will be interruption on both UL and DL of all the serving cells before and after RACH transmission due to RF retuning, i.e., 0.5ms of FR1 and 0.25ms for FR2.

- UL timing adjustment will cause interruption on UL of all the serving cells.

- During the RACH transmission to neighbour cell, there will be scheduling restriction for both UL and DL of all the serving cells on the symbols overlapping RO symbols and one more symbol before & after RO symbols.

2. Actions:

To RAN2 group.

ACTION: 	RAN4 kindly asks RAN1 to take above RAN4 agreements into consideration.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #108		              21 – 25 August 2023			Toulouse, France

TSG RAN WG4 Meeting #108-bis		              9 – 13 October 2023			Xiamen, China