# 3GPP TSG-RAN4 Meeting #107	R4-2307629

# Incheon, Korea, 22th– 26th May 2023

Title:	[draft] LS on the network assistant signalling for the advanced receiver for MU-MIMO

Release:	Rel-18

Work Item:	NR_demod_enh3

Source:	RAN WG4

To:	[RAN WG1, RAN WG2]

Cc:

Contact Person:

Name:	Jingzhou Wu

E-mail Address:	wujingzhou@chinatelecom.com

Attachments:

1. Overall Description:

Within the Release 18 work item on NR demodulation performance evolution (NR_demod_enh3), RAN4 has discussed the network assistant signalling for advanced receiver to cancel inter-user interference for MU-MIMO scenario.

As an outcome, following agreements are reached by RAN4:

- For the target UE to perform interference cancellation to the co-scheduled UE under MU-MIMO, RAN4 has agreed that the target UE should have the following default network configuration assumptions for the co-scheduled UE for MU-MIMO transmission:

- The scramblingID0, scramblingID1 and nSCID as defined in 7.4.1.1.1 of TS38.211, is same as that of the target UE.

- The PRB bundling size of co-scheduled UE(s) is same as that of target UE.

- The ‘ratio of PDSCH EPRE to DM-RS EPRE’ as defined in 4.1 of TS38.214 of the co-scheduled UE(s) should be the same as that of the target UE.

- The same PDSCH symbols are allocated to the target and the co-scheduled UE(s).

- The target PDSCH is not overlapped with the CSI-RS of the co-scheduled UE(s).

- Meanwhile, new [RRC and/or DCI] based network assistant signalling is agreed to be introduced to assist the target UE to perform interference cancellation to the co-scheduled UE in Rel-18.

- Regarding the content of the Rel-18 new [DCI-based] network assistant signalling, RAN4 reach following agreements:

- TBA

- Regarding the content of the Rel-18 new [RRC-based] network assistant signalling, RAN4 reach following agreements:

- TBA

2.

To RAN WG1 group.

ACTION: RAN4 respectfully request RAN1 to take the above information into account and design the corresponding [DCI-based] network assistance signalling for UE advanced receiver to cancel inter-user interference for MU-MIMO scenario.

To RAN WG2 group.

ACTION: RAN4 respectfully request RAN2 to take the above information into account and design the corresponding [RRC-based] network assistance signalling for UE advanced receiver to cancel inter-user interference for MU-MIMO scenario.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #108 	21 Aug - 25 Aug 2023

TSG-RAN WG4 Meeting #108-bis 	09 Oct - 13 Oct 2023