3GPP TSG RAN WG4 Meeting #107	R4-2307908

Incheon, Republic of Korea, May 22 – May 26, 2023

Source: 	Ericsson

Title:  	UL timing adjustment solutions

Agenda Item:	8.13.4.3

Document for:	Discussion

# Introduction

WID on enhanced NR support for high-speed train scenario in frequency range 2 (FR2) [1] is introduced in Rel-18 release. In last meeting, issues on UL timing adjustment are discussed and the agreements are captured in [2].

In this contribution, we present our viewpoints and proposals on the issues listed in WF.

# Discussion

## Sub-topic#1-1: MAC-CE based solution for cross-RRH TCI state switch

Issue 1-1-2: Information indicated in MAC-CE

Agreement: - Introduce 1-bit TCI State Indication in UE-specific PDCCH MAC CE for whether or not UE shall follow the Rel-17 UL timing solution for the indicated TCI state ID. - FFS in RAN4, UE behaviour after receiving the 1-bit indication.

In last meeting, we only agreed on TCI state indication but the exact TCI state switch command which 1-bit indication can apply to was not discussed.

Proposal 1: 1-bit indication also shall be applicable to enhanced TCI States Activation/Deactivation.

Proposal 2: In enhanced TCI States Activation/Deactivation, a straightforward solution is that each TCI state ID shall be associated with one 1-bit indication.

Issue 1-1-3: Impact of signalling on RACH-based timing adjustment procedure

Way forward: Open issue needs further discussion - Option 1: Agreed signalling can be used to trigger RACH procedure also if RACH procedure is used for timing adjustment at cross-RRH TCI state switch. - Option 2: Do not take signalling into account for triggering RACH procedure

PDCCH order can initiate RACH procedure was mentioned at a prior meeting, but no specifics emerged with more details. The consequence is RACH initiation may happen at any time from UE perspective. on the contrary, initiating RACH with a 1-bit signal can result in stable/clear rules on employing the RACH method.

Proposal 3: The 1-bit signaling can be used to trigger RACH procedure if RACH procedure is used in one-short timing adjustment.

## Sub-topic #1-2: UL TX timing adjustment at UL spatial relation switch

Issue 1-2-1: A need for timing adjustment at UL spatial relation switch

Way forward: - FFS the scenario in which UL timing exceed allowed timing adjustment range at UL spatial relation: - Option 1: Apply existing one-shot larger UL timing adjustment mechanism (Clause 7.1.2.3) at UL spatial relation switch - Option 2: UL spatial relation shall always be executed strictly when corresponding DL TCI state switches - Option 3: The existing gradual timing adjustment requirements can be applied, and there is no need to define additional UL transmit timing adjustment.

As per discussions in last meeting, we lean towards that timing change at switching UL spatial relation could be an issue to be addressed. Same UL timing change may happen when updating UL spatial relation or DL TCI state, we don’t see difference.

Proposal 4: We agree on Option 1. If Option 1 is concerned with complexity, we favor Option 2 as a substitute.

## Sub-topic #1-3: General

Issue 1-3-1: Impact of large propagation delay jump on timeAlignemntTimer

Way Forward: - FFS whether to ask for clarification from RAN2 on the following questions: - Does the procedure of maintaining UL time alignment consider TCI state switch between non-collocated RRHs? - What is the understanding of “UL time aligned” in the definition of timeAlignmentTimer? - FFS Potential impacts of large jump in propagation delay on UE MAC timeAlignmentTimer - Option 1: In HST FR2 scenarios, UE MAC timeAlignmentTimer should be stopped or suspended after inter-RRH TCI state switch - FFS applicability of such timer behaviour to R17 one-short large timing adjustment or to through RACH-based method - Option 2: Not to introduce timeAlignemntTimer enhancements at UL timing adjustment

If one-short large timing adjustment is adopted by UE, the problematic is UL timing in a unlimited time period is out of normal requirements in the following:

#### 7.1.2.3 One shot large UL timing adjustment for FR2 Power Class 6 UE When highSpeedMeasFlagFR2-r17 is configured and highSpeedLargeOneStepUL-TimingFR2-r17 is enabled for UE supporting FR2 power class 6 and [largeOneStepUL-timingFR2-r17] capability, the following requirements apply to the UE: - If the absolute value Told-Tnew≤[CP/4], the requirement in clause 7.1.2.1 apply to the first UL transmission after a TCI state switch. - Otherwise, the UE transmit timing immediately after TCI state switch shall be Tnew-(NTA+NTA offset)+2 (Told-Tnew) and clause 7.1.2.1 requirements don’t apply. - The UE UL transmission timing error after the TCI state switching procedure shall be less than or equal to ±Te as specified in clause 7.1.2 if the new target TCI state is within active TCI state list, otherwise ±[7Ts], and the reference point is Tnew-(NTA+NTA offset)+2 (Told-Tnew). Above, - Tnew (in Tc units) is the DL timing defined as the time when UE receives downlink frame with new target TCI state. - Told (in Tc units) is the DL timing defined as the time when UE receives downlink frame with old source TCI state.

In this unknown period, the maximal UL transmission timing error is ±[7Ts] which is close to CP/2 with margin. Depending on implementation of gNB’s reception, it can’t guarantee to validate correct UL reception. Give that, our concern is: if UL transmission can’t be ensured, whether the status of timeAlignmentTimer shall be updated also?

For RACH-based method, the RACH procedure is covered in legacy procedure of maintaining timeAlignmentTimer, i.e. timeAlignmentTimer shall be stopped once Timing Advance Command is received in a Random Access Response message.

Observation 1:  For one-short large timing adjustment, UL transmission timing error is ±[7Ts] and UL transmission can’t be ensured, timeAlignmentTimer might be impacted.

Observation 2: For RACH-based method, legacy procedure of maintaining timeAlignmentTimer might be enough.

Issue 1-3-2: Applicability of gradual timing adjustment in between one-shot large timing adjustments

Way forward: - FFS the applicability of gradual timing adjustment requirement (Clause 7.1.2.1) in between the one-shot large UL timing adjustments for FR2 Power Class 6 UE. - Option 1: UE to report the value of one-shot large UL timing adjustment back to the network. - Option 2: Follow the current UE autonomous timing adjustment procedure and requirements. - Option 3: Describe UE behavior after one shot UL timing adjustment in the TR.

If network can acquire the practical UL timing adjustment value, network can adopt operations optimally, e.g. sending TA command to avoid residual UL timing adjustment or arranging scheduling restriction when necessary.

Proposal 5: We’re leaning towards Option 1 to facilitate network’s further operation if UE behavior in Option 3 cannot be regulated.

# Conclusion

Observation 1:  For one-short large timing adjustment, UL transmission timing error is ±[7Ts] and UL transmission can’t be ensured, timeAlignmentTimer might be impacted.

Observation 2: For RACH-based method, legacy procedure of maintaining timeAlignmentTimer might be enough.

Proposal 1: 1-bit indication also shall be applicable to enhanced TCI States Activation/Deactivation.

Proposal 2: In enhanced TCI States Activation/Deactivation, a straightforward solution is that each TCI state ID shall be associated with one 1-bit indication.

Proposal 3: The 1-bit signaling can be used to trigger RACH procedure if RACH procedure is used in one-short timing adjustment.

Proposal 4: We agree on Option 1. If Option 1 is concerned with complexity, we favor Option 2 as a substitute.

Proposal 5: We’re leaning towards Option 1 to facilitate network’s further operation if UE behavior in Option 3 cannot be regulated.

# References

- RP-222272 ‘New WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2)’, Samsung

- R4-2306398 ‘WF on NR FR2 HST UL Timing Adjustment Solutions and Tunnel Deployment’, Nokia, Nokia Shanghai Bell