3GPP TSG-RAN WG4 Meeting #107 	 R4-2308105

Incheon, KR, May 22 – May 26, 2023

Title:	Net gain analysis and next steps for MPR/PAR - objective

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	8.28.2.1

Document for:	Approval

# 1	Introduction

This contribution relates to a work item agreed in RAN#94-e, namely “Further NR coverage enhancements” [1]. We consider power domain enhancements and the following objectives captured in the WID:

- Study and if necessary specify following power domain enhancements

  - […]

  - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

In this paper we discuss net gain results and next steps for MPR/PAR reduction in Rel-18.

# 2	Discussion

The following was agreed in RAN1 #110bis-e.

Agreement

The following work split principles will be adopted in RAN1 for power domain enhancement throughout Rel-18 from RAN1 perspective and send LS to RAN4 in this meeting:

- RAN1 performs link level simulations of candidate solutions for power domain enhancements to study at least the SNR variation, PAPR/CM, and EVM, brought by each solution.

  - Transparent MPR/PAR reduction solutions can be considered as a benchmark for studying the performance of non-transparent solutions.

- RAN1 is not expected to perform RF simulations of candidate solutions for power domain enhancements

  - Results of RF simulations can be included in RAN1 contributions

- RAN1 will assess RAN1 specification impact of candidate MPR/PAR reduction solutions

  - A list of candidate solutions, including necessary parameters, from RAN1 perspective should be ready before the end of RAN1 #111, and should be included in an LS to RAN4.

- RAN1 understands that RAN4 is responsible for selecting the Rel-18 MPR/PAR reduction solution, if any.

The following was agreed in RAN4 #104bis-e.

Agreement:

- RAN4 follows below RAN1 agreements and focus on prepare for RF simulations

  - Establish evaluation parameters and side-conditions if any for both transparent and non-transparent schemes

    - The parameters and side-conditions will be updated if needed according to RAN1 input

    - Share the agreements with RAN1 that could affect RAN1 link level simulation

  - RAN4 can perform evaluations without RAN1 input for both transparent and non-transparent schemes

    - No discussion on simulation results of non-transparent scheme at least in RAN4#105

Agreement:

Actual conclusion of the MPR/PAR reduction methods should be based on net coverage gain results combining transmitter and receiver performance

Finally, the following was agreed in RAN4 #106bis-e.

Agreement:

- RAN4 agrees to analyse provided simulation results as attached to this WF via Excel and provide conclusions based on these at RAN4#107.

  - It is expected a decision on selecting the Rel-18 MPR/PAR reduction solution(s) according to the WID can be taken at RAN4#107

There has been considerable progress in defining the evaluation assumptions in both RAN1 and RAN4 [6]. On top of that both RAN1 and RAN4 send LSs summarizing the key parameters [8, 9]. Based on those, the assumptions behind LLS (RAN1) and RF simulations (RAN4) seem to be well aligned.  RAN1 has sent also a LS [11] with link level results to RAN4 to support RAN4 decision of MPR/PAR reduction solutions for Rel-18 power domain enhancements.

Observation 1: Based on the progress in both RAN1 and RAN4, the simulation parameters are well aligned between RAN1 and RAN4

Observation 2: RAN4 is ready for selecting the Rel-18 MPR/PAR reduction solution(s)

# 3	Priority scenarios for MPR/PAR reduction

As shown below, there was considerable progress in RAN4 #106 to define priority scenarios for MPR/PAR reduction. Based on that RAN4 seems to have common understanding on the priority scenarios for MPR/PAR reduction.

Agreement:

- QPSK is the targeted modulation for further coverage enhancements

  - At least for simulation study

- RAN4 shall prioritizes DFT-S-OFDM as a solution for coverage enhancements

  - FSS on CP-OFDM if companies can show gains

- RAN4 shall evaluate both FR1 and FR2 scenarios.

- RAN4 shall not consider other channels and signals (than PUSCH and the associated DMRS)

- RAN4 shall not consider intra band UL CA scenario in Rel-18 WI

MPR/PAR reduction schemes:

The following was agreed in RAN4 #104bis-e and RAN1 #110bis-e.

Agreement: (RAN4 #104bis-e)

- Frequency domain spectrum shaping without spectrum extension for DFT-S-OFDM is the transparent scheme thus far according to the WID

- Other techniques can be discussed depending on RAN Plenary decision

Agreement: (RAN1 #110bis-e)

At least the following candidate solutions for MPR/PAR reduction will be studied in RAN1.

- Frequency domain spectrum shaping w/ spectrum extension

- Frequency domain spectrum shaping w/o spectrum extension

- Tone reservation (which can only be w/ spectrum extension)

There seem to be a common ground between RAN1 and RAN4 on the MPR/PAR reduction schemes - the priority is with the schemes covered by WID. This would mean that we have the following schemes on table:

- Reference case (legacy) without FDSS and without spectrum extension

- FDSS without spectrum extension (transparent scheme)

- FDSS with spectrum extension (non-transparent scheme)

- Tone reservation (non-transparent scheme)

Observation 3:  According to WID and agreements made until now, there are four MPR/PAR reduction schemes on table:

- Reference case (legacy) without FDSS and without spectrum extension

- FDSS without spectrum extension (transparent scheme)

- FDSS with spectrum extension (non-transparent scheme)

- Tone reservation (non-transparent scheme)

# 4.	Net gain analysis

The following agreement was made in RAN4 #106bis-e.

Agreement:

- Use the following equation as the baseline

- Net Gain [dB]=(Pout_(Filter i)-Pout_Ref )+(SNR10%_Ref-SNR10%_(Filter i) )

- Where Pout_(Filter i) is the achieved (TX) output power of the filtered  waveform being compared against the reference, Pout_Ref is the output power of the reference, SNR10%_Ref is the required SNR to achieve 10% BLER with the reference, and SNR10%_(Filter i) is the required SNR to achieve 10% BLER using the filtered waveform being compared against the reference.

- Use the following equation when providing the results for net gain

- Net Gain [dB]=-DOPR_M0 +(SNR10%_Ref-SNR10%_(Filter i) )

In the following we analyse the net gain using the agreed equation(s).

## 4.1 Analysis for Results in RAN4 Excel

The set of configurations defining MCS/PRB allocations for simulations was made in RAN1 #111 and the configurations are shown in Table 1. These parameters ensure that such MCS/PRB combinations yield the same spectral efficiency, i.e., the results are comparable.

Table 1 Set of configurations for simulations

|  |  | No spectrum extension |  | With spectrum extension |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TBS value | Tput estimation for DDDSU @4GHz | #PRBs | MCS | #PRBs before extension | #PRBs after extension | MCS | Spectrum extension factor |
| 2408 | 963.2 kbps | 16 | 7 | 14 | 16 | 8 | 1/8 |
| 5376 | ~2.15 Mbps | 32 | 8 | 28 | 32 | 9 | 1/8 |
| 272 | 108.8 kbps | 8 | 0 | 6 | 8 | 1 | ¼ |
| 1032 | 412.8 kbps | 8 | 6 | 6 | 8 | 8 | ¼ |
| 2152 | ~0.9 Mbps | 40 | 2 | 30 | 40 | 3 | ¼ |
| 4992 | ~2.0 Mbps | 40 | 6 | 30 | 40 | 8 | ¼ |
| 552 | 220.8 kbps | 16 | 0 | 10 | 16 | 2 | 3/8 |
| 1736 | 694.6 kbps | 32 | 2 | 20 | 32 | 4 | 3/8 |
| 432 | 172.8 kbps | 8 | 2 | 6 | 8 | 3 | ¼ |
| 808 | 323.2 kbps | 24 | 0 | 18 | 24 | 1 | ¼ |

Table 2 and Table 3 summarizes the net gain for Nokia results provided in the Results Excel [12]. They illustrate the maximum net gain among the four tested filters (#1-4). Table 2 represents 700 MHz rural scenario, and Table 3 4GHz urban scenario, respectively.

Table 2 Net gain results for FDSS (transparent) and FDSS-SE (non-transparent) for scenarios shown in Table 1, 700MHz rural, QPSK modulation, DFT-s-OFDM.

Table 3 Net gain results for FDSS (transparent) and FDSS-SE (non-transparent) for scenarios shown in Table 1, 4GHz urban, QPSK modulation, DFT-s-OFDM. Net gain simulated also for the additional case with extension factor 1/9 (as per discussion in RAN4 #106bis-e).

The following observations can be made from Table 2 and Table 3:

- FDSS-SE provides considerable net gain (compared to “no filter” -case)

  - The gain is up-to 1.3 dB for outer allocations (extension factor 0.25)

  - The gain is up-to 1.0 dB for inner allocations (extension factor 0.25)

  - The net gain is biggest in the following cases:

    - FDSS-SE

    - MCS “0 and 1”, “2 and 3”

  - The net gain performance between 700MHz and 4GHz is quite similar

  - The net gain with extension factor 0.375 is between 0.6 and 1.3 dB

  - The net gain with extension factor 0.125 is up-to 0.4 dB (it’s worth of noting that coding rate is very high in these cases). Results with extension factor 0.111 are inline with extension factor 0.125.

- Net gain for transparent scheme (FDSS) is smaller compared to FDSS-SE. The biggest difference happens for inner allocations, where net gain for FDSS is close to zero.

## 4.2 Analysis for results for code-rate based scenarios

While MCS-based method (discussed in Section 4.1) is one way to define TBS for each PRB allocation, another way being the code rate -based method used in [2]. LLS comparison between the methods is shown in Figure 1 below as SNR vs. baseline case code rate for both methods. As can be seen the result is the same for both methods. Code rate -based results in [2] provide a broader view of net gain performance of the MPR reduction methods and results with MCS-based method discussed in Section 4.1 can be considered as a subset.

Figure 1. Code rate vs. MCS based method.

The net gain analysis in [2] covers all possible RB start values in the following scenarios:

- 700 MHz rural, 20 MHz, FR1:

    - Subcarrier spacing: 15 kHz

    - 4 allocation sizes: [16, 32, 64, 96] RBs

    - 6 code rates wo extension (1/16, 1/8, 1/6, ¼, 1/3, ½)

    - 3 extension sizes: [0.125, 0.25, 0.375]

      - 72 scenarios

- 4GHz urban, 100 MHz, FR1:

    - Subcarrier spacing: 30 kHz

    - 5 allocation sizes: [16, 32, 64, 128, 256] RBs

    - 6 code rates wo extension (1/16, 1/8, 1/6, ¼, 1/3, ½)

    - 3 extension sizes: [0.125, 0.25, 0.375]

      - 90 scenarios

- 28 GHz, 400 MHz, FR2:

    - Subcarrier spacing: 120 kHz

    - 5 allocation sizes: [16, 32, 64, 128, 256] RBs

    - 6 code rates wo extension (1/16, 1/8, 1/6, ¼, 1/3, ½)

    - 3 extension sizes: [0.125, 0.25, 0.375]

      - 90 scenarios

Thus, the total number of scenarios studied in [2] is 252 (on top of 10 scenarios shown in Table 1).  The net gain results are summarized in [2]/Section 3.5 in the following way:

- Net gain from non-transparent scheme varies according to Tx filter, extension size, code rate, and RB allocation.

  - Considerable net gains are observed for all RB allocations. For inner allocations, the gains can be more than 1.5 dB. For outer allocations, the gains can be more than 2 dB.

  - Net gain is biggest for the largest RB allocations. However, net gain around 1 dB is available also for the smallest RB allocation (16RBs) considered in simulation scenarios.

  - Net gain reduces with when the code rate increases. However, positive net gain is available even for the highest code rates considered in the simulation scenarios.

  - 25% extension maximizes the net gain in most of the simulation scenarios.

  - Trend: Less aggressive filters (such as TRRC) maximize the net gain for small/inner allocations, while more aggressive filters (such as [0.335 1 0.335]) maximize the net gain for large/outer allocations.

  - 20 MHz CBW and 100 MHz CBW provide similar net gains.

  - Similar net gains are available for both FR1 and FR2.

- Net gain from transparent scheme is typically 1-1.5 dB smaller compared to that of non-transparent schemes. For the largest allocations, the difference is up-to 2 dB. In many cases (esp. with FR2) the net gain from transparent schemes is close to zero.

Based on the net gain results, we make the following proposal:

Proposal 1: Support FDSS with spectrum extension as a solution for MPR/PAR reduction in Rel-18.

# 5.	Conclusion

In this paper we discuss the scope of MPR/PAR reduction in Rel-18. Based on the discussion we make the following observations and a proposal:

Observation 1: Based on the progress in both RAN1 and RAN4, the simulation parameters are well aligned between RAN1 and RAN4

Observation 2: RAN4 is ready for selecting the Rel-18 MPR/PAR reduction solution(s)

Observation 3:  According to WID and agreements made until now, there are four MPR/PAR reduction schemes on table:

- Reference case (legacy) without FDSS and without spectrum extension

- FDSS without spectrum extension (transparent scheme)

- FDSS with spectrum extension (non-transparent scheme)

- Tone reservation (non-transparent scheme)

Proposal 1: Support FDSS with spectrum extension as a solution for MPR/PAR reduction in Rel-18.

# References

- RP-213579, “New WI: Further NR coverage enhancements”, China Telecom

- R4-2308108, “RF simulation results for non-transparent schemes”, Nokia, Nokia Shanghai Bell

- R4-2217745, “WF on enhancements to reduce MPR/PAR”, Nokia, Nokia Shanghai Bell

- R1-2210326, “Final FL summary of power domain enhancements (AI 9.14.2)”, Moderator (Nokia, Nokia Shanghai Bell)

- R1-2008703, “Discussion on approaches and solutions for NR PUSCH coverage enhancement”, Nokia, Nokia Shanghai Bell

- R4-2308106, “RF simulation parameters for MPR/PAR evaluations”, Nokia, Nokia Shanghai Bell

- R4-2308107, “RF simulation results for transparent schemes”, Nokia, Nokia Shanghai Bell

- R4-222084, “LS on RF simulation parameters to be used in RAN4 for power domain enhancements”

- R1-2212917, “LS to RAN4 for further information on RAN1 assumptions for LLS performance evaluation of MPR/PAR reduction solutions”

- R1-2301655, “RAN1 impacts for power domain enhancements”, Nokia, Nokia Shanghai Bell

- R1-2302081, “LS to RAN4 for results of the LLS performance evaluation of MPR/PAR reduction solutions”

- R4-2308110, “Collected Simulation Results f or MPR/PAR evaluations”, Nokia, Nokia Shanghai Bell

2-tap filter [1 0.28], 3-tap filter [0.335 1 0.335], 3-tap filter [0.28 1 0.28], TRRX [0.5 0.1667]