3GPP TSG-RAN4 Meeting #107												R4-2308204

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Agenda item:	8.29.1.1

Source: 	CMCC

Title: 	Discussion on NCR MT feature list

Document for:	Discussion

# 1. Introduction

In last RAN4 meeting, a WF is approved [1]. In this meeting, we focus on the discussion of NCR RF feature list.

The agreement for NCR feature list is listed as below:

- Further discuss NCR-MT feature list, FFS whether feature list required or not for NCR-fwd from RAN4 perspective - to focus on the mandatory UE capability firstly and to check whether this is still applicable for NCR-MT and identify some features which are not applicable for NCR-MT; [for the rest of UE features, it should be optional for NCR-MT].

# 2. Discussion

## 2.1 R15 feature list

Following table list all the feature in TR 38.822 for R15 RF part. We have following observations:

Observation 1: summary of all R15 UE features:

- Following are mandatory feature without capability signaling

- 64 QAM, 256 QAM and pi/2-BPSK related, i.e. index 1-2, 1-3, 1-4, 1-6, 1-7

- 7.5kHz UL raster shift, i.e. index 1-11

- Max CBW, i.e. index 2-1

- Band combination related, i.e.

  - index 2-2 Simultaneous reception or transmission with same or different numerologies in CA,

  - index 2-3 Non-contiguous intra-band CA frequency separation class for FR2,

  - 2-4, 2-5, 2-9 Simultaneous reception and transmission,

  - 2-16, 2-17 PA architectures

- UE power class, i.e. index 2-8

- Multiple frequency band indication, i.e. index 2-10

- Multiple NS/P-Max, i.e. index 2-12

Before we have conclusion of all features for NCR-MT, we should find out following issues:

- whether 256QAM, 64QAM and pi/2 BPSK are mandatory supported or not and whether related RF requirements are needed or not

- for 7.5kHz raster shift, this feature is applicable for the band that is re-farmed from LTE or even 2G era. RF repeater has been deployed from 2G era in our commercial network, so the re-farmed spectrum will also be used by NCR, therefore, this feature is suggested to be mandatory for NCR-MT

- for max CBW, considering NCR is fixed, it is not mandatory to support all CBW and it can only support the CBW that are used by commercial network. so this feature is suggested as optional;

- for band combination related feature, at first we should find out whether NCR-MT should support band combination including CA, EN-DC, SUL in this WID. If only single CC is supported for NCR-MT, we can ignore all these band-combination related feature. We are open on this issue.

- UE power class, this wait for the conclusion of MT UL power class, if we reuse the same agreements as RF repeater UL, power class feature is not needed because UE max output power will depend on declaration and output power granularity would be different from 3dB. but for MT part since it plays the same role as UE it seems the same power class as normal UE is also reasonable.

- Multiple frequency band indication, this feature is also suggested as mandatory for NCR-MT since it should support all operator’s operation bands.

- Multiple NS/P-max, this also needs to wait for the NS signaling discussion. From our perspective, such feature should be mandatory.

Based on above analysis, we have following proposals and observations:

Proposal 1: it’s suggested to discuss whether 256 QAM, 64 QAM and pi/2 BPSK modulation scheme are mandatory or not for NCR-MT before concluding related feature. Our suggestion is as below

  - 64 QAM mandatory for both FR1 and FR2 UL and DL

  - 256QAM mandatory for FR1 DL, optional for FR1 UL and FR2 UL and DL

  - pi/2 BPSK optional for FR1 and mandatory for FR2

  - Power boosting for Pi/2 BPSK NCR-MT is optional, the same as UE

proposal 2: 7.5kHz raster shift, multiple frequency band indication and Multiple NS/P-max are mandatory for NCR-MT

proposal 3: max CBW is optional for NCR-MT

proposal 4: it’s suggested to figure out whether NCR-MT could support band combinations or only support single carrier in this release. If NCR-MT support band-combinations, the same feature as UE except for max duty cycle related feature which need further check

proposal 5: UE power class feature wait for conclusion of NCR-MT power class/class definition.

As for optional feature, one issue is whether MPR should be defined for NCR-MT, it so, Modified MPR behaviour is also suggested for NCR-MT as optional.

Proposal 6: if MPR requirements are defined for NCR-MT, then Modified MPR behaviour is still optional for NCR-MT, otherwise, such feature is not applicable

As for max duty cycle feature, since NCR-MT is not in close proximity as UE, such feature is not applicable for NCR-MT.

Proposal 7: max duty cycle related feature are not applicable for NCR-MT.

| Features | Index | Feature group | Components | Need of FR1/FR2 differentiation | Note | Mandatory/Optional for UE |
| --- | --- | --- | --- | --- | --- | --- |
| 1. System parameter | 1-1 | 60kHz of subcarrier spacing for FR1 | 60kHz subcarrier spacing for data channel in FR1 | Applicable only to FR1 |  | Optional with capability signalling |
|  | 1-2 | 64QAM modulation for FR2 PDSCH | 64QAM modulation for FR2 PDSCH | Applicable only to FR2 | Capability can be discussed in future, e.g. when low cost device (e.g. IoT) and/or higher frequency band in FR2 are introduced | Mandatory without capability signalling |
|  | 1-3 | 64QAM for PUSCH | 64QAM for PUSCH | No | Capability can be discussed in future, e.g. when low cost device (e.g. IoT) and/or higher frequency band in FR2 are introduced | Mandatory without capability signalling |
|  | 1-4 | 256QAM for PDSCH | 256QAM for PDSCH | Yes | For FR1, it can be revisited in the future whether the 256QAM is mandated in all UE types or categories | Mandatory with capability signalling for FR1 |
|  |  |  |  |  | For FR2, RAN4 agreed that no BS and UE requirements will be introduced in Rel.15. | Optional with capability signalling for FR2 |
|  | 1-5 | 256QAM for PUSCH | 256QAM for PUSCH | Yes | For FR1, RAN4 can further discuss to mandate 256QAM for PUSCH for FR1 in future release. For FR2, RAN4 agreed that no BS and UE requirements will be introduced in Rel.15. | Optional with capability signalling (for both FR1 and FR2) |
|  | 1-6 | pi/2-BPSK for PUSCH | pi/2-BPSK for PUSCH | Yes | RAN4 will define the same minimum requirements for pulse-shaped pi/2 BPSK and non-pulse shaped pi/2 BPSK for FR2. | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 |
|  | 1-7 | pi/2-BPSK for PUCCH format 3/4 | pi/2-BPSK for PUCCH format 3/4 | Yes |  | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 |
|  | 1-9 | Support of EN-DC with LTE-NR coexistence in UL sharing from UE perspective | 1) LTE and NR UL Transmission in the shared carrier via TDM only 2) LTE and NR UL Transmission in the shared carrier via FDM only 3) LTE and NR UL transmission in the shared carrier via FDM or TDM | Applicable only to FR1 |  | Optional with capability signalling |
|  | 1-10 | Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective | Support of switching type between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective. Type 1: <0.5us Type 2: <20us | Applicable only to FR1 | This feature is the switching time between LTE UL and NR UL in the same carrier Per band combination signalling UE Capability signalling elements. 1: <0.5us switching type. 2: <20us switching type. | Mandatory to support either type 1 or type 2 with capability signalling if UE reports its capability in 1-10 as 1) LTE and NR UL Transmission in the shared carrier via TDM only, or 3) LTE and NR UL transmission in the shared carrier via FDM or TDM |
|  | 1-11 | 7.5kHz UL raster shift | 7.5kHz UL raster shift | No |  | Mandatory in the SUL bands with uplink sharing either from UE perspective or from network perspective 7.5KHz raster shift as mandatory without capability signalling. 7.5kHz UL raster shift is mandatory for the bands described in clause 5.4.2.1 of Release 15 TS 38.101-1. RAN4 can revisit the above bands in the future release. 7.5KHz raster shift is not mandatory for other LTE refarming band except the bands which were agreed to support 7.5kHz UL raster shift as mandatory |
| 2. UE RF | 2-1 | Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports within a single CC | 1) FR1 channel bandwidths in TS38.101-1 Table 5.3.5-1 2) FR2 channel bandwidths in TS38.101-2 Table 5.3.5-1 | No | UE capability signalling shall follow RP-172832 (Per-band capability signalling, separately for DL and UL and for each SCS) Whether a bandwidth newly introduced in future is mandatory for UE shall be discussed case by case. | For FR1, all the bandwidths listed in TS38.101-1 v15.0.0 Table 5.3.5-1 for each band shall be mandatory with a single CC. The bandwidths listed in the slide #3 of R4-1805985 are mandatory with a single CC. 90MHz is optional for n41, n77, n78. For FR2, the set of mandatory CBW is 50, 100, 200 MHz. |
|  | 2-2 | Simultaneous reception or transmission with same or different numerologies in CA | Support of simultaneous reception or transmission with same or different numerologies in CA | No | From RAN4 perspective UE shall be able to signal the supported SCS per CC for each band combination Same numerology for intra-band NR CA including both continuous and non-continuous is mandatory support for Rel15 The capability of supporting SCS within the single carrier in the CA configuration will be signalled separately, i.e., there is no need to mandatory UE to support mixed numerologies in CA case If a UE supports inter-band NR CA including both FR1 band(s) and FR2 band(s), the UE shall support two mixed numerologies between FR1 band(s) and FR2 band(s) in DL and UL with capability signalling. | Same numerology for intra-band NR CA including both continuous and non-continuous is mandatory with capability in both FR1 and FR2. Two mixed numerologies between FR1 band(s) and FR2 band(s) in DL and UL are mandatory with capability if UE supports inter-band NR CA including both FR1 band(s) and FR2 band(s). Optional for other cases. |
|  | 2-3 | Non-contiguous intra-band CA frequency separation class for FR2 | 1) Support of frequency separation classes to handle the total frequency span for DL for intra-band non-contiguous CA 2) Support of frequency separation classes to handle the total frequency span for UL for intra-band non-contiguous CA | Applicable only to FR2 | UE signals the supported Frequency separation classes with per band granularity (Type 1) based on R4-1803363 Separate Frequency separation classes can be signalled for DL and UL | Mandatory to support a frequency separation class within {I, II, III} specified in TS38.101-2 with capability if UE supports non-contiguous CA in FR2 |
|  | 2-4 | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | No | For TDD-FDD and TDD-TDD band combinations for which simultaneous RxTx capability is agreed to be supported, corresponding capability indication must be set to "supported". Band combinations for which simultaneous RxTx capability is mandatory EN-DC combinations (Both FR1 LTE – FR1 NR and FR1 LTE- FR2 NR) are captured in TS 38.101-3. | Mandatory/Optional support depends on band combination and captured in TS 38.101-3 |
|  | 2-5 | Simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD) | Simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD) | No | For TDD-FDD and TDD-TDD band combinations for which simultaneous RxTx capability is agreed to be supported, corresponding capability indication must be set to "supported". Band combinations for which simultaneous RxTx capability is mandatory are captured in TS 38.101-1, TS 38.101-2 and TS 38.101-3. | Mandatory/Optional support depends on band combination and captured in TS 38.101-1, TS 38.101-2 and TS 38.101-3 |
|  | 2-6 | Asynchronous FDD-FDD intra-band EN-DC DC | Asynchronous FDD-FDD intra-band EN-DC | Applicable only to FR1 |  | Optional with capability signalling |
|  | 2-7 | Almost contiguous UL CP-OFDM | Support of almost contiguous UL CP-OFDM transmissions | Yes | RAN4 had defined the requirements for "Almost contiguous UL CP-OFDM" in Rel-15. | Optional with capability signalling |
|  | 2-8 | UE power class | 1) Support of FR1 UE power class 2) Support of FR2 UE power class 3) Support of FR1 UE power class for EN-DC 4) Support of FR1 UE power class for NR-CA | No | Capability signalling - FR1 UE power class (per band) - FR2 UE power class (per band) - FR1 UE power class for EN-DC (per band combination) - FR1 UE power class for NR CA (per band combination) Default power class for each component is indicated in TS38.101-1/2/3. If the default power class is not indicated, UE shall report supported power class. The component 2) is also used as power class for intra-band NR-CA in FR2 | Mandatory to support at least one power class with capability. The capability signalling is absent if UE supports only default power class |
|  | 2-9 | Simultaneous reception and transmission for SA SUL band combinations | Simultaneous reception and transmission for SA SUL band combinations | No |  | Mandatory/Optional support depends on band combination and captured in TS 38.101-1 |
|  | 2-10 | Multiple frequency band indication | Multiple frequency band indication | No | Per UE capability | Mandatory without capability signalling |
|  | 2-11 | Modified MPR behaviour | Modified MPR behaviour | No | Per band capability | Optional with capability signalling |
|  | 2-12 | Multiple NS/P-Max | Multiple NS/P-Max | No | Per UE capability | Mandatory without capability signalling |
|  | 2-13 | Maximum uplink duty cycle for FR1 power class 2 UE | Maximum percentage of uplink symbols can be scheduled within a certain evaluation period provided by regulatory bodies. The value range is {60%, 70%, 80%, 90%, 100%}. If the field is absent, 50% shall be applied. | Applicable only to FR1 | Per band capability. If this capability is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%, or this capability is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than this capability, apply all requirements for the default power class. The evaluation period is up to UE implementation, no less than one radio frame. UE do not need to do UL duty cycle calculation when it's transmit power is below 23dBm and all the UL/DL configurations can be scheduled. | Optional with capability signalling. The capability signalling is absent if UE supports 50% |
|  | 2-14 | Power boosting for Pi/2 BPSK for power class 3 UE | Power boosting for Pi/2 BPSK for power class 3 UE in TDD bands n40, n77, n78 and n79 with duty cycle less than 40% | Applicable only to FR1 | Per band capability | Optional with capability signalling |
|  | 2-15 | Maximum uplink duty cycle for FR2 | 1) Maximum percentage of uplink transmission time that can be scheduled within 1s time window in order to ensure compliance with applicable electromagnetic power density exposure requirements provided by regulatory bodies. The value range is {15%, 20%, 25%, 30%, 40%, 50%, 60%, 70%, 80%, 90%, 100%}. | Applicable only to FR2 | Per band capability. If the field of UE capability is present and the percentage of uplink symbols transmitted within any 1 s evaluation period is larger than this capability, the UE follows the uplink scheduling and can apply P-MPR as in TS38.101-2. If the field of UE capability is absent, the compliance to electromagnetic power density exposure requirements are ensured by means of scaling down the power density or by other means. This capability is applicable for all power classes in FR2 | Optional with capability signalling |
|  | 2-16 | PA architectures for intra-band EN-DC | Support of dual PA | No | Per band per band combination capability Single PA is default architecture The following requirements are involved by this capability - A-MPR/MPR and MSD values for dual uplink. Whether two sets of requirements will be introduced in RAN4 can be further discussed for each specific band combination - Switching time between LTE UL and NR UL in single switched UL operation mode for intra-band EN-DC | Mandatory to support either single or dual PA architectures with capability if UE supports intra-band EN-DC configuration in uplink. The capability signalling is absent if UE supports single PA architecture. |
|  | 2-17 | PA architectures for intra-band UL CA | Support of dual PA | No | Per band per band combination capability Single PA is default architecture The following requirements are involved by this capability - A-MPR/MPR and MSD values for dual uplink. Whether two sets of requirements will be introduced in RAN4 can be further discussed for each specific band combination | Mandatory to support either single or dual PA architectures with capability if UE supports intra-band CA configuration in uplink. The capability signalling is absent if UE supports single PA architecture |

## 2.2 R16 feature list

According to the analysis as in section 2.2.1 ~2.2.3, we have following observations:

Observation 2: summary of all R16 UE features:

- There is only one mandatory feature, i.e. feature index 8-5 Inter-band DL CA(FR2) to report supported beam management type in TR 38.822, all other feature are optional.

- Features could be categorized as below

  - Band combination related, including dynamic Tx switching related feature, NR CA related feature, e.g. DC location, CA class, Non-contiguous intra-band DL CA (index 8-4) , Inter-band DL CA (index 8-5) and EN-DC related, e.g. max duty cycle index 2-18 and 2-20

  - Transient period, i.e. index 7-4

  - MPE, i.e. index 8-1 P-MPR reporting

  - MPR Enhancement, i.e. index 8-6

  - Beam correspondence related, i.e. SSB based Beam correspondence (index 8-2), CSI-RS based Beam correspondence (index 8-3)

  - Transparent Tx Diversity, i.e. index 2-21

As for the optional feature, except for beam correspondence feature, all other features are suggested as optional.

For beam correspondence, it’s approved that this is based on NCR-MT declaration, if so, related feature is not applicable anymore.

Proposal 8: beam correspondence related feature is not applicable for NCR-MT. except for beam correspondence feature, all other optional R16 feature are also optional for NCR-MT.

### 2.2.1 FR1

Following table list all the feature in TR 38.822 for R16 RF part. Observations are listed as below:

- All R16 FR1 feature are optional.

- Features could be categorized as below

  - Band combination related, including dynamic Tx switching related feature and NR CA related feature, e.g. DC location and CA class

  - Transient period, e.g. index 7-4

Following lists the feature list for information and last column is the recommendation for NCR-MT feature.

Table 5.3.4-1: RF and RRM Feature List for RF requirements for NR frequency range 1 (FR1)

| Features | Index | Feature group | Components | Note | Mandatory/Optional for normal UE | Mandatory/Optional for NCR-MT |
| --- | --- | --- | --- | --- | --- | --- |
| 7. RF requirements for NR frequency range 1 (FR1) | 7-1 | Dynamic Tx switching between two uplink carriers | 1) Indicate support of dynamic UL Tx switching between two uplink carriers for inter-band UL CA, SUL or inter-band EN-DC 2) Indicate the supported switching period for Tx switching between two uplink carriers in inter-band EN-DC, inter-band UL CA or SUL band combinations | Candidate value set for UL CA and SUL combinations: {35us, 140 us, 210us} Candidate value set for EN-DC: {35us, 140 us} NOTE: Signalling structure is up to RAN2 If UE reports support of this feature group, it means UE supports both components. | Optional with capability signalling | Depends on whether NCR-MT support band combinations. |
|  | 7-2 | Application of DL interruptions due to UL Tx switching between two uplink carriers | Capability to indicate that for the band where DL interruption is needed, the RRM interruption requirements defined in RAN4 shall be applied for duplex mode combinations except the combinations - SUL+TDD - TDD+TDD CA with the same UL-DL pattern - TDD+TDD EN-DC with the same UL-DL pattern | The capability is introduced according to the agreement in R4-2005665. NOTE: Signalling structure is up to RAN2 The following duplex mode combinations do not require DL interruption (carrier 1+ carrier 2): - SUL+TDD, - TDD+TDD CA with the same UL-DL pattern, - TDD+TDD EN-DC with the same UL-DL pattern RAN4 will specify for UL CA and EN-DC for which band combinations DL interruptions are allowed. | Optional with capability signalling | Depends on whether NCR-MT support band combinations. |
|  | 7-3a | NR CA class List for intra-band non-contiguous CA | Indicate the UL frequency separation class that UE can support which includes both the aggregated bandwidth and the gap bandwidth between two non-contiguous CCs for intra-band non-contiguous CA Note: UL frequency separation class means maximum frequency span between lower edge of lowest component carrier and upper edge of highest component carrier that UE can support in uplink | Based on the agreed WF R4-2005660 both 1PA and 2PA architecture for intra-band non-contiguous UL CA will be considered for UE capability, and MIMO supporting with 4TX for 2PA UL NC CA should not be excluded. The maximum UL CC number for intra-band UL CA is 2 in Rel-16. NC CA UL separation class candidate values: - Class I: NC CA separation class≤ 100MHz - Class II: 100< NC CA separation class≤ 200MHz - Class III: NC CA separation class > 200MHz and <600MHz | Optional with capability signalling | Depends on whether NCR-MT support band combinations. |
|  | 7-3b | NR CA class List for Intra-band contiguous CA | 1. Indicate the contiguous CA bandwidth class that UE can support in uplink 2. On the condition that component 1 is indicated, indicate the PA architecture, i.e., 1PA or 2PA 3. On the condition that component 1 and component 2 are indicated, indicate the MIMO layer number for each UL CC separately NOTE1: there is dependency for the three components as given above NOTE2: component 1/2/3 are existing signaling from Rel-15, the dependency and conditioned relation need to be ensured in Rel-16 signalling. It is up to RAN2 to decide how to ensure dependency and conditioned relation or new Rel-16 signaling is needed. | for each contiguous CA bandwidth class, if 2PA architecture is indicated, MIMO is not supported for both UL CCs by default | Optional with capability signalling | Depends on whether NCR-MT support band combinations. |
|  | 7-4 | Transient period | Report the shorter transient capability supported by the UE: 2, 4 or 7us | No value reported means UE supports the legacy 10us transient period | Optional with capability signalling | Optional for NCR-MT |
|  | 7-5 | DC location for intra-band CA | Indicate whether UE support Additional DC location reporting for intra-band UL CA |  | Optional with capability signalling | Depends on whether NCR-MT support band combinations. |
|  | 7-6 (RAN2) | Indicating support of the uplink codebook subset when uplink Tx switching is triggered between last transmitted SRS and scheduled PUSCH transmission | UE indicating support of full coherent codebook subset shall also support non-coherent codebook subset. If the field is absent, the supported uplink codebook subset indicated by pusch-TransCoherence applies when the uplink switching is triggered between last transmitted SRS and scheduled transmission. |  | Optional with capability signalling | Depends on whether NCR-MT support band combinations. |

### 2.2.2 FR2

Following table list all the feature in TR 38.822 for R16 RF part. Observations are listed as below:

- Except for one mandatory feature index 8-5 Inter-band DL CA to report supported beam management type, all other feature are optional.

- Features could be categorized as below

  - MPE, i.e. index 8-1 P-MPR reporting and

  - Beam correspondence related, i.e. SSB based Beam correspondence (index 8-2), CSI-RS based Beam correspondence (index 8-3)

  - Band combination related, i.e. Non-contiguous intra-band DL CA (index 8-4) , Inter-band DL CA (index 8-5)

  - MPR Enhancement, i.e. index 8-6

Following lists the feature list for information and last column is the recommendation for NCR-MT feature.

Table 5.3.5-1: RF requirement enhancements for NR frequency range 2 (FR2)

| Features | Index | Feature group | Components | Note | Mandatory/Optional for UE | Mandatory/Optional for NCR-MT |
| --- | --- | --- | --- | --- | --- | --- |
| 8. NR RF Requirement Enhancements for FR2 | 8-1 | MPE | 1 P-MPR reporting |  | Optional with capability signalling |  |
|  | 8-2 | SSB based Beam correspondence | Support for beam correspondence based on SSB A UE indicating support for beam correspondence based on SSB has the ability to select its uplink beam based on measurements of SSB. Supported by UEs with capability beamCorrespondenceWithoutUL-BeamSweeping = {0,1} If a UE supports beam correspondence based on SSB, then the network can expect the UE to also fulfill Rel-15 beam correspondence requirements. |  | Optional with capability signalling |  |
|  | 8-3 | CSI-RS based Beam correspondence | Support for beam correspondence based on CSI-RS A UE indicating support for beam correspondence based on CSI-RS has the ability to select its uplink beam based on measurements of CSI-RS in scenarios when the SSB PSD is X dB below CSI-RS PSD. Supported by UEs with capability beamCorrespondenceWithoutUL-BeamSweeping = {0,1} If a UE supports beam correspondence based on CSI-RS, then the network can expect the UE to also fulfill Rel-15 beam correspondence requirements. |  | Optional with capability signalling |  |
|  | 8-4 | Non-contiguous intra-band DL CA | Support for frequency separation class for DL-only spectrum (Fsd): DL-only spectrum is available for configuration of only DL CCs and not UL CCs. The spectrum covered by the DL-only frequency separation extends on one-side of the bidirectional spectrum in a contiguous manner with no frequency gap between the two. The bidirectional spectrum is defined as the UL/DL common spectrum in which the UE supports the configuration of uplink or downlink CCs and is signalled by UL and DL frequency separation from Rel-15. The combined downlink spectrum (DL Fs + Fsd) cannot exceed 2400 MHz. The component value range is defined in TS38.101-2 |  | Optional with capability signalling |  |
|  | 8-5 | Inter-band DL CA | 1 Indicate the supported beam management type for inter-band CA within FR2. Beam management type can be independent beam management (IBM) or common beam management (CBM) | Candidate value set for beam management type: {IBM, CBM} The capability is restricted to IBM for the band combinations specified in Rel-16 until CBM requirement is specified in a future release. | Mandatory to report the supported beam management type |  |
|  | 8-6 | MPR Enhancement | UE Tx power boost feature when IBE is suspended |  | Optional with capability signalling |  |

### 2.2.3 others

Following table list all the feature in TR 38.822 for R16 RF part. Observations are listed as below:

- All features are optional.

- Features could be categorized as below

  - Max duty cycle for EN-DC, i.e. index 2-18 and 2-20

  - Transparent Tx Diversity, i.e. index 2-21

Following lists the feature list for information and last column is the recommendation for NCR-MT feature.

Table 5.3.12-1: Others

| Features | Index | Feature group | Components | Need of FR1/FR2 differentiation | Note | Mandatory/Optional for UE | Mandatory/Optional for NCR-MT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UE RF | 2-18 | Maximum uplink duty cycle for TDD+TDD EN-DC power class 2 (maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16) | Indicates the maximum percentage of symbols during a certain evaluation period that can be scheduled for NR uplink transmission under different EUTRA TDD uplink-downlink configurations so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. This field is only applicable for inter-band TDD+TDD EN-DC power class 2 UE as specified in TS 38.101-3. If the field is absent, 30% shall be applied to all EUTRA TDD uplink-downlink configurations. If eutra-TDD-Configx is absent, 30% shall be applied to the corresponding EUTRA TDD uplink-downlink configuration. Value n20 corresponds to 20%, value n40 corresponds to 40% and so on. | FR1 only |  | Optional with capability signalling |  |
|  | 2-20 | Maximum uplink duty cycle for FDD+TDD EN-DC power class 2 | The maximum percentage of symbols during a certain evaluation period that can be scheduled for NR uplink transmission and EUTRA FDD uplink transmission so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies for FDD+TDD EN-DC power class 2 UE. | FR1 only | Introduce 2 UE capabilities of maxUplinkDutyCycle-FDD&TDD-EN-DC1 and maxUplinkDutyCycle-FDD&TDD-EN-DC2 which indicate the maxUplinkDutyCycle capability of NR band corresponding to different LTE reference configurations as described in TS 38.101-3 clause 6.2B.1.3. The value range is as below: - maxUplinkDutyCycle-FDD&TDD-EN-DC1, maxUplinkDutyCycle-FDD&TDD-EN-DC2 ∈ {30%, 40%, 50%, 60%, 70%, 80%, 90%, 100%} This field is only applicable for inter-band FDD+TDD EN-DC power class 2 UE as specified in TS 38.101-3. | Optional with capability signalling |  |
|  | 2-21 | Transparent Tx Diversity | Indicates whether the UE supports Tx diversity requirements as specified in TS 38.101-1. The capability applies to all power classes equally in all the applicable releases via a release independent manner. | FR1 only |  | Optional with capability signalling |  |
|  | 2-22 (RAN2) |  | Indicates power class 1.5 the UE supports when operating according to this band combination. If the field is absent, the UE supports the default power class. If this power class is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClass in BandNR), the latter determines maximum TX power available in each band. | FR1 only |  | Optional with capability signalling |  |
|  | 2-23 (RAN 2) |  | Indicates NR part power class the UE supports when operating according to this band combination. This field only applies for MR-DC BCs containing only single CC or intra-band CA in NR side in this release. | FR1 only |  | Optional with capability signalling |  |

## 2.3 R17 feature list

Following table list all the feature in TR 38.822 for R17 RF part. Observations are listed as below:

- All R17 feature are band combinations related and all as optional.

- Features could be categorized as below

  - Dynamic Tx switching related feature, i.e. Tx switching period, DL interruption and MIMO coherent related features;

  - UE power class/power related feature, i.e.

Observation 3: all R17 UE RF features are band combination related feature and all are optional.

Following list the feature list for information and last column is the recommendation for NCR-MT feature.

Table 6.3.3-1: RF Feature List for NR_RF_FR1_enh

| Features | Index | Feature group | Components | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional for UE | Mandatory/Optional for NCR-MT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16. NR_RF_FR1_enh | 16-1 | Dynamic Tx switching between 2CC 2Tx-2Tx switching | Indicate the supported switching period for dynamic UL Tx switching between two uplink carriers with two transmit antenna connectors in inter-band UL CA or SUL | No need | Applicable only to FR1 | Candidate value set: {35us, 140 us, 210us} Detailed information can refer to the LS to RAN2 in R4-2103234 and R4-2107847. | Optional with capability signalling | Depends on whether NCR-MT support band combinations. |
| 16. NR_RF_FR1_enh | 16-2 | Dynamic Tx switching between 3CC 1Tx-2Tx switching | Indicate the supported switching period for dynamic UL Tx switching between one band (with one carrier) capable of one transmit antenna connector and one band (with two carriers) capable of two transmit antenna connectors in inter-band UL CA or SUL | No need | Applicable only to FR1 | Candidate value set: {35us, 140 us, 210us} Detailed information can refer to the LS to RAN2 in R4-2103234 and R4-2107847. | Optional with capability signalling | Depends on whether NCR-MT support band combinations. |
| 16. NR_RF_FR1_enh | 16-3 | Dynamic Tx switching between 3CC 2Tx-2Tx switching | Indicate the supported switching period for dynamic UL Tx switching between one band (with one carrier) capable of two transmit antenna connectors and one band (with two carriers) capable of two transmit antenna connectors in inter-band UL CA or SUL | No need | Applicable only to FR1 | Candidate value set: {35us, 140 us, 210us} Detailed information can refer to the LS to RAN2 in R4-2103234 and R4-2107847. | Optional with capability signalling | Depends on whether NCR-MT support band combinations. |
| 16. NR_RF_FR1_enh | 16-4 | Application of DL interruptions due to dynamic UL Tx switching | Capability to indicate that for the band where DL interruption is needed, the RRM interruption requirements defined in RAN4 shall be applied for duplex mode combinations except the combinations - SUL+TDD - TDD+TDD CA with the same UL-DL pattern SUL+TDD | No need | Applicable only to FR1 | The same capability for Rel-16 DL interruption due to Tx switching is reused. Detailed information can refer to the LS to RAN2 in R4-2103234. | Optional with capability signalling | Depends on whether NCR-MT support band combinations. |
| 16. NR_RF_FR1_enh | 16-5 | UL-MIMO coherence capability for dynamic Tx switching between 3CC 1Tx-2Tx switching | Capability to indicate whether UL-MIMO coherence is supported when dynamic Tx switching between 3CC (within 2 bands) 1Tx-2Tx switching is conducted. | No need | Applicable only to FR1 | The Rel-16 UL-MIMO capability for 2CC 1Tx-2Tx switching is reused. Detailed information can refer to the LS to RAN2 in R4-2120039. | Optional with capability signalling | Depends on whether NCR-MT support band combinations. |
| 16. NR_RF_FR1_enh | 16-6 | UL-MIMO coherence capability for dynamic Tx switching between 2Tx-2Tx switching | Capability to indicate whether UL-MIMO coherence is supported when dynamic Tx switching between 2CC or 3CC (within 2 bands) 2Tx-2Tx switching is conducted. | No need | Applicable only to FR1 | Detailed information can refer to the LS to RAN2 in R4-2120039. | Optional with capability signalling | Depends on whether NCR-MT support band combinations. |
| 16. NR_RF_FR1_enh | 16-8 | UE power class per band per band combination | Per band per band combination power class | No | FR1 only |  | Optional with capability signalling | Depends on whether NCR-MT support band combinations. |

Table 6.3.4-1: RF Feature List for NR_RF_FR2_req_enh2

| Features | Index | Feature group | Components | Need of FR1/FR2 differentiation | Note | Mandatory/Optional for UE | Mandatory/Optional for NCR-MT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 17. FR2 interband CA | 17-4 | Support of beam management | Capability of support of specific beam management type. | FR2 only | Indicate the supported beam management type for inter-band CA within FR2. Beam management type can be independent beam management (IBM) or common beam management (CBM), or both. The capability is only applicable to band combinations with two bands. UE is not allowed to report CBM or both in Rel-17. | Optional with capability signalling | Depends on whether NCR-MT support band combinations. |
| 17. DC-location | 17-5 | Support of UL DC location(s) report | Capability of support for the extended DC location reporting (based on indicated default DC location) for at least 2 UL CCs in one band. | No |  | Optional with capability signalling | Depends on whether NCR-MT support band combinations. |
| 17. New CA BW classes | 17-6 | Support of new CA BW Classes | RAN4 has introduced new CA BW Classes R2~R12, and ['R, S, T, U'] for REL17 | FR2 only |  | Optional with capability signalling | Depends on whether NCR-MT support band combinations. |

Table 6.3.13-1: RF and RRM Feature List for Higher Power Limit CA_DC

| Features | Index | Feature group | Components | Need of FR1/FR2 differentiation | Note | Mandatory/Optional for UE | Mandatory/Optional for NCR-MT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 26. Increased MOP for CA and DC | 26-1 | Higher Power Limit CA_DC | Support of increase in maximum output power above the power class indication | FR1 only |  | Optional with capability signaling | Depends on whether NCR-MT support band combinations. |

# 3. Conclusions

In this contribution, NCR feature list are discussed with following observations and proposals:

Observation 1: summary of all R15 UE features:

- Following are mandatory feature without capability signaling

- 64 QAM, 256 QAM and pi/2-BPSK related, i.e. index 1-2, 1-3, 1-4, 1-6, 1-7

- 7.5kHz UL raster shift, i.e. index 1-11

- Max CBW, i.e. index 2-1

- Band combination related, i.e.

  - index 2-2 Simultaneous reception or transmission with same or different numerologies in CA,

  - index 2-3 Non-contiguous intra-band CA frequency separation class for FR2,

  - 2-4, 2-5, 2-9 Simultaneous reception and transmission,

  - 2-16, 2-17 PA architectures

- UE power class, i.e. index 2-8

- Multiple frequency band indication, i.e. index 2-10

- Multiple NS/P-Max, i.e. index 2-12

Proposal 1: it’s suggested to discuss whether 256 QAM, 64 QAM and pi/2 BPSK modulation scheme are mandatory or not for NCR-MT before concluding related feature. Our suggestion is as below

  - 64 QAM mandatory for both FR1 and FR2 UL and DL

  - 256QAM mandatory for FR1 DL, optional for FR1 UL and FR2 UL and DL

  - pi/2 BPSK optional for FR1 and mandatory for FR2

  - Power boosting for Pi/2 BPSK NCR-MT is optional, the same as UE

proposal 2: 7.5kHz raster shift, multiple frequency band indication and Multiple NS/P-max are mandatory for NCR-MT

proposal 3: max CBW is optional for NCR-MT

proposal 4: it’s suggested to figure out whether NCR-MT could support band combinations or only support single carrier in this release. If NCR-MT support band-combinations, the same feature as UE except for max duty cycle related feature which need further check

proposal 5: UE power class feature wait for conclusion of NCR-MT power class/class definition.

Proposal 6: if MPR requirements are defined for NCR-MT, then Modified MPR behaviour is still optional for NCR-MT, otherwise, such feature is not applicable

Proposal 7: max duty cycle related feature are not applicable for NCR-MT.

Observation 2: summary of all R16 UE features:

- There is only one mandatory feature, i.e. feature index 8-5 Inter-band DL CA(FR2) to report supported beam management type in TR 38.822, all other feature are optional.

- Features could be categorized as below

  - Band combination related, including dynamic Tx switching related feature, NR CA related feature, e.g. DC location, CA class, Non-contiguous intra-band DL CA (index 8-4) , Inter-band DL CA (index 8-5) and EN-DC related, e.g. max duty cycle index 2-18 and 2-20

  - Transient period, i.e. index 7-4

  - MPE, i.e. index 8-1 P-MPR reporting

  - MPR Enhancement, i.e. index 8-6

  - Beam correspondence related, i.e. SSB based Beam correspondence (index 8-2), CSI-RS based Beam correspondence (index 8-3)

  - Transparent Tx Diversity, i.e. index 2-21

Proposal 8: beam correspondence related feature is not applicable for NCR-MT. except for beam correspondence feature, all other optional R16 feature are also optional for NCR-MT.

Observation 3: all R17 UE RF features are band combination related feature and all are optional.

# 4. Reference

[1] R4-2305931, WF on system parameters and RF requirements, ZTE

[2] RP-223505, Revised WID on NR network-controlled repeaters, ZTE, Sanechips.

[3] R1-2304216, Session Notes of AI 9.17.4, AT&T