3GPP TSG-RAN WG4 Meeting # 107	R4-2308239

# Incheon, KR, May 22nd – May 26th, 2023

Source: 	vivo

Title: 	Further discussion on 8Rx UE RF requirements

Agenda Item:	8.5.1.2

Document for:	Discussion

Introduction

In RAN4#106bis-e, progresses were made for 8Rx requirements. The main progress was documented in the WF [1]. The detailed documents and discussion summary can reference to the topic summary in [2].

In this paper, we provide more analysis based on the WF.

Discussion

Indication of ΔTRxSRS

This topic has not been really discussed in last meeting and most companies chose to wait for RAN1 reply. Here we simply reference the related observations and proposals from previous paper [3].

Observation 1: Performance loss due to SRS IL imbalance is limited for 1T4R and 1T8R, while there is almost no performance loss for 1T2R.

Observation 2: Due to non-ideal UE reporting and gNB compensation, including IL measurement error and IL reporting quantization error, performance gain of gNB compensation is marginal.

Proposal 2: Considering the various limitations and in accuracies, no SRS IL reporting seems needed at least for 4Rx / 2Rx case.

Observation 3: Almost no performance loss is caused by IL imbalance for 1T8R/1T4R/1T2R, when considering better condition of the IL imbalance between UL and DL together.

Proposal 3: Considering the overall UL/DL, the need of enhancement is further reduced for UE IL imbalance issue from PDSCH performance perspective.

Currently we continue to have the following proposal:

Proposal 1: No SRS IL reporting needed at least for 4Rx / 2Rx case. Further check this issue after receiving RAN1 reply.

Other RF Rx single carrier requirements

For UE equipped with 8Rx, the UE RF Rx requirements other than REFSENS has several options. Based on our understanding, the current wording in the WF means:

- Option 1: Verified with 4Rx. In another word, 8Rx only verify REFSENS.

- Option 2: Verified with both 4Rx and 8Rx. In another word, 8Rx also verify for RX requirements other than REFSENS in addition to 4Rx requirements.

- Option 3: Verified with 8Rx only if 8Rx is supported, and in the mean time 4Rx not need to be verified.

Based on our understanding, we tend to prefer option 1 which might be the most efficient one and can also satisfy the need for verification. Option 3 may also be ok since the overall test seems not increased.

Proposal 2: For UE equipped with 8Rx, UE should verify 8Rx REFSENS requirements. Other Rx requirements may be verified under either 8Rx or 4Rx condition.

CA requirements

As discussed, the WID may need certain update to fully reflect this part. One issue is regarding the whether the following proposal is agreeable or not:

  - Proposal 3: Clarify the relationship between MSD and ΔRIB, 8R by adding the following sentence in section 7.3A.1.

    - For operations with 8 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,8R in Table 7.3.2-x when MSD > 0.

Based on our understanding, this is a natural extension based on 4Rx case and no other issues are identified. This proposal can be agreed at least technically and further wording refinements can still be considered.

Proposal 3: Agree the following proposal in principle:

  - Clarify the relationship between MSD and ΔRIB, 8R by adding the following sentence in section 7.3A.1.

    - For operations with 8 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,8R in Table 7.3.2-x when MSD > 0.

Conclusion

In this paper, some views are provided for 8Rx for FR1 enh.

Proposal 1: No SRS IL reporting needed at least for 4Rx / 2Rx case. Further check this issue after receiving RAN1 reply.

Proposal 2: For UE equipped with 8Rx, UE should verify 8Rx REFSENS requirements. Other Rx requirements may be verified under either 8Rx or 4Rx condition.

Proposal 3: Agree the following proposal in principle:

  - Clarify the relationship between MSD and ΔRIB, 8R by adding the following sentence in section 7.3A.1.

    - For operations with 8 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,8R in Table 7.3.2-x when MSD > 0.

- References

[1] R4-2306597, WF on UE RF for 8Rx, NTT DOCOMO, RAN4#106

[2] R4-2306287, Topic summary for [106-bis-e][125] FR1_enh2_part3, Moderator (NTT DOCOMO), RAN4#106bis-e

[3] R4-2305082, Discussion on 8Rx UE RF requirements, vivo, RAN4#106bis-e