3GPP TSG-RAN WG4 #107                                                                           R4-2308267

Incheon, KR, May 22nd – May 26th, 2023

Agenda item:			8.20.2.2.4

Source:	vivo

Title:	TP to TR 38.858 on Feasibility of FR2-1 UE aspects

Document for:	Approval

1      Introduction

In RAN4#106bis-e, there was one TP [2] endorsed on feasibility of FR2-1 UE aspects for TR 38.858 [1]. However, after reviewing the content for FR2-1, there was some misalignment with the latest agreement in [3]. In this contribution, we propose some changes based on the endorsed TP [2].

The reasons for changes are listed as follows:

- Noise figure was agreed as 10 dB for FR2-1 instead of a range.

- Subband selectivity was agreed as 23 dB for FR2-1 and the content needs to be updated.

- Subband selectivity definition needs to align with the agreement in [3]

  - Subband in-channel selectivity is the ratio of the interference power in the assigned downlink subband to the received jammer power in the adjacent uplink subband, as measured after the FFT operation

The changes for the TP contents are made with revision marks.

2      References

- R4-2302887 3GPP TR 38.858 V0.4.0

- R4-2305971, TP to TR 38.858 feasibility of and impact on FR2 UE RF requirements, Qualcomm, RAN4#106bis-e.

- R4-2305969, WF for SBFD feasibility study and requirements impact from UE aspect, Qualcomm, RAN4#106bis-e.

3      Text proposal

Begin changed section ******************************************************

10.7 FR2-1 Feasibility of UE aspects

10.7.1    Interference analysis

10.7.1.1  UE-UE co-channel inter-subband CLI modeling

Editor's note: This section captures the CLI modeling.

10.7.1.1.1 Receiver aspects

Existing co-channel UE RX performance requirements

For legacy UEs, the current UE RF architecture can be assumed without any RF architecture modification.

Currently there are no RF requirements for UE co-channel Rx performance.

Sub-band filtering and legacy UEs

For legacy UEs, no sub-band filtering is implemented, and therefore RAN4 has not assumed any subband filtering.

Sub-band filtering for a SBFD-aware UEs – UEs with a new feature

SBFD-aware UEs support half duplex operation while receiving the sub-band configuration from the network. For this kind of UE, the RF architecture and applicable RF requirements need further study for future releases. In the present study, legacy UEs are the main focus.

For new SBFD aware UEs, we need further study on whether sub-filtering can be considered or not. The subband/in-channel selectivity needs further study.

.

Thermal self-noise aspects (both adjacent channel and co-channel)

RAN4 decided on a simple fixed-value noise figure model for the UE receiver. Generally, the receiver noise figure will vary with the input power level, however the single value noise figure model was considered to be sufficient for the purpose of system studies for SBFD. RAN4 agreed on a NF of 10 dB.

Subband in-channel selectivity

It is worth noting that the RF degradations can cause inter-subband interference . We analyzed the design of the FR2-1 receiver. There are multiple considerations in the receiver design … for example residual sideband, reciprocal mixing, integrated phase noise, IM3 distortion, and ADC distortions. The effect of all these distortions is lumped into a single parameter we call selectivity. Based on the discussion and analysis from the meeting, RAN4 agreed to adopt 23 dB as subband selectivity for FR2-1. The receiver performance is simply modelled as being 23 dB below the total input power level Pin. Pin the total power into the receiver, whether it is signal or jammer.

The definition of Sub-band/In-channel selectivity is introduced for clarity in the SBFD feasibility study:

- [Subband in-channel selectivity is the ratio of the interference power in the assigned downlink subband to the received jammer power in the adjacent uplink subband, as measured after the FFT operation.]

FFT leakage and selectivity

As in FR1, the FFT leakage aspect is not a dominant factor in the UE receiver, and we do not include any factor for this effect.

Jammer frequency and time offset were studied are not significant factors influencing UE-UE interference.

10.7.1.1.2 Transmitter aspects

Inband emissions (co-channel)

RAN4 has decided to use the IBE requirements from 38.101-2 clause 6.4.2.3.4 (power class 3 UE). It is understood these requirements are minimum performance requirements as opposed to typical requirements. RAN4 has agreed to use typical requirements for the UE parameters, however, did not conclude on the typical values so we are using the formulation from the MPS.

It should also be assumed the LO location is in the center of the channel for the purposes of system studies in RAN4. The LO location is important as it allows placement of the image.

Analysis indicates that the IBE interference is higher and dominates the sub-band co-channel selectivity.

Apart from the selectivity, the degradation can be caused by transmitter leakage from the UL sub-band into the DL sub-band. For co-channel case, the leakage was agreed to be modelled using IBE based model. Additionally, the IQ image contribution for the IBE model for co-channel CLI can be ignored for the DUD configuration.

For UE co-channel Tx model, UE IBE in TS 38.101-2 can be used in the feasibility study as shown in Table 10.7.1.1-1. This model consists of three parts, General, IQ image, Carrier leakage. In the system level simulation, the general and IQ image parts shall be considered, while the carrier leakage part can be ignored in the feasibility study. For DUD configuration, the IQ image from the uplink is fully contained in the UL sub-band and does not land in the DL subband, thus the IQ image can also be ignored in the simulation.

Table 10.7.1.1-1: Requirements for in-band emissions in TS 38.101-2 (For Power class 3)

| Parameter description | Unit | Limit (NOTE 1) |  |  | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- |
| General | dB |  |  |  | Any non-allocated (NOTE 2) |
|  |  |  | Output power for FR2-1 | Output Power for FR2-2 |  |
| IQ Image | dB | -25 | > 10 dBm | > 8.1 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | ≤ 10 dBm | ≤ 8.1 dBm |  |
| Carrier leakage | dBc | -25 | > 0 dBm | > -1.9dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 0 dBm | -14.9 dBm ≤ Output power ≤ -1.9 dBm |  |

10.7.1.2  UE-UE adjacent channel CLI modeling

Editor's note: This section captures the CLI modeling.

10.7.1.2.1 Receiver aspects

Effect of adjacent channel aggressor UE jammer

We analyzed the design of the FR2-1 receiver. There are multiple considerations in the receiver design … for example residual sideband, reciprocal mixing, integrated phase noise, IM3 distortion, and ADC distortions. The receiver performance is simply modelled as being 23 dB below the total input power level Pin. Pin the total power into the receiver, whether it is signal or jammer.

10.7.1.2.2 Transmitter aspects

ACLR (adjacent channel)

ACLR is one aspect modelled as an interference aspect from a nearby aggressor UE transmitting in an adjacent UL subband. UE ACLR is modeled as 24 dB at max power, improving 1 dB/dB with backoff up to a maximum of 10 dB of improvement. Therefore, when the backoff is 10 dB, the ACLR is 34 dB. This model is an approximation of the performance of a typical UE.

10.7.2	Summary

Editor's note: This section captures the conclusion of feasibility.

End changed section***********************************************************

4      Annex

- RAN4#104-e

Topic 1: RAN4 feasibility study and RF requirement impact for SBFD operation From UE perspective

Agreement:

•	Using existing UE RF requirements to estimate UE performance and if needed extrapolating them for system level studies

Topic 4: co-channel inter-subband UE-UE CLI model according to RAN1 LS

Candidate considerations for UE-UE CLI model:

- TX model can refer to existing UE requirement in TS38.101-1 and TS38.101-2

  - In band emission as starting point

  - FFS is not precluded for other candidates such as ACLR

- RX model can refer to existing UE requirement in TS38.101-1 and TS38.101-2

  - Maximum input power as threshold based on above specification

  - FFS is not precluded for other candidates such as ACS, ICI, and estimated RX model based on legacy UE.

Topic 6: adjacent-channel UE-UE CLI model according to RAN1 LS

Agreement on feasibility and how to model UE-UE CLI modelling considering unwanted emission and receiver selectivity:

- Model as starting point : UE ACLR based model on TX and UE ACS based model on RX which is the same ACIR model as Rel-16 CLI study.

- FFS on below model

  - UE ACLR model with 2step size(FR1 example: ACLR1/2=28/33dB) on TX

  - UE ACS based model on RX if blocker is smaller than maximum input level of UE, and additional SNR degradation at the victim receiver due to receiver gain backoff

- FFS on how the per-sub-band/RB aspect is characterised. Other aspect is also not precluded

- RAN4#104bis-e

UE TX aggressor toward adjacent channel victim (FR1)

What base value for ACLR1 in TX model for FR1 power class 3?

Agreement:

•	30 dB is the total distortion power on either side of a fully allocated uplink sub-band. The ACLR1 distortion PSD is modeled as flat over that range (From the agreement below)

•	FFS whether we need to consider whether we need to model allocations that are less than fully  allocated uplink sub-bands

What base value value should the model use for FR1 PC3 ACLR2?

Agreement:

•              Follow Ericsson suggestion and evaluate the effect of UE-UE CLI with ACLR1 only.

•              Revisit the discussion on ACLR2 if UE-UE CLI becomes significant

Do we need to model TX power classes other than FR1 PC3?

Agreement: Power class 3 only

What is the frequency resolution (granularity) of the model(ACLR)?

Agreement: Option 2 – Distortion is modeled as a flat power spectral densitity across the frequency range of the distortion

Should the ACLR-based interference be scaled with backoff?

Agreement:

•	Option 3 – Do not model improved ACLR with backoff

•	Revisit the discussion on backoff-dependent ACLR if UE-UE CLI becomes significant

Should the model use ACLR or OBW as the base value?

Agreement: For FR2-1 use OBW as basis (23 dB)

Should the FR2-1 model include an ACLR2-type aspect, similar to FR1?

Agreement: ACLR-2 model aspect is precluded for FR2-1

1.2	Co-channel model

1.2.1	UE TX aggressor toward co-channel victim (FR1)

Agreement: Use IBE-based model for co-channel

Agreement: IBE-based model granularity is 1 RB.

Agreement: The IBE-based model should Include the image aspect of IBE and assume the LO is in the middle of the channel to allow for correct placement of the image frequency.

1.2.2	UE TX aggressor toward co-channel victim (FR2-1)

Agreement: Use the same approach as in adjacent channel aggressor model for FR2-1

2	UE RX modelling aspects

2.1	Adjacent channel model

2.1.1	UE RX victim from adjacent channel aggressor (FR1)

Agreement: agree 33 dB value (33 dB comes from ACS) as performance point in the RX model

Agreement: If the blocker is higher than -25dBm, it is assumed it will result large receiver degradation and hence the RX will not correctly decode the data (100% packet loss)

2.1.2	UE RX victim from adjacent channel aggressor (FR2-1)

Agreement: agree 23 dB value (from ACS) as performance point in the FR2-1 model

Agreement: For FR2-1 Use the same method as in FR1, with changes being related to the parameters of ACS value, REFSENS, and maximum input power level

2.2.2	UE RX victim from co-channel aggressor (FR2-1)

Agreement: For FR2-1 use the same method as in co-channel RX victim for FR1. Note that the co-channel RX victim method for FR1 has not been agreed yet.

- RAN4#105

Issue 2-1-3.2: Receiver sub-band selectivity

Proposed agreement:

- For legacy UE: For receiver sub-band selectivity, no rejection/attenuation due to RF/BB filtering is assumed on interference in adjacent sub-band as legacy UEs do not operate this way.

  - Use typical model for UE selectivity value

  - The selectivity and performance of the FFT is included in RAN4 study for co-channel case

    - FFS whether the adjacent channel case requires the selectivity and performance of the FFT.

  - RAN4 should consider interferer with timing or frequency offset or both w.r.t. the desired signal for the co-channel case

    - FFS whether this applies to the adjacent channel case

- For new SBFD capable UE, further analysis of the possibility to improve selectivity performance under the assumption that UE channel bandwidth not equal the sub-band bandwidth.

- Companies come next meeting with technical proposals on the level of interference from an UL sub-band co-channel interferer to the UE DL sub-band. So far companies have proposed:

  - 33 dB at the ADC output (for FR1) based on typical performance. FFS for FR2-1

  - 25 dB (for FR1 and FR2-1)

  - 0 dB (for FR1 and FR2-1)

  - Other values not precluded for discussion next meeting.

- RAN4#106

Sub-band selectivity

Agreement:

- The definition of sub-band/in-channel selectivity for SBFD feasibility study purpose

  - For one input level and one jammer level, Sub-band/In channel selectivity is the ratio of the receive power on the assigned sub-band to the receive power on the adjacent sub-band after FFT operation.

    - For legacy UE, no sub-band filtering considered

    - For new SBFD aware UE: FFS whether sub-filtering can be considered or not

- For FR1 companies are encouraged to further discuss values in the range of [20 to 33 dB] for sub-band/in-channel selectivity with accompanying clarification as how they calculate DL subband interference based on one value from this range and what guard band is assumed.

For FR2-1 companies are encouraged to further discuss values in the range of [20 to 34 dB] for sub-band/in-channel selectivity with accompanying clarification as how they calculate DL subband interference based on one value from this range and what guard band is assumed.