3GPP TSG-RAN WG4 Meeting #107	R4-2308387

Incheon, South Korea, May 22nd – 26th, 2023

Agenda Item:	8.7.3.1

Source:	MediaTek Inc.

Title: 	On beam correspondence requirement applicability for IA and RRC_INACTIVE mode

Document for:	Discussion

# Introduction

FR2 beam correspondence for initial access and RRC_INACTIVE has been discussed for a couple of RAN4 meetings, and the agreed way forward in RAN4#106bis-e includes the following two open issues related to Msg1 [1] [2]:

## Sub-topic 1-1 Minimum peak EIRP and spherical coverage for msg1 Issue 1-1-1: minimum peak EIRP for msg1 + spherical coverage package - Proposals - Option 1: Do not specify the min peak EIRP requirements but Same spherical coverage as RRC_Connected mode - Option 2: Lower than the min peak EIRP of RRC_Connected mode + Same spherical coverage as RRC_Connected mode - Option 3: Same as min peak EIRP of RRC_Connected mode + Same spherical coverage as RRC_Connected mode - Option 4: For beam correspondence in initial access, reuse the same beam lock mode and the same beam type assumption as that of connected mode beam correspondence. And accordingly EIRP requirements and RS side condition can also be reused. Way forward/Agreements: Further discuss in RAN4#107 meeting. Implication between Issue 1-1/1-5/1-6 need to be investigated. - Companies are encouraged to provide justification. ## Sub-topic 1-6 UE capability - Proposals - Option 1: Only the UE support both beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 is considered can support msg1 beam correspondence. - Option 2: For supporting UE beam correspondence requirements for RRC_INACTIVE and initial access UE needs to support both beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 UE capabilities. - Option 3: RAN4 specs to clarify that Rel-18 or newer UEs supporting beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 is considered as a Msg1 beam correspondence capable UE. Way forward/Agreements: - Follow previous RAN4 decision that new UE capability is not introduced for initial access. - FFS for the options above.

In this contribution we provide our views and propose to conclude these two open issues.

# Discussion

## 2.1 Beam correspondence requirements for Msg1

For initial access, a UE transmits Msg1 which gNB needs to make a detection on Msg1, and in RRC_CONNECTED, gNB demodulates and decodes PUSCH. Actually there is some gap between the required SNR for detecting Msg1 and decoding PUSCH at gNB. If we take the example of TDLA30-300 and 60k SCS according to BS type 2-O performance requirements [3], the required SNR for 1% miss detection on Msg1 can be -6.9dB, and for PUSCH type A demodulation, the required SNR for 70% maximum throughput is -2.5dB which is higher than that for Msg1. Given that UE has the same maximum output power for PRACH and PUSCH, the PRACH coverage is larger than the PUSCH coverage.

And this enables the possibility of having a relaxed peak EIPR requirements for Msg1 compared with that for RRC_CONNECTED state. In this way, a UE does not have to switch on all its antennas when transmitting Msg1. To address the concern on spherical coverage for Msg1, it could be a good compromise to reuse the requirement for RRC_CONNECTED state, thus Option 2 would be a good choice.

However, according to the discussions up to last RAN4 meeting, it seems difficult to reach consensus on whether or not a relaxed requirement should be specified for IA and RRC_INACTIVE compared with that in RRC_CONNECTED. Option 1 (i.e., do not specify EIRP but apply the same spherical coverage) could be a compromise.

Proposal 1: For the sake of progress, RAN4 to agree on a compromise that no EIRP but the same spherical coverage requirement is applied to IA and RRC_INACTIVE, i.e., Option 1.

## 2.2 UE applicability

RAN4#105 has agreed that there is no need to introduce a new UE capability for beam correspondence, and in particular, if we extend the interpretation on the existing UE capabilities, i.e., the proposal “Only the UE support both beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 is considered can support msg1 beam correspondence”, there should be some way to avoid confusion on UEs implementation. For example, if both some legacy UEs and Rel-18 UEs may support both beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16, but legacy UEs cannot support beam correspondence for Msg1, while the new Rel-18 UEs can support. One way to avoid confusion is that RAN4 specs clarify that Rel-18 or newer UEs supporting beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 is considered as a Msg1 beam correspondence capable UE.

Proposal 2: RAN4 specs to clarify that Rel-18 or newer UEs supporting beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 is considered as a Msg1 beam correspondence capable UE, i.e., Option 3.

# Conclusion

In this contribution we have the following proposals for beam correspondence requirements for IA and RRC_INACTIVE:

Proposal 1: For the sake of progress, RAN4 to agree on a compromise that no EIRP but the same spherical coverage requirement is applied to IA and RRC_INACTIVE, i.e., Option 1.

Proposal 2: RAN4 specs to clarify that Rel-18 or newer UEs supporting beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 is considered as a Msg1 beam correspondence capable UE, i.e., Option 3.

# References

- R4-2306600, WF on beam correspondence in initial access and RRC_INACTIVE, Nokia

- R4-2306289	Topic summary for [106-bis-e][127] FR2_enh_req_Ph3_part1, Moderator(Nokia)