3GPP TSG-RAN WG4 Meeting # 107                               R4-2308575

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on RF requirements for CA_n5-n8

Agenda Item:	7.30.2.1

Document for:	Approval

Introduction

In the way forward [1] on CA_n5-n8, UE RF requirements for three options are considered. In this paper, we’d like to provide our views and analysis on these requirements.

Discussion on MSD for n8 DL from n5 UL case for three options

Based on the way forward [1], one candidate test point was proposed to further evaluate this case below.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n8 | 844 | 10 | 15 | 25 (RBstart=27) | 951.5 | 5 | TBD | >ACLR2 |

As the frequency separation between band n5 UL and n8 DL is about 100MHz, -130dBm/Hz can be assumed for n5 transmitter noise floor.

As we investigated in the paper [2], the performances of band n5 duplexer and band n8 Rx filter (3-antenna implementation assuming the existing filters) are listed as below according to the real products performance as shown in figure 1 and 2.

Table 1 Attenuation and isolation values

| Attenuation and isolation parameter | Value |
| --- | --- |
| (Band n5 Tx to Ant) Attenuation at DL band n8 frequency range | 40dB |
| (Band n8 filter Ant to Rx)Attenuation at UL band n5 frequency range | 40dB |
| Antenna ISO | 10dB |
| RF front end loss | 4dB |

Figure 1 The performance for band n5 Tx filter (Tx to Ant)

Figure 2 The performance for band n8 Rx filter (ANT to Rx)

Even if we take a look into band n26 duplexer performance, at least 38dB rejection at band n8 DL frequency range can be observed for band n26 Tx filter in figure 3.

Figure 3 The performance for band n26 Tx filter (Tx to Ant)

Even if we take 35dB rejection at band n8 DL frequency range for band n26 Tx filter into consideration, we can get the band n5 PA transmitter at band n8 DL is -108.5 dBm = -130 + 60 + 10log(4.5) – 35 (Tx filter performance) – 10 (Antenna ISO). So the impacts on band n8 DL is limited from band n5 Tx band.

Proposal 1: there is no MSD for band n8 DL due to band n5 UL interference since RF components can provide necessary isolation.

Discussion on MSD due to IMD for option 2

Based on the IMD analysis for the impacts on band n8 DL from UL CA_n5-n8, IMD3 can be observed. For IMD5, there is no reasonable test point which can be found.

Table 2 IMD analysis for the impacts on band n8 DL from UL CA_n5-n8

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 880 | 915 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 31 | 91 | 1704 | 1764 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 733 | 818 | 911 | 1006 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2528 | 2613 | 2584 | 2679 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1557 | 1667 | 1791 | 1921 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3352 | 3462 | 3464 | 3594 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 182 | 62 | 3408 | 3528 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2836 | 2671 | 2516 | 2381 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1097 | 942 | 787 | 642 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4344 | 4509 | 4176 | 4311 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4288 | 4443 | 4232 | 4377 |

Referring to MSD due to IMD3 for CA_n8-n20, 25dB MSD can be considered with the following test point (minor correction on frequency center compared to the way forward)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n8 | n5 | 845 | 5 | 25 (RBstart=0) | N/A | N/A | FDD | N/A |
|  | n8 | 890 | 5 | 25 (RBstart=0) | 935 | [25] | FDD | IMD3 |

Proposal 2: To consider the following band n8 DL MSD due to IMD3 from UL CA_n5-n8 for option 2 referring to CA_n8-n20.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n8 | n5 | 845 | 5 | 25 (RBstart=0) | N/A | N/A | FDD | N/A |
|  | n8 | 890 | 5 | 25 (RBstart=0) | 935 | [25] | FDD | IMD3 |

Discussion on MSD due to IMD for option 3

For option 3, UL frequency range of band n8 is restricted as 904~915MHz (n8R). Thus, the IMD analysis may be different from option 2.

Table 3 IMD analysis for the impacts on band n8 DL from UL CA_n5-n8R for option 3

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 904 | 915 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 55 | 91 | 1728 | 1764 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 733 | 794 | 959 | 1006 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 2613 | 2632 | 2679 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1557 | 1643 | 1863 | 1921 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3376 | 3462 | 3536 | 3594 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 182 | 110 | 3456 | 3528 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2836 | 2767 | 2492 | 2381 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1097 | 1014 | 739 | 642 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4440 | 4509 | 4200 | 4311 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4360 | 4443 | 4280 | 4377 |

Based on the way forward [1], the following near miss IMD3 MSD test point was proposed to be further evaluated.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n8 | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n8 | 906.5 | 5 | 25 | 951.5 | TBD | FDD | Near miss IMD3 |

The proposed channels for band n5 and n8 are summarized below.

|  | UL frequency range (MHz) | DL frequency range (MHz) |
| --- | --- | --- |
| n5 | 844~849 | 889~894 |
| n8R | 904~909 | 949~954 |

Thus, we can calculate the frequency range which is hit by IMD3 from UL CA_n5-n8R as 959~974 MHz. It can be observed the IMD3 missed the DL channel of band n8 by 5MHz.

Proposal 3: For option3, since the IMD3 missed the DL channel of band n8, there is no need to specify such MSD due to IMD.

- Summary

Proposal 1: there is no MSD for band n8 DL due to band n5 UL interference since RF components can provide necessary isolation.

Proposal 2: To consider the following band n8 DL MSD due to IMD3 from UL CA_n5-n8 for option 2 referring to CA_n8-n20.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n8 | n5 | 845 | 5 | 25 (RBstart=0) | N/A | N/A | FDD | N/A |
|  | n8 | 890 | 5 | 25 (RBstart=0) | 935 | [25] | FDD | IMD3 |

Proposal 3: For option3, since the IMD3 missed the DL channel of band n8, there is no need to specify such MSD due to IMD.

- References

[1] R4-2306467, WF on CA_n5-n8, Skyworks Solutions, Inc.

[2] R4-2302101, Discussion on potential solutions for CA_n5-n8, Huawei, HiSilicon