3GPP TSG-RAN WG4 Meeting # 107                               R4-2308581

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon, Spark NZ

Title: 	Discussion on RF architecture and potential MSD issues for CA_n5-n105

Agenda Item:	8.4.2.1

Document for:	Approval

Introduction

In RAN4#106bis meeting, we have the following WF [1] on RF architecture and critical issues for CA_n5-n105. It seems that this band combination is not very difficult to implement. The remaining issues are related to delta Tib / Rib and MSD study due to cross band isolation. In this paper, we’d like to discuss these parts.

Discussion on Delta Tib and Rib

Based on the discussion, it seems that two-antenna solutions with quadplexer or triplexer are not very challenging for CA_n5-n105 as the frequency gap between band n5 UL part and band n105 DL part is larger than 100MHz. Thus, general values for low band combinations can be used for Delta Tib and Rib.

Proposal 1: The following Delta Tib and Rib values for CA_n5-n105 can be considered.

Table 1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n105A | n5 | 0.5 |
|  | n105 | 0.5 |

Table 2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n105A | n5 | 0 |
|  | n105 | 0 |

Discussion on MSD due to cross band isolation

Referring to contribution [2], the -130dBm/Hz UL noise floor of the band n5 and band n105 transmitters was assumed. In addition, the following respective UL filter rejection in the other band DL was considered.

n5 Tx filter rejection at 612~652MHz is assumed as 30dB.

n105 Tx filter rejection at 869~894MHz is assumed as 30dB.

Three-antenna RF architecture is assumed for MSD evaluation.

We have the following evaluation for MSD due to cross band isolation for CA_n5-n105.

Table 1 MSD due to cross band isolation n5 => n105

|  | parameters | n105 main path | n105 diversity path |
| --- | --- | --- | --- |
| transmit power for n5, dBm |  | 23 | 23 |
| Tx band n5 BW(MHz, Lcrb) | 20 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n5 uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| n105 receival singal at ANT port, dBm |  | 13 | 13 |
| n105 filter rejection at 814~849MHz, dB | 40 |  |  |
| signal After n105 filter, dBm |  | -31 | -31 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -112 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -108 |
| noise figure dB | 11.8 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -162.2 |  |  |
| Rx band n105 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -95.67 | -95.67 |
| ACLR1, dB | 30 |  |  |
| ACLR2, dB | 43 |  |  |
| n5 PA leakage PSD at PA output port at 1st adjacent channel, dBm/MHz |  | -20.01 |  |
| n5 PA leakage PSD at PA output port at 2nd adjacent channel, dBm/MHz |  | -33.01 |  |
| n5 PA noise PSD at PA output port, dBm/Hz | -130 |  |  |
| n5 PA noise PSD at PA output port, dBm/RxBW |  | -63.47 |  |
| n5 Tx filter rejection at 612~652MHz, dB | 30 |  |  |
| n5 PA noise power at Rx n105 ant port at 612~652MHz, dBm |  | -103.47 | -103.47 |
| Total noise level at ANT port |  | -94.79 | -94.79 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -95.79 | -95.79 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(5MHz BW), dBm | -96.80 |  |  |
| MSD | 0.40 |  |  |

Table 2 MSD due to cross band isolation n105 => n5

|  | parameters | n5 main path | n5 diversity path |
| --- | --- | --- | --- |
| transmit power for n105, dBm |  | 23 | 23 |
| Tx band n105 BW(MHz, Lcrb) | 20 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n105 uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| n5 receival singal at ANT port, dBm |  | 13 | 13 |
| n5 filter rejection at 663~703MHz, dB | 40 |  |  |
| signal After n5 filter, dBm |  | -31 | -31 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -112 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -108 |
| noise figure dB | 11 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -163 |  |  |
| Rx band n5 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -96.47 | -96.47 |
| ACLR1, dB | 30 |  |  |
| ACLR2, dB | 43 |  |  |
| n105 PA leakage PSD at PA output port at 1st adjacent channel, dBm/MHz |  | -20.01 |  |
| n105 PA leakage PSD at PA output port at 2nd adjacent channel, dBm/MHz |  | -33.01 |  |
| n105 PA noise PSD at PA output port, dBm/Hz | -130 |  |  |
| n105 PA noise PSD at PA output port, dBm/RxBW |  | -63.47 |  |
| n105 Tx filter rejection at 869~894MHz, dB | 30 |  |  |
| n105 PA noise power at Rx n105 ant port at 869~894MHz, dBm |  | -103.47 | -103.47 |
| Total noise level at ANT port |  | -95.43 | -95.43 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -96.43 | -96.43 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(5MHz BW), dBm | -97.44 |  |  |
| MSD | 0.56 |  |  |

Based on the evaluation above, the REFSENS degradation is neglect. Thus, there is no need to specify MSD due to cross band isolation for CA_n5-n105.

Proposal 2: Based on the evaluation, the REFSENS degradation is neglect for CA_n5-n105. Thus, there is no need to specify MSD due to cross band isolation for CA_n5-n105.

- Summary

Proposal 1: The following Delta Tib and Rib values for CA_n5-n105 can be considered.

Table 1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n105A | n5 | 0.5 |
|  | n105 | 0.5 |

Table 2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n105A | n5 | 0 |
|  | n105 | 0 |

Proposal 2: Based on the evaluation, the REFSENS degradation is neglect for CA_n5-n105. Thus, there is no need to specify MSD due to cross band isolation for CA_n5-n105.

- References

[1] R4-2306469, WF on RF architecture and critical issues for CA_n5-n105, Xiaomi

[2] R4-2304560, Input on new CA_n5A-n105A band combinations study, Skyworks Solutions, Inc.