3GPP TSG-RAN WG4 Meeting #107	R4-2308672

Incheon, KR, 22 – 26 May, 2023

Title: 	Discussion on RRM requirements for CPP

Source: 	Huawei, HiSilicon

Agenda item:	8.23.3.6

Document for:	Discussion

Introduction

RRM requirements for Carrier Phase Positioning (CPP) are discussed in RAN4#106-bis-e, and outcomes are captured in WF [1]. Based on [1] the following issues are to be further discussed.

- General issue

- Core requirements

- Perf requirements

In this paper we will provide our views on RRM requirements for CPP.

Discussion

General issue

Issue 2-1-1: Measurement types to be defined for DL carrier phase measurement requirements: Agreements: - Wait for more progress and outcomes on the definition of measurement from RAN1/RAN2 to define the measurement period requirements for CPP.

In April meeting, RAN1 agreed to support both RSCP and RSCPD as DL CP measurements for CPP.

Agreement Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note. - DL RSCP can be reported together with UE Rx – Tx time difference measurement - DL RSCPD can be reported together with RSTD measurement - FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed - Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed.

Therefore, RAN4 should define measurement period requirements for both measurements. For which measurements to define accuracy can be discussed in Perf part.

Proposal 1: RAN4 to define measurement period requirements for both RSCP and RSCPD. RAN4 to discuss for which measurements to define accuracy in Perf part.

Issue 2-1-6: PRU support: < Way forward > - Option 1: - RAN4 to investigate measurement requirements for PRU, in particular if PRU is expected to support tighter measurement accuracy requirements than target UE.

In last meeting, some companies proposed to define specific requirements for PRU, e.g. a tighter accuracy. We do not support the proposal. In our view, PRU is a UE with positioning function (i.e. to measure PRS and to transmit SRS) and with known location to LMF. Also, the concept has existed since Rel-17. It should meet same requirements as normal UE, and of course, the spec does not preclude a PRU to do better.

Proposal 2: RAN4 not to define specific requirements for PRU.

Core requirements

Issue 2-2-1: Measurement period requirements for DL carrier phase measurement: < Way forward > - Option 1: - If RSCPD/RSCP being reported with other legacy measurements together, RAN4 can define the measurement reporting delay requirements for them with the existing requirements for the associated legacy measurement. - Option 2: - Current core requirements for PRS measurement are used as baseline for carrier phase measurement. Adaptations can be FFS e.g. sample number and applicable PRS configuration. - Option 3: - Other options are not precluded.

RAN1 agreed that RSCP can be reported together with UE Rx-Tx, and RSCPD with RSTD. In such cases, CP measurements are assisting measurements similar to the PRS-RSRP reported together with RSTD or UE Rx-Tx. Following the same principle for PRS-RSRP, measurement period for the other measurements apply for RSCP or RSCPD.

In last meeting, one issue raised up during the discussion is whether other measurements e.g. RSTD can be measured on multiple PFLs while CP is measured on a single PFL. We understand it is possible, and RAN1 will discuss the measurement configuration for CP measurement in existing measurement request. RAN4 can wait for further RAN1 conclusion and check if there is any impact to RAN4 requriements.

Agreement Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including - UE in RRC_CONNECTED state with measurement gap. - FFS: UE in RRC_CONNECTED state without measurement gap - UE in RRC_ INACTIVE state

RAN4 can also discuss the measurement period for CP as stand-alone measurements if agreed in RAN1.

Proposal 3: When RSCP or RSCPD is reported together with other measurements, measurement period for the other measurements apply for RSCP or RSCPD.

Issue 2-1-5: Applicable number of measurement instances: < Way forward > - Option 1: - RAN4 to investigate and specify the measurement period, reporting, and accuracy requirements for DL CPP measurements, for the cases of a single and multiple measurement instances aligned to requirements for legacy positioning techniques in Rel-16 and Rel-17.

In last meeting, number of samples for CP measurement is discussed. If Proposal 3 is agreeable, there would be no need to discuss sample number for measurement period requirements.

On the other hand, we believe there is a need to discuss sample number for accuracy requirements, e.g. whether it is possible to combine phase measurement across multiple samples considering the random phase offset at each sample. In our view, combining across sample might be possible for RSCPD but not for RSCP. For RSCPD, RAN4 may also need to discuss whether it is possible to derive the measurement if two PRS resources for the two TRP are not received in the same time, e.g. the applicable PRS configurations may need to be discussed.

Proposal 4: RAN4 to discuss the number of samples for accuracy requirements for RSCP and RSCPD, as well as the applicable PRS configuration.

Issue 2-2-2: Applicable RF frequencies for DL RSCP measurement period requirements: < Way forward > - Option 1: - The basic measurement requirement for DL RSCP is defined for each RF frequency. And RAN4 to discuss whether to define measurement requirements for multiple RF frequencies.

In April meeting, RAN1 agreed on the frequency reference for a CP measurement from a PFL. Although RAN1 has discussed more than one CP measurements per PFL for more than one frequencies, it has not been agreed in RAN1.

Agreement The specific RF frequency associated with a DL carrier phase measurement is defined as the center frequency of the DL PFL by default. - Note: It is open to further discussion whether a frequency other than the center frequency of the DL PFL can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them.

From RAN4 side, we suggest to waits for RAN1 conclusion on support of multiple CP measurements for multiple RF frequencies. If this is supported in RAN1, RAN4 can further check whether and how it will impact RAN4 requirements.

Proposal 5: RAN4 waits for RAN1 conclusion on support of multiple CP measurements for multiple RF frequencies before discussing the impact on RAN4 requirements.

Issue 2-2-3: Applicable RF frequencies for DL RSCPD measurement period requirements: < Way forward > - Option 1: - The RF frequencies for target TRP and reference TRP in RSCPD measurement can be different.

We understand this issue is related to measurement definition, so it is in RAN1 scope. There is no need for RAN4 to continue discussion on this issue, and RAN4 can further check whether and how it will impact RAN4 requirements based on RAN1 conclusions.

Proposal 6: RF frequencies for target TRP and reference TRP in RSCPD measurement is up to RAN1.

Issue 2-3-4: Measurement reporting requirements: < Way forward > - Option 1: - RAN4 to investigate and specify measurement reporting for NR DL CPP based on DL measurement types specified in TS 38.215. - RAN4 to investigate and specify measurement reporting requirements for NR UL CPP based on UL measurement types specified in TS 38.215 - RAN4 to specify joint measurement reporting requirements for the use case scenario of coexistent NR CPP and legacy NR positioning techniques. - RAN4 to await further RAN1 progress on additional measurement reporting and assistance information to improve the measurement accuracy.

As discussed for Proposal 3, when RSCP or RSCPD is reported together with other measurement, it should be contained in the same measurement report as other measurements. Measurement reporting requirements for other measurements also apply for CP measurements.

Proposal 7: When RSCP or RSCPD is reported together with other measurements, measurement reporting requirements for other measurements apply for CP measurements.

Perf requirements

Issue 2-3-1: Channel conditions for carrier phase measurement accuracy requirements: Agreements: - RAN4 to discuss and decide which fading channels to be used for evaluating and specifying the CPP accuracy requirements in addition to AWGN channel.

Based on RAN1 conclusion, CP measurements are per path measurement, similar to PRS-RSRPP. In this sense, RAN4 could consider using same channel model for PRS-RSRPP for defining accuracy for CP measurements, i.e. two-path static channel.

Proposal 8: RAN4 to consider using same channel model for PRS-RSRPP for defining accuracy for CP measurements.

Issue 2-1-4: Applicable BW range of reference signals for DL and UL carrier phase measurement: Agreements: - The measurement period requirements (formula) for DL measurements are agnostic to the PRS BW. - FFS the PRS/SRS BW range for measurement accuracy requirements. < Way forward > - Option 1: - RAN4 to align the targeted PRS BW range for NR DL CPP in RRC_CONNECTED to that for positioning techniques specified in Rel-16 and in RRC_INACTIVE to that for positioning techniques specified in Rel-17 in regard to both BW related RRM performance requirements. - RAN4 to align the targeted BW range for both reference signals, SRS for positioning and MIMO SRS, for NR UL CPP to that for positioning techniques specified in Rel-16 in regard to RRM performance requirements.

Issue 2-3-2: PRS/SRS BW configurations for measurement accuracy requirements: < Way forward > - Option 1: - RAN4 to consider the PRS BW configurations used for accuracy requirements in Rel-16 and Rel-17 as baseline for NR CPP accuracy requirements - RAN4 to consider the SRS BW configurations used for accuracy requirements in Rel-16 as baseline for NR UL CPP accuracy requirements for both reference signals, SRS for positioning and MIMO SRS.

RAN4 has not investigated the performance of CP measurements, so we believe it would be too early to agree on re-using the BW ranges from existing measurements. We suggest RAN4 to discuss this issue in detail during the Perf part.

Proposal 9: RAN4 to discuss the BW ranges for defining accuracy requirements for CP measurements in the Perf part.

Issue 2-3-3: Side condition: < Way forward > - Option 1: - SINR side condition for the NR carrier phase measurement can be same as these for Rel16/17 positioning measurement with PRS.

Similarly, RAN4 has not evaluated the accuracy performance of CP measurements, so it is too early to conclude existing conditions can be re-used, especially the -13dB Es/Iot. We suggest RAN4 to discuss this issue in detail during the Perf part.

Proposal 10: RAN4 to discuss the Es/Iot side condition for defining accuracy requirements for CP measurements in the Perf part.

Issue 2-3-5: Report mapping tables: < Way forward > - Option 1: - Introduce report mapping tables for absolute and differential carrier phase measurement, and discuss the following issues to define report mapping tables: - The unit for carrier phase report - The range for carrier phase report - The resolution for carrier phase report - Option 2: - The different report mapping tables shall be defined for RSCP and RSCPD Issue 2-3-6: Reporting range: < Way forward > - Option 1: - For DL RSCP and UL RSCP the reporting range shall be should be [0, 2pi], whereas for DL RSCPD, it should be [-pi, pi]. Issue 2-3-7: Reporting granularity: < Way forward > - Option 1: - For Rel18 CPP measurements, the reporting granularity for DL RSCP, DL RSCPD and UL RSCP should be configurable up to UE capability. - Option 2: - For carrier phase positioning, RAN4 should consider the higher positioning resolution compared to the one for the time-based measurement metric.

Report mapping needs to be defined for all CP measurements (DL RSCP, DL RSCPD, UL RSCP), and in our view, same mapping can be used for all measurements. We suggest to re-use the existing mapping for AOA measurement as baseline except for the resolution. For the resolution, we think 1 degree can be sufficient considering 1 degree already corresponds to quite a small distance.

We are open to further discussion on possible necessary adaptations.

Proposal 11: Define same report mapping for all CP measurements (DL RSCP/RSCPD, UL RSCP). Use the following mapping as baseline, FFS adaptation.

- Unit: degree

- Range: [-pi, +pi)

- Resolution: 1 degree

Conclusions

In this paper we provided our views on RRM requirements for CPP.

Proposal 1: RAN4 to define measurement period requirements for both RSCP and RSCPD. RAN4 to discuss for which measurements to define accuracy in Perf part.

Proposal 2: RAN4 not to define specific requirements for PRU.

Proposal 3: When RSCP or RSCPD is reported together with other measurements, measurement period for the other measurements apply for RSCP or RSCPD.

Proposal 4: RAN4 to discuss the number of samples for accuracy requirements for RSCP and RSCPD, as well as the applicable PRS configuration.

Proposal 5: RAN4 waits for RAN1 conclusion on support of multiple CP measurements for multiple RF frequencies before discussing the impact on RAN4 requirements.

Proposal 6: RF frequencies for target TRP and reference TRP in RSCPD measurement is up to RAN1.

Proposal 7: When RSCP or RSCPD is reported together with other measurements, measurement reporting requirements for other measurements apply for CP measurements.

Proposal 8: RAN4 to consider using same channel model for PRS-RSRPP for defining accuracy for CP measurements.

Proposal 9: RAN4 to discuss the BW ranges for defining accuracy requirements for CP measurements in the Perf part.

Proposal 10: RAN4 to discuss the Es/Iot side condition for defining accuracy requirements for CP measurements in the Perf part.

Proposal 11: Define same report mapping for all CP measurements (DL RSCP/RSCPD, UL RSCP). Use the following mapping as baseline, FFS adaptation.

- Unit: degree

- Range: [-pi, +pi)

- Resolution: 1 degree

Reference

- R4-2306351, WF on RRM requirements for NR sidelink positioning and carrier phase positioning, CATT