3GPP TSG-RAN WG4 Meeting # 107 	    R4-2308791

Incheon, South Korea, 22nd May – 26th May 2023

Agenda Item:	8.23.3.4

Source: 	Ericsson

Title:	On issues related to RedCap positioning

Document for:	Approval

# Introduction

In RAN4#106bis-e companies presented their views on core part requirement and an initial view on performance of RedCap UE positioning. The outcome of the discussion is captured in the WF document. In this contribution we present our view on RedCap positioning and address the FFS items from the last meeting.

# Discussion

## General aspects/scenarios

RAN4#106bis-e general aspects/scenarios for which RAN4 shall define requirements for RedCap UE positioning. As an outcome of the discussion following were agreed:

Issue 2-1-3: Relation with Rel-16/Rel-17 positioning:

Agreements:

- PRS requirements without frequency hopping (FH):

  - Option 1:

    - PRS requirements for both 1Rx and 2Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques.

  - Option 2:

    - PRS requirements for 1Rx RedCap UE without FH shall be defined for all the Rel-16 positioning features/techniques.

    - PRS requirements for 2 Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques.

- PRS requirements with FH:

  - The Rel-16/Rel-17 positioning features/techniques for which the PRS requirements for RedCap UE with FH will be defined can be decided on case-by-case basis and based on RAN1 agreements on the FH. Details are FFS.

- NOTE:

  - Rel-16 features/techniques refer to: measurement with gaps in RRC connected, 4-samples measurement in RRC connected.

  - Rel-17 features/techniques refer to: measurement without gaps in RRC connected, measurement in RRC inactive, reduced-sample measurement in RRC connected/inactive, Tx/Rx timing mitigation with TEG in RRC connected/inactive, FR2 with reduced Rx beam sweeping factor in RRC connected/inactive.

In the agreements above, one of the open questions is whether 1Rx RedCap UE can support enhancements such as reduced number of samples for positioning measurement, PRS measurement without gap, and reduced Rx beam sweeping factor for positioning measurements introduced in Rel. 17. Among the Rel. 17 enhancements reduced Rx beam sweeping factor does not apply to 1Rx RedCap UE as 1Rx capability for RedCap UE is applicable to FR1 and reduced Rx beam sweeping factor capability only applies to FR2. Therefore, reduced Rx beam sweeping factor is out of discussion scope for 1Rx RedCap UE. For reduced number of samples and PRS measurement without gap some applicable conditions were defined in Rel. 17. In our view when those conditions are met, 1Rx RedCap UEs should be able to perform positioning measurements with reduced number of samples and can measure PRS outside of the MG. In this regard, we do not see any technical issue that limits 1Rx RedCap UE from supporting Rel. 17 enhancements for positioning measurements. We therefore propose to define PRS requirements for both 1Rx and 2Rx RedCap UEs without FH for all Rel. 16/Rel. 17 features/techniques.

Proposal 1: PRS requirements for both 1Rx and 2Rx RedCap UEs without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques. For 1Rx RedCap UE reduced Rx beam sweeping factor for positioning measurement is out of scope.

## Issues related to without frequency hopping (FH)

### Performance of 1Rx RedCap UE without FH

RAN4#106bis-e discussed performance requirement of 1Rx RedCap UE without FH and agreed on the following:

# Side conditions for 1Rx without FH:

Agreements:

- Side conditions for 1Rx without FH

  - For AWGN channel, re-use the Rel-17 side conditions, and relax accuracy requirements.

    - The agreement applies for 4 measurement samples case. FFS if it applies for lower number of samples.

  - FFS for fading channel.

    - Option 1: Reuse Rel-17 side conditions and relax accuracy requirements.

    - Option 2: Reuse approximately Rel-17 accuracy requirements and relax the side condition.

Note: Updated simulation assumptions to address open issues are discussed under issue 2-2-8.

# Collection of simulation results: without reduced number of samples for 1Rx without FH:

Agreements:

- Further simulation results based on the updated simulation assumptions (issue 2-2-8) are needed to draw conclusions.

# PRS measurement accuracy: without reduced number of samples for 1Rx without FH:

Agreements:

- PRS measurement accuracy to be discussed and defined under performance part of the WI.

# Updated simulation assumptions: with and without reduced number of samples for 1Rx without FH:

Agreements:

- Updated simulation assumptions to account for at least the open issues identified under issue 2-2-3 are approved in R4-2306350.

# Updated simulation assumptions: with and without reduced number of samples for 1Rx without FH:

Agreements:

- PRS measurement accuracy to be discussed and defined under performance part of the WI.

In our companion paper R4-2308792 simulation results based on updated simulation assumptions for without FH approved in R4-2306350 are reported. The following can be observed in the simulation results:

Observation 1: Under AWGN propagation condition, accuracies of 4 sample RSTD and Rx-Tx measurements performed by 1Rx and 2Rx UEs under Rel. 17 side conditions are comparable to each other. For 1Rx UEs accuracy requirement can be relaxed by 1Tc.

Observation 2: Under AWGN propagation condition, Rel. 17 4 sample RSTD and Rx-Tx measurement accuracy requirements can be met by 1Rx RedCap UE at higher side condition Es/Iot = (-3, -6, -6)dB.

Observation 3: Under fading propagation conditions:

- Rel. 17 4 sample accuracy (RSTD and UE Rx-Tx) cannot be met by 1Rx RedCap UE under Rel. 17 side condition.

  - Performance degradation is more pronounced for smaller number of PRBs.

- Accuracies of 4 sample RSTD and Rx-Tx measurements performed by 1Rx and 2Rx UEs under Rel. 17 side conditions are comparable to each other for larger BWs.

  - Relaxed accuracy requirement can be defined only for BWs more than 104 PRBs for 15kHz SCS.

- Rel. 17 4 sample accuracy (RSTD and UE Rx-Tx) cannot be met by 1Rx RedCap UE under higher side condition, i.e., Es/Iot = (-3, -6, -6)dB and Es/Iot = (0, -6, -6)dB.

  - Performance degradation is more pronounced for smaller number of PRBs.

- Accuracies of 4 sample RSTD and Rx-Tx measurements performed by 1Rx under higher side condition and 2Rx UEs under Rel. 17 side conditions are comparable to each other for larger BWs.

  - Relaxed accuracy requirement can be defined only for BWs more than 104 PRBs for 15kHz SCS.

Proposal 2: For AWGN channel condition, consider the following options to define accuracy requirements for 1Rx RedCap UE. Our preference is option 2 below.

- Option 1: Reuse rel. 17 4 sample RSTD and UE Rx-Tx accuracy requirement defined for AWGN channel model to define accuracy requirement for 4 sample RSTD and UE Rx-Tx accuracy for 1Rx RedCap UE for AWGN channel model under higher side condition, i.e., Es/Iot = (-3, -6, -6)dB, for without FH case.

- Option 2: Accuracy requirement for 4 sample RSTD and UE Rx-Tx measurements are defined by reusing Rel. 17 side conditions for without FH case. The 4 sample RSTD and UE Rx-Tx accuracy requirement can be relaxed by 1Tc in comparison to Rel. 17 4 sample RSTD and UE Rx-Tx accuracy requirement.

Proposal 3: For fading channel conditions, consider the following options to define accuracy requirements for 1Rx RedCap UE. Our preference is option 2 below.

- Option 1: For fading propagation condition, define 4 sample accuracy requirements for 15kHz (104 PRBs), 30kHz (24/48 PRBs), and 60kHz (24 PRBs) under higher side condition, i.e., Es/Iot = (-3, -6, -6)dB, for without FH case. The accuracy requirement can be relaxed in comparison to Rel. 17 accuracy requirement.

- Option 2: Do not define accuracy requirement for 1Rx RedCap UE applicable to fading propagation conditions.

### Core requirement for without FH case

In the last meeting, following agreements were reached on core requirement for PRS measurement when UE does not perform FH to receive DL PRS resources.

# Applicable RRC states:

Agreements:

- RedCap UE positioning without frequency hopping

  - Define the requirements in RRC_CONNECTED and RRC_INACTIVE states.

- RedCap UE positioning with frequency hopping

  - Define the requirements in RRC_CONNECTED state

  - FFS for requirements in RRC_INACTIVE state.

- FFS whether to define requirements in RRC_IDLE state

# PRS measurement requirements for 2Rx without FH:

Agreements:

- Existing PRS measurement period requirements in Rel-17 can be reused for defining corresponding PRS measurement period requirements for 2Rx RedCap UE without FH.

  - Details related to e.g., CSSFPRS,i , Kcarrier_PRS, TEG etc., are FFS.

# PRS measurement requirements for 1Rx without FH:

Agreements:

- Existing PRS measurement period requirements in Rel-17 can be reused for defining corresponding PRS measurement period requirements for 1Rx RedCap UE without FH.

  - Details related to e.g., CSSFPRS,i , Kcarrier_PRS, TEG etc., are FFS.

Observation 4: PRS measurement requirement will be defined for RRC_CONNECTED and RRC_INACTIVE states for RedCap UE positioning without frequency hopping.

Proposal 4: For positioning measurement core requirement for RedCap UEs without FH in RRC_INACTIVE state, Kcarrier_PRS in Rel. 17 RRC_INACTIVE core requirement is updated to Kcarrier_PRS_RedCap. Kcarrier_PRS_RedCap = 1 if RedCap UE is able to perform RRM and PRS measurements in parallel to each other, otherwise Kcarrier_PRS_RedCap is defined as:

- If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS_RedCap=Kcarrier_RedCap+1, where Kcarrier_RedCap is defined in clause 4.2B.2.4.

- If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS_RedCap=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

Proposal 5: For positioning measurement core requirement for RedCap UEs without FH in RRC_CONNECTED state, CSSFPRS,i in Rel. 17 RRC_CONNECTED core requirement is updated to CSSFPRS,RedCap,i. The value of CSSFPRS,RedCap,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in positioning frequency layer i as defined in clause 9.1A.5.2.

Proposal 6: Rel. 17 TEG framework is supported for RedCap positioning.

## Issues related to FH

In the last meeting it was agreed to wait for further progress to be made in RAN1 and issue related to RF switching time between hops in RF group is concluded before starting RAN4 work on RRM requirements that are applicable to FH case.

In our view RAN1 progress is still not enough for RAN4 to start working on RRM requirements. Besides, RF group has considered multiple values as a starting point for RF switching time between hops discussion and the conclusion if it is going to be a single value or multiple values is yet to be concluded. We therefore see to uphold agreements reached in the last meeting and wait for further progress in RAN1 and RF group discussion.

Proposal 7: RAN4 to uphold agreements from the last meeting and wait for further progress to be made in RAN1 and RF group discussion on RF switching time between hops.

# Summary

Discussion presented in this paper can be summarised in terms of following observations and proposals:

Observation 1: Under AWGN propagation condition, accuracies of 4 sample RSTD and Rx-Tx measurements performed by 1Rx and 2Rx UEs under Rel. 17 side conditions are comparable to each other. For 1Rx UEs accuracy requirement can be relaxed by 1Tc.

Observation 2: Under AWGN propagation condition, Rel. 17 4 sample RSTD and Rx-Tx measurement accuracy requirements can be met by 1Rx RedCap UE at higher side condition Es/Iot = (-3, -6, -6)dB.

Observation 3: Under fading propagation conditions:

- Rel. 17 4 sample accuracy (RSTD and UE Rx-Tx) cannot be met by 1Rx RedCap UE under Rel. 17 side condition.

  - Performance degradation is more pronounced for smaller number of PRBs.

- Accuracies of 4 sample RSTD and Rx-Tx measurements performed by 1Rx and 2Rx UEs under Rel. 17 side conditions are comparable to each other for larger BWs.

  - Relaxed accuracy requirement can be defined only for BWs more than 104 PRBs for 15kHz SCS.

- Rel. 17 4 sample accuracy (RSTD and UE Rx-Tx) cannot be met by 1Rx RedCap UE under higher side condition, i.e., Es/Iot = (-3, -6, -6)dB and Es/Iot = (0, -6, -6)dB.

  - Performance degradation is more pronounced for smaller number of PRBs.

- Accuracies of 4 sample RSTD and Rx-Tx measurements performed by 1Rx under higher side condition and 2Rx UEs under Rel. 17 side conditions are comparable to each other for larger BWs.

  - Relaxed accuracy requirement can be defined only for BWs more than 104 PRBs for 15kHz SCS.

Observation 4: PRS measurement requirement will be defined for RRC_CONNECTED and RRC_INACTIVE states for RedCap UE positioning without frequency hopping.

Proposal 1: PRS requirements for both 1Rx and 2Rx RedCap UEs without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques. For 1Rx RedCap UE reduced Rx beam sweeping factor for positioning measurement is out of scope.

Proposal 2: For AWGN channel condition, consider the following options to define accuracy requirements for 1Rx RedCap UE. Our preference is option 2 below.

- Option 1: Reuse rel. 17 4 sample RSTD and UE Rx-Tx accuracy requirement defined for AWGN channel model to define accuracy requirement for 4 sample RSTD and UE Rx-Tx accuracy for 1Rx RedCap UE for AWGN channel model under higher side condition, i.e., Es/Iot = (-3, -6, -6)dB, for without FH case.

- Option 2: Accuracy requirement for 4 sample RSTD and UE Rx-Tx measurements are defined by reusing Rel. 17 side conditions for without FH case. The 4 sample RSTD and UE Rx-Tx accuracy requirement can be relaxed by 1Tc in comparison to Rel. 17 4 sample RSTD and UE Rx-Tx accuracy requirement.

Proposal 3: For fading channel conditions, consider the following options to define accuracy requirements for 1Rx RedCap UE. Our preference is option 2 below.

- Option 1: For fading propagation condition, define 4 sample accuracy requirements for 15kHz (104 PRBs), 30kHz (24/48 PRBs), and 60kHz (24 PRBs) under higher side condition, i.e., Es/Iot = (-3, -6, -6)dB, for without FH case. The accuracy requirement can be relaxed in comparison to Rel. 17 accuracy requirement.

- Option 2: Do not define accuracy requirement for 1Rx RedCap UE applicable to fading propagation conditions.

Proposal 4: For positioning measurement core requirement for RedCap UEs without FH in RRC_INACTIVE state, Kcarrier_PRS in Rel. 17 RRC_INACTIVE core requirement is updated to Kcarrier_PRS_RedCap. Kcarrier_PRS_RedCap = 1 if RedCap UE is able to perform RRM and PRS measurements in parallel to each other, otherwise Kcarrier_PRS_RedCap is defined as:

- If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS_RedCap=Kcarrier_RedCap+1, where Kcarrier_RedCap is defined in clause 4.2B.2.4.

- If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS_RedCap=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

Proposal 5: For positioning measurement core requirement for RedCap UEs without FH in RRC_CONNECTED state, CSSFPRS,i in Rel. 17 RRC_CONNECTED core requirement is updated to CSSFPRS,RedCap,i. The value of CSSFPRS,RedCap,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in positioning frequency layer i as defined in clause 9.1A.5.2.

Proposal 6: Rel. 17 TEG framework is supported for RedCap positioning.

Proposal 7: RAN4 to uphold agreements from the last meeting and wait for further progress to be made in RAN1 and RF group discussion on RF switching time between hops.

# References

- R4-2306350, “Updated simulation assumptions for 1Rx RedCap UE PRS measurement without FH”, Ericsson.

- R4-2306349, “WF on RRM requirements for RedCap positioning and PRS/SRS BW aggregation”, Ericsson.

- R4-2308792, “Simulation results for 1Rx RedCap”, Ericsson.