3GPP TSG-RAN WG4 Meeting # 107 	    R4-2308795

Incheon, South Korea, 22nd – 26th May, 2023

Agenda Item:	8.23.3.6

Source: 	Ericsson

Title:	On carrier phase positioning requirements

Document for:	Approval

# Introduction

RAN4#106bis-e started detailed discussion on RAN4 issues related to CPP. In the contribution we present our view on open issues from the last meeting.

# Discussion

# Measurement types to be defined

RAN1#112bis-e agreed to introduce DL RSCPD and DL RSCP measurements as DL carrier phase measurements. In addition, RAN1 agreed DL RSCPD to be reported together with RSTD measurement and the DL RSCP measurement to be reported together with UE Rx-Tx measurement.

Proposal 1: RAN4 to define measurement delay and accuracy requirements for DL RSCPD and DL RSCP measurements. Accuracy requirement is further discussed during the performance part of the WI.

# Measurement delay requirement

RAN1#112bis-e made the following agreements:

Agreement (RAN1#112bis-e)
Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including

- UE in RRC_CONNECTED state with measurement gap.

- FFS: UE in RRC_CONNECTED state without measurement gap

- UE in RRC_ INACTIVE state

In the last meeting RAN4#106bis-e agreed to define core requirements for CPP in RRC_CONNECTED and RRC_INACTIVE states. According to WID, the following objectives shall be achieved to close the Rel. 18 WI.

- Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4].

  - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements.

  - Specify measurements that are limited to a single carrier/PFL.

  - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements with measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

Proposal 2: RAN4 to not define measurement delay requirements for the case when CPP measurement is performed by UE outside of the MG or within PPW in RRC_CONNECTED state.

Based on RAN1 agreement DL RSCPD is to be reported together with legacy RSTD and RSCP measurement is to be reported together with legacy UE Rx-Tx measurement, the measurement delay requirements for RSTD and UE Rx-Tx measurement shall also apply when UE is configured to perform and report DL RSCPD and DL RSCP measurements together with legacy RSTD and UE Rx-Tx measurements.

Proposal 3: Measurement delay requirements for RSTD and UE Rx-Tx measurements apply when UE is configured to perform and report DL RSCPD and DL RSCP measurements together with legacy RSTD and UE Rx-Tx measurements.

# Applicable RF frequency

RAN4#106bis-e discussed RF frequency applicable to CPP measurement and agreed on the WF below.

Applicable RF frequencies for DL RSCP measurement period requirements:

< Way forward >

The basic measurement requirement for DL RSCP is defined for each RF frequency. And RAN4 to discuss whether to define measurement requirements for multiple RF frequencies.

Applicable RF frequencies for DL RSCPD measurement period requirements:

< Way forward >

The RF frequencies for target TRP and reference TRP in RSCPD measurement can be different.

Some relevant discussion also took place in RAN1#112bis-e. The agreement reached in RAN1 is copied below for reference.

Agreement (RAN1#112bis-e)
The specific RF frequency associated with a DL carrier phase measurement is defined as the centre frequency of the DL PFL by default.

- Note: It is open to further discussion whether a frequency other than the centre frequency of the DL PFL can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them.

Based on RAN1 agreement, CPP measurement is defined on resource level. From the measurement delay point of it is our understanding that RAN4 requirements for CPP is defined for a single RF frequency where single RF frequency associated with a DL CPP measurement corresponds to the centre frequency of the DL PFL. Based on this definition of RF frequency RSCPD measurement performed on different PFLs.

Proposal 4: RAN4 to define requirements for the case, where centre frequency of the DL PFL is the specific RF frequency associated with a DL carrier phase measurement.

Proposal 5: RAN4 to not define measurement requirements for DL carrier phase measurement for multiple RF frequencies.

Proposal 6: For RSCPD measurement RF frequencies for target TRP and reference TRP should belong to same PFL.

Proposal 7: When RSCPD measurement is reported together with RSTD measurement, then RSTD measurement is performed on the same target TRP and reference TRP on which RSCPD measurement is performed.

# Performance requirement

RAN4#106bis-e started preliminary discussions on issues related to CPP performance requirement. Based on the discussion following issues were identified FFS.

PRS/SRS BW configurations for measurement accuracy requirements:

< Way forward >

Option 1:

RAN4 to consider the PRS BW configurations used for accuracy requirements in Rel-16 and Rel-17 as baseline for NR CPP accuracy requirements

RAN4 to consider the SRS BW configurations used for accuracy requirements in Rel-16 as baseline for NR UL CPP accuracy requirements for both reference signals, SRS for positioning and MIMO SRS.

One of the open questions relating to CPP performance requirement is the PRS and SRS bandwidths for which RAN4 should define requirements for. As agreed by RAN1, RSCPD/RSCP measurement is reported together with RSTD/UE Rx-Tx measurement. When a UE is configured to report CPP and legacy positioning measurements together, UE is expected to perform measurements on same resource/PFL. It is therefore our view that PRS BW configurations used to define RSTD/UE Rx-Tx measurement shall be used to define accuracy requirement for corresponding CPP measurement.

Proposal 8: DL RSCPD and DL RSCP accuracy requirements are defined by considering PRS bandwidths used to define legacy RSTD and UE Rx-Tx accuracy requirements, respectively.

The second issue listed in WF above needs to be addressed after there is an agreement on if it is necessary to define accuracy requirement for UL CPP measurement, which was agreed FFS in the last meeting.

Measurement types to be defined for UL carrier phase measurement requirements:

Agreements:

RAN4 not to define measurement period requirements for UL RSCP measurement and FFS for accuracy requirements.

In our understanding accuracy requirement for UL CPP measurement is not within the scope and therefore similar to core requirement, we do not see a need to define accuracy requirement for UL CPP measurement or at least down prioritize accuracy requirement for UL CPP measurement discussion.

Proposal 9: Deprioritize accuracy requirement discussion for UL CPP measurement.

Side condition:

< Way forward >

Option 1:

SINR side condition for the NR carrier phase measurement can be same as these for Rel16/17 positioning measurement with PRS.

As mentioned in one of the earlier sections, it is our understanding that the UE same resource/PFL is used for both legacy positioning and CPP measurement. It therefore makes sense to define CPP accuracy requirement under the same side condition for which legacy measurement accuracy is defined for.

Proposal 10: SINR side condition for the NR carrier phase measurement can be same as these for Rel.16/17 positioning measurement with PRS.

Measurement reporting requirements:

< Way forward >

Option 1:

RAN4 to investigate and specify measurement reporting for NR DL CPP based on DL measurement types specified in TS 38.215.

RAN4 to investigate and specify measurement reporting requirements for NR UL CPP based on UL measurement types specified in TS 38.215

RAN4 to specify joint measurement reporting requirements for the use case scenario of coexistent NR CPP and legacy NR positioning techniques.

RAN4 to await further RAN1 progress on additional measurement reporting and assistance information to improve the measurement accuracy.

Report mapping tables:

< Way forward >

Option 1:

Introduce report mapping tables for absolute and differential carrier phase measurement, and discuss the following issues to define report mapping tables:

The unit for carrier phase report

The range for carrier phase report

The resolution for carrier phase report

Option 2:

The different report mapping tables shall be defined for RSCP and RSCPD

Reporting range:

< Way forward >

Option 1:

For DL RSCP and UL RSCP the reporting range shall be should be [0, 2pi], whereas for DL RSCPD, it should be [-pi, pi].

Reporting granularity:

< Way forward >

Option 1:

For Rel18 CPP measurements, the reporting granularity for DL RSCP, DL RSCPD and UL RSCP should be configurable up to UE capability.

Option 2:

For carrier phase positioning, RAN4 should consider the higher positioning resolution compared to the one for the time-based measurement metric.

CPP is a new measurement introduced in Rel. 18. In this sense, RAN4 should define applicable measurement reporting for at least DL RSCPD and DL RSCP measurements. For both measurements, unit, range, and the resolution for report needs to be specified. To achieve higher positioning accuracy than the legacy measurement, CPP measurement resolution can be higher than the resolution defined for legacy positioning measurements.

Proposal 11: RAN4 to specify measurement reporting requirements for DL RSCPD and DL RSCP measurements.

Proposal 12: Reporting resolution for carrier phase measurement is 0.002 radians.

Proposal 13: Reporting range for DL RSCP measurement is [0, 2𝜋].

Proposal 14: Reporting range for DL RSCPD measurement is [-𝜋, 𝜋].

# Summary

Open issues from RAN4#106bis-e are addressed in this contribution. Based on the presented discussion following are proposed in this paper.

Proposal 1: RAN4 to define measurement delay and accuracy requirements for DL RSCPD and DL RSCP measurements. Accuracy requirement is further discussed during the performance part of the WI.

Proposal 2: RAN4 to not define measurement delay requirements for the case when CPP measurement is performed by UE outside of the MG or within PPW in RRC_CONNECTED state.

Proposal 3: Measurement delay requirements for RSTD and UE Rx-Tx measurements apply when UE is configured to perform and report DL RSCPD and DL RSCP measurements together with legacy RSTD and UE Rx-Tx measurements.

Proposal 4: RAN4 to define requirements for the case, where centre frequency of the DL PFL is the specific RF frequency associated with a DL carrier phase measurement.

Proposal 5: RAN4 to not define measurement requirements for DL carrier phase measurement for multiple RF frequencies.

Proposal 6: For RSCPD measurement RF frequencies for target TRP and reference TRP should belong to same PFL.

Proposal 7: When RSCPD measurement is reported together with RSTD measurement, then RSTD measurement is performed on the same target TRP and reference TRP on which RSCPD measurement is performed.

Proposal 8: DL RSCPD and DL RSCP accuracy requirements are defined by considering PRS bandwidths used to define legacy RSTD and UE Rx-Tx accuracy requirements, respectively.

Proposal 9: Deprioritize accuracy requirement discussion for UL CPP measurement.

Proposal 10: SINR side condition for the NR carrier phase measurement can be same as these for Rel.16/17 positioning measurement with PRS.

Proposal 11: RAN4 to specify measurement reporting requirements for DL RSCPD and DL RSCP measurements.

Proposal 12: Reporting resolution for carrier phase measurement is 0.002 radians.

Proposal 13: Reporting range for DL RSCP measurement is [0, 2𝜋].

Proposal 14: Reporting range for DL RSCPD measurement is [-𝜋, 𝜋].

# References

- R4-2306351, “WF on RRM requirements for NR sidelink positioning and carrier phase positioning”, CATT.

- R1-2303890, “FL Summary #3 for NR DL and UL carrier phase positioning”, CATT.