3GPP TSG-RAN WG4 Meeting #107	R4-2308977

Incheon, KR, May 22 – May 26, 2023

Title:	Discussion on TPMI configuration for one-layer UL MIMO TRP test

Source:	OPPO

Agenda item:	8.16.2.1

Document for:	Approval

- Introduction

The proper TPMI-index configuration for one-layer UL-MIMO TRP test was warmly discussed in the past few RAN4 meetings. In the last RAN4 meeting #106bis-e, four options were listed for further down-selection. [1]

This contribution presents our considerations and proposals on down-selection of TPMI-index configuration for one-layer UL-MIMO TRP test.

- Discussion

There are four options for TPMI-index configuration. And as we discussed in the past meetings, the Option 4 is spherical coverage concept for radiated performance evaluation, rather than TRP concept. Therefore, Option 4 should be excluded firstly because of out of WI scope.

Regarding Option 1/2/3, Option 2 is quite different with the other two options. Although Option 2 which selects the best TPMI in every measurement grid probably fits the UE behavior in the real network, this kind of metrics do not comply with the traditional TRP concept, because the EIRP measured on each measurement grid may belong to different antenna pattern. As long as the concept of TRP does not modified in RAN4, Option 2 is not in the scope of WID.

Comparing Option 1 and Option 3 (including 3a), Option 1 is the TRP with one fixed TPMI index, which clearly conform to the traditional TRP concept. While Option 3 is averaging of several TRP with different TPMI index. The positive value of Option 3 is eliminating the unwanted random errors when measuring single TRP with one fixed TPMI index, as elaborated in the contribution [2]. From this point of view, Option 3/3a is better than Option 1. The main concern is the test time increasing of Option 3 with multiple TPMI indexes measured. From the TE vendors’ response in the last RAN4 meeting, the TPMI can be flexible modified when performing EIRP measurement. Thus, the test time of Option 3 will not significantly increase compared with Option 1.

To summary up, Option 3/3a is the preferable option for one-layer UL MIMO TRP test.

Proposal 1: Option 3/3a is the preferable option for one-layer UL MIMO TRP test.

However, it should be noted that not all the UE can support TPMI index from 2 to 5. For non-coherent UE supporting uplink full power transmission, only TPMI index 2 can be supported. So Option 1 is the only choice for this kind of UE.

Proposal 2: Option 1 is the only choice for non-coherent UE supporting uplink full power transmission.

- Conclusions

This paper presents our our considerations and proposals on down-selection of TPMI-index configuration for one-layer UL-MIMO TRP test. And provide the following proposal.

Proposal 1: Option 3/3a is the preferable option for one-layer UL MIMO TRP test.

Proposal 2: Option 1 is the only choice for non-coherent UE supporting uplink full power transmission.

- References

[1] R4-2305904, WF for Rel-18 TRP TRS requirements

[2] R4-2305701 Further discussion on FR1 UL MIMO 2TX TRP - r5