3GPP TSG-RAN WG4 Meeting #107	R4-2309125

Incheon KR, May 22 – 26, 2023

Agenda item:	8.10.2.3

Source: 	Qualcomm Incorporated

Title: 	On joint requirements for Rel-17 measurement gap enhancements – Case 2 (NCSG + Type-2 MG)

Document for:	Discussion

# Introduction

RAN4 continued discussing joint RRM requirements for Rel-17 concurrent measurement gaps (MG), pre-configured measurement gaps and NCSG in RAN4#106bis-e. Further agreements and open issues were captured in a WF [1].

In this paper, we provide our views and proposals on requirements for Case 2 in the WID [2].

# Discussion

## Gap combinations

The first issue for Case 2 is whether to support two NCSGs simultaneously in the same FR. In our view, two NCSGs in the same FR would not be a typical configuration. It could be supported optionally with UE capability.

Issue 4-2-5: [Case 2] Whether to consider a new capability for NCSG + NCSG in an FR

< Way forward >

- FFS the following options

  - Option 1a:

    - No, without UE capability if two NCSG patterns are supported based on single RF chain.

  - Option 2:

    - Yes, with UE capability

  - Option 2a:

    - Yes, with UE capability if two RF chains are in use to support NCSG+NCSG, we are fine to introduce new UE capability

  - Option 3:

    - Postpone the discussion on new capability for NCSG + NCSG until RAN4 has a consensus on parallel measurement.

Proposal 1: Two NCSGs in the same FR can be supported with UE capability.

In addition, a separate UE capability should be added to signal support of NCSG + Type2 MG in Case 2.

Proposal 2: Introduce two separate UE capabilities to indicate support of NCSG + Type-2 MG and NCSG + NCSG in Case 2.

## Parallel measurements

Once again, in RAN4#106bis-e there was discussion about defining requirements for parallel measurements with two NCSGs configured in the same FR [1]. Options under consideration are listed below.

### Issue 4-2-1: [Case 2] Whether to consider parallel measurements upon gap collision

< Agreements from GTW >

- Option 1: Support of parallel measurements upon NCSGs collision is up to UE capability

- Option 2: Do not support parallel measurements upon NCSGs collision

In our view, the topic of parallel measurements is beyond the scope of this work item. The proponents of this enhancements are asking the UE to perform multiple measurements simultaneously within two fully overlapping gaps. i.e. effectively within a single gap occasion. In that sense, this would be an enhancement to measurements performed with a single NCSG, which is not in scope of this Rel-18 WI.

The enhancement proposed above is not compatible with prior RAN4 agreements for Case 2. RAN4 has agreed already to leverage the priority rule and proximity condition framework developed in Rel-17 [3]. Therefore, when two NCSGs are configured together, two adjacent NCSG occasions will be separated in time by more than 4 ms (otherwise, one of them will be dropped). Then, there is no possibility to perform parallel measurements with NCSG + NCSG unless the parallel measurements are performed in one of the NCSGs. Again, such an enhancement is clearly not in scope of this WI.

Observation 1: According to RAN4 agreements, if two NCSG are configured in the same FR and gap occasions of the two NCSG overlap in time, the occasion of the NCSG with the higher priority is preserved and the other one is dropped. i.e. only one NCSG occasion is kept.

Proposal 3: Do not support parallel measurements with colliding NCSG occasions.

RAN4 has already reached agreements on how to address collisions for Case 2. It is not necessary to consider any further enhancements regarding collision definition and/or resolution.

Issue 4-2-4: [Case 2] Potential changes to UE behaviour upon gap collision

< Wayforward >:

- RAN4 to postpone the gap collision rule changes’ discussion until RAN4 has a consensus on parallel measurement scenarios.

Proposal 4: RAN4 not to consider changes to collision definition and resolution for Case 2.

## Other

The last issue for Case 2 considers a clarification of deactivated SCell measurement requirements with NCSG. In our view, this is a Rel-17 maintenance issue, not specific to Case 2. Nevertheless, we shall discuss it here.

### Issue 4-4-1: [Case 2] Potential changes for NCSG upon SCell activation

< Agreements from GTW >:

- UE behavior for deactivated SCell measurements with NCSG in Case 2 is FFS

  - Option 1: Legacy UE behavior (i.e. UE measures the deactivated SCell outside of MG)

  - Option 2: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped.

NR intra-frequency measurement requirements with NCSG in 38.133, clause 9.2.7 include the note below. According to this note, a deactivated SCell is measured within NCSG whenerver the SCell’s SMTC overlaps at least partially with the NCSG. i.e. at least some SMTC occasions overlap with NCSG occasions.

Note: 	Requirements for measurement on deactivated SCC in this clause do not apply if SMTC on the deactivated SCC is fully non-overlapped with NCSG, and the requirements for measurement on deactivated SCC specified in clause 9.2.5 apply.

It makes sense that a deactivated SCell is measured within NCSG even when the SMTC doesn’t fully overlap with the NCSG. That is because the measurement periodicity for deactivated SCells is controlled by measCycleSCell, which is always larger than or equal to the VIRP of the NCSG. Even if there are SMTC occasions outside the NCSG, the UE is not expected to measure them anyway. As long as there is at least one STMC occasion within NCSG every measCycleScell, the SCell can be measured with NCSG according to the requirement.

measCycleSCell                      ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}

measCycleSCell

The parameter is used only when an SCell is configured on the frequency indicated by the measObjectNR and is in deactivated state, see TS 38.133 [14]. gNB configures the parameter whenever an SCell is configured on the frequency indicated by the measObjectNR, but the field may also be signalled when an SCell is not configured. Value sf160 corresponds to 160 sub-frames, value sf256 corresponds to 256 sub-frames and so on.

Observation 2: In Rel-17, measurement requirements of deactivated SCells within NCSG apply whenever the SMTC partially overlaps with the NCSG.

Proposal 5: Clarification of the Rel-17 requirement: a deactivated SCell is measured within NCSG if the SMTC is partially or fully overlapped with the NCSG.

# Conclusions

Proposal 1: Two NCSGs in the same FR can be supported with UE capability.

Proposal 2: Introduce two separate UE capabilities to indicate support of NCSG + Type-2 MG and NCSG + NCSG in Case 2.

Observation 1: According to RAN4 agreements, if two NCSG are configured in the same FR and gap occasions of the two NCSG overlap in time, the occasion of the NCSG with the higher priority is preserved and the other one is dropped. i.e. only one NCSG occasion is kept.

Proposal 3: Do not support parallel measurements with colliding NCSG occasions.

Proposal 4: RAN4 not to consider changes to collision definition and resolution for Case 2.

Observation 2: In Rel-17, measurement requirements of deactivated SCells within NCSG apply whenever the SMTC partially overlaps with the NCSG.

Proposal 5: Clarification of the Rel-17 requirement: a deactivated SCell is measured within NCSG if the SMTC is partially or fully overlapped with the NCSG.

# References

[1] R4-2306330, WF on Rel-18 NR MG enhancements, RAN4#106bis-e

[2] RP-222337, Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, RAN#97-e.

[3] R4-2214346, WF on further enhancements on measurement gaps and measurements without gaps, RAN4#104-e.