3GPP TSG-RAN WG4 Meeting #107	R4-2309133

Incheon KR, May 22 – 26, 2023

Agenda item:	8.23.3.4

Source: 	Qualcomm Incorporated

Title: 	On requirements for RedCap positioning

Document for:	Discussion

# Introduction

RAN4 continued discussing requirements for RedCap positioning in RAN4#106bis-e. The latest agreements and open issues were captured in a WF [1].

In this paper we provide further views and proposals on general aspects, requirements for measurements without frequency hopping, and requirements for measurements with frequency hopping.

# Discussion

## General

In the previous meeting, RAN4 discussed the applicable RRC states in which requirements for RedCap positioning will be defined in Rel-18. An initial agreement, shown below, left as FFS the definition of requirements in RRC_IDLE. However, a subsequent RAN4 agreement under the LPHAP topic (issue 1-3-1), asserted that requirements for RedCap UEs will be defined in RRC_IDLE. Therefore, we understand that at least requirements for measurements without hopping will be defined in RRC_IDLE.

Issue 2-1-2: Applicable RRC states:

Agreements:

- RedCap UE positioning without frequency hopping

  - Define the requirements in RRC_CONNECTED and RRC_INACTIVE states.

- RedCap UE positioning with frequency hopping

  - Define the requirements in RRC_CONNECTED state

  - FFS for requirements in RRC_INACTIVE state.

- FFS whether to define requirements in RRC_IDLE state

Issue 1-3-1: New PRS measurement requirements

- Agreement (from GTW)

  - Define requirements for normal (non-RedCap) and RedCap type of devices

  - Requirements for PRS measurement in INACTIVE, both with and without eDRX, are re-used for PRS measurement in IDLE.

Observation 1: RAN4 agreed to define requirements for RedCap positioning in RRC_IDLE state.

Proposal 1:

- Define requirements for RedCap positioning without frequency hopping in RRC_IDLE state

- FFS define requirements for RedCap positioning with frequency hopping in RRC_IDLE state

RAN4 also discussed which NR positioning features would be covered by the RedCap requirements to be defined in Rel-18. The initial agreement is shown below.

Issue 2-1-3: Relation with Rel-16/Rel-17 positioning:

Agreements:

- PRS requirements without frequency hopping (FH):

  - Option 1: HW, E///, QC, CATT, OPPO, Nokia

    - PRS requirements for both 1Rx and 2Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques.

  - Option 2: MTK

    - PRS requirements for 1Rx RedCap UE without FH shall be defined for all the Rel-16 positioning features/techniques.

    - PRS requirements for 2 Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques.

- PRS requirements with FH:

  - The Rel-16/Rel-17 positioning features/techniques for which the PRS requirements for RedCap UE with FH will be defined can be decided on case-by-case basis and based on RAN1 agreements on the FH. Details are FFS.

- NOTE:

  - Rel-16 features/techniques refer to: measurement with gaps in RRC connected, 4-samples measurement in RRC connected.

  - Rel-17 features/techniques refer to: measurement without gaps in RRC connected, measurement in RRC inactive, reduced-sample measurement in RRC connected/inactive, Tx/Rx timing mitigation with TEG in RRC connected/inactive, FR2 with reduced Rx beam sweeping factor in RRC connected/inactive.

In our view, there is no strong reason to exclude any of the Rel-17 NR positioning enhancements from the requirements for RedCap UEs. All of the Rel-17 enhancements are supported via corresponding UE capabilities. OEMs can differentiate their RedCap offerings by choosing to support all or a subset of the Rel-17 features. In addition, the RAN4 specification work to extend the associated requirements to RedCap devices should be moderate.

Proposal 2:  PRS requirements for both 1Rx and 2Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques.

Regarding requirements with frequency hopping, our assumption is that Rel-16 feature set would be the baseline. RAN4 can further discuss which Rel-17 features can be considered as RAN1 and RAN4 make more progress.

Proposal 3: PRS requirements for both 1Rx and 2Rx RedCap UE with FH will be defined for all the Rel-16 positioning features. FFS which Rel-17 positioning features will be covered by the requirements with FH.

## Requirements with frequency hopping

RAN1 reached further agreements on RedCap positioning measurements with frequency hopping in RAN4#112bis-e [5]. The latest agreements are reproduced below for convenience.

Conclusion

For the positioning of redcap UEs, for the DL PRS reception and UL SRS transmission, the maximum hopping bandwidth for a single hop is 20MHz for FR1 and 100MHz with FR2.

Agreement

For RedCap UEs, SRS for positioning Tx frequency hopping is configured within one SRS for positioning resource.

Agreement

For DL Rx hopping or UL Tx hopping, support the UE or gNB to report the following:

- A single measurement based on receiving multiple hops of the DL PRS or UL SRS for positioning

- One [or more] measurements where each measurement is associated with one received hop

- FFS: indication of how many received hops / which received hops where used in the measurement report.

- Note: no new measurement definition is introduced in RAN1

- FFS: conditions when the above measurements are reported, and whether the above measurements can be reported together

Agreement

For UL SRS Tx hopping, the frequency hopping pattern is configured with overlapping or non-overlapping hops.

- FFS: exact patterns to be supported

- FFS: whether the overlapping hops may or may not be adjacent in the time domain

- Note: RAN1 assumes that no additional UE requirements shall be specified for the case of Tx hopping with non-overlapping hops compared to the case of Tx hopping with overlapping hops, e.g., a UE is not responsible for keeping phase continuity across the hops in either case of overlapping or non-overlapping hops.

Agreement

For RedCap UEs positioning transmitting the UL SRS with frequency hopping, regarding the collisions between other UL and DL signals/channels and the UL SRS with frequency hopping, study whether to support one or both of the following options, according to UE capabilities:

- Option 1: UL time window where the UE is not expected to receive/transmit other signals/channels and is only expected to transmit FH SRS for positioning.

  - FFS details of an UL time window

  - Note: it implies that UE drops the transmission of other signals/channels and transmits SRS for positioning

- Option 2: additional collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels

  - FFS: details on the collision rules

It is clear from the above agreements that there are still many pending issues/questions in RAN4 regarding this topic. E.g. it was agreed by RAN1 to support one measurement based on multiple hops of a DL PRS resource and also one or more measurements each associated with one received hop. In the first case, it is not clear whether the assumption is that the single multi-hop measurement needs to cover the entire bandwidth of the PRS resource. This is a key issue for requirements applicability and it would need to be addressed by RAN4 if not addressed in RAN1. Additionally, since a UE could report either a multi-hop or a single-hop measurement, there must be a way to signal which type of measurement is being reported.

Note also that RAN1 agreements on new UE capabilities to support positioning measurements with FH are still pending.

RAN4 agreed in RAN4#106bis-e to wait for further progress in RAN1 before starting to work on core requirements for RedCap positioning measurements with FH. In our view, more progress in RAN1 is still needed before starting the work in RAN4.

Issue 2-3-1: When to start PRS measurements for RedCap with FH?

Agreements:

- RAN4 to start work on PRS requirements for RedCap with frequency hopping after RAN1 has made agreements on frequency hopping for RedCap.

Proposal 4: RAN4 to wait for more progress in RAN1 before starting to work on detailed PRS measurement period requirements with frequency hopping for RedCap UEs.

Given that RAN1 has already agreed to support DL PRS measurements with Rx frequency hopping within measurements gaps [7], RAN4 can start discussing at least some aspects of those requirements.

For instance, one aspect that RAN4 should start to discuss relates to the RF retuning time between hops. RAN4 reached an initial agreement on the feasible values of retuning time between hops for FR1 and FR2 in RAN4#106bis-e [6]. The feasible values agreed by RAN4 range from a minimum of two (four) symbols for FR1 (FR2) to a maximum of four (16) symbols in FR1 (FR2).

Agreement:

- For DL PRS Rx frequency hopping, use the switching time of {70us, 140us} for FR1 as the starting point

- PRS Rx frequency hopping range can be up to 100MHz

- Which specific value for frequency hopping is applied depends on UE capability, if multiple values are agreed

Agreement:

- For DL PRS Rx frequency hopping, use the switching time of {35us, 70us, 140us} for FR2 as the starting point

- PRS Rx frequency hopping range can be up to 400MHz

- Which specific value for frequency hopping is applied depends on UE capability, if multiple values are agreed

To maximize the benefit of coherent processing of the PRS signals received across multiple hops, it is desirable to avoid, or at least minimize, time-alignment errors between the individual signals. In our view, the most practical way to achieve this would be by receiving the multiple hops during a contiguous period of time, where the receiver is always turned on. e.g. for measurements within gaps, the multiple hops would be received within a single measurement gap instance. Note that for all the feasible values of inter-hop returning time agreed by RAN4, it would be possible for the UE to perform PRS measurements on multiple hops within a single gap instance.

Proposal 5: For defining DL PRS measurement requirements with FH within measurement gaps, prioritize the case where the multiple hops are all received within a single gap instance.

## Requirements without frequency hopping

In RAN4#106bis-e, companies provided initial simulation results to evaluate whether it is possible to reuse the Rel-16 assumptions for number of PRS samples/repetitions and the PRS Es/Iot side-conditions to define PRS measurement accuracy requirements without frequency hopping (FH) for 1Rx RedCap UEs [2]. Based on those results, RAN4 agreed to reuse the Rel-17 side conditions to define the requirements for 1Rx without FH in AWGN with 4 samples. For reduced number samples in AWGN, it was left open whether to reuse the Rel-17 side-conditions based on simulations. The simulation assumptions were updated to include Nsamples = 1 [3].

In fading channels, it was observed from the simulation results that performance degraded significantly if the Rel-17 side-conditions are reused. RAN4 may consider evaluating a higher SINR condition to define requirements for 1Rx RedCap devices, e.g. -10 dB.

Issue 2-2-2: Side conditions for 1Rx without FH:

Agreements:

- Side conditions for 1Rx without FH

  - For AWGN channel, re-use the Rel-17 side conditions, and relax accuracy requirements

    - The agreement applies for 4 measurement samples case. FFS if it applies for lower number of samples.

  - FFS for fading channel

    - Option 1: Reuse Rel-17 side conditions and relax accuracy requirements

    - Option 2: Reuse approximately Rel-17 accuracy requirements and relax the side condition

Note: Updated simulation assumptions to address open issues are discussed under issue 2-2-8

We have provided simulation results for 1Rx with reduced number of samples in AWGN in a companion paper submitted in this meeting [4]. Based on our results, we propose to also reuse the Rel-17 side-conditions to define the accuracy requirements for that case.

Proposal 6: Reuse the Rel-17 side-conditions to define measurement accuracy requirements for 1Rx without FH in AWGN with reduced number of samples.

Proposal 7: RAN4 to evaluate using a higher side-condition to define measurement accuracy requirements for 1Rx without FH in fading conditions.

# Conclusions

Observation 1: RAN4 agreed to define requirements for RedCap positioning in RRC_IDLE state.

Proposal 1:

- Define requirements for RedCap positioning without frequency hopping in RRC_IDLE state

- FFS define requirements for RedCap positioning with frequency hopping in RRC_IDLE state

Proposal 2:  PRS requirements for both 1Rx and 2Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques.

Proposal 3: PRS requirements for both 1Rx and 2Rx RedCap UE with FH will be defined for all the Rel-16 positioning features. FFS which Rel-17 positioning features will be covered by the requirements with FH.

Proposal 4: RAN4 to wait for more progress in RAN1 before starting to work on detailed PRS measurement period requirements with frequency hopping for RedCap UEs.

Proposal 5: For defining DL PRS measurement requirements with FH within measurement gaps, prioritize the case where the multiple hops are all received within a single gap instance.

Proposal 6: Reuse the Rel-17 side-conditions to define measurement accuracy requirements for 1Rx without FH in AWGN with reduced number of samples.

Proposal 7: RAN4 to evaluate using a higher side-condition to define measurement accuracy requirements for 1Rx without FH in fading conditions.

# References

[1] R4-2306349, WF on RRM requirements for RedCap positioning and PRS/SRS BW aggregation, RAN4#106bis-e

[2] R4-2305778, Summary of simulation results for 1Rx RedCap UE positioning measurements, RAN4#106bis-e

[3] R4-2306350, Updated simulation assumptions for 1Rx RedCap UE PRS measurement without FH, RAN4#106bis-e

[4] R4-2305672, Simulation results for 1Rx RedCap UEs without frequency hopping – reduced number of samples, Qualcomm Inc, RAN4#107

[5] Session notes for 9.5 Expanded and Improved NR Positioning, RAN1#112bis-e

[6] R4-2306659, LS reply on switching time for DL PRS or UL SRS frequency hopping for RedCap UEs, RAN4#106bis-e

[7] RP-230327, Status report for Rel-18 NR_pos_enh2, RAN#99